RAMLESS MONITOR

The Ramless Monitor provides a subset of the functions of

the RAM-Monitor without using any memory to perform its work.  The

syntax and output of commands are as close as possible to the

RAM-Monitor.  Both backspace and delete work as erase characters.

Hitting the escape key at any time immediately returns to the

ramless monitor.  The space before the last character of a command

specification is optional.

arguments in [] are optional

arguments in <> are required

for arguments separated by comma, pick any one

The Ramless Monitor Commands are:

COMPARE

<addr1> <addr2> <count> C<cr>

compare a block of memory

DISPLAY

<addr> <count> D<cr>

display a block of memory

FILL

<addr> <count> <val> F<cr>

fill a block of memory with the

byte value val starting at addr for count bytes

GO

<addr> G<cr>

go and start execution at a specified addres

HELP ?

?<cr>

print a synopsis of ramless monitor commands

MOVE

<srcaddr> <destaddr> <count> M<cr>

move a block of memory

OPEN A LOCATION "`" "\" "/"

byte,word, or double mode

[addr]/			Open a double location (32 bits)

at addr or last open address.

Subsequent open mode commands

will use 32-bit operations.

[addr]\			Open a word location (16 bits)

at addr or last open address.

Subsequent open mode commands

will use 16-bit operations.

[addr]`			Open a byte location (8 bits)

at addr or last open address.

Subsequent open mode commands

will use 8-bit operations.

open mode commands:	Once a location has been opened,

there are the following options:

Typing a /, \, or ` reopens the

current location in the size specified.

This rereads the specified location.

Typing a new address and then

one of /, \, or ` opens a new location.

Typing a new value and a <cr> writes

the value to the current location

and closes the location.  Typing

a <cr> with no value will close the

location without writing anything.

Typing a ^ opens the proceeding

location after writing data to the

current location if any was entered.

Typing a line feed opens the next

location after writing data to the

current location if any was entered.

Typing a @ will use the current

location as a pointer and open a new

location at that address.

IMAGE LOAD

I<cr>			Start up the serial loader.

The serial loader is program to load binary

data over a serial line (tty01) into

memory.  It uses the following protocol:

(1) Wait till receive the 4 sync characters

AISL.

(2) Read in 4 bytes to form load address

most significant byte first.

(3) Read in 4 bytes to form a image size

most significant byte first.

(4) Read in 4 bytes to form a checksum

most significant byte first.

(5) Read in image size bytes calculating

a checksum of received characters.

(6) Compare checksum of received characters

with that received earlier.

Print a message if they are not equal.

(7) Recalculate the checksum by using the

data loaded into memory and compare

with original checksum.

(8) If all is well print the load address

and the image size and return to the

ramless monitor.

WRITE CONTINUOUSLY--------------------------------------------------

[addr] [value] X<cr>	write the location at addr continuously

with the given value

READ CONTINUOUSLY---------------------------------------------------

[addr] Y<cr>		read the location at addr continuously

ZAP--------------------------------------------------------------

<addr> <value> <b,w,d> Z<cr>	write a location, without reading