##APPENDIX D##

# DESCRIPTIO� PRINTER DRIVER#

#Channel Assignments# :

PRTDEV - 06        Parallel printer driver

#Operation Codes# :

---------------------------------

GSTATRG- 01 -      Get status register

SSTATUS- 03 -      Set status

WRITE  - 05 -      Write

WRFLUSH- 07 -      Flush output channel

#Error Codes# :

NOERR  - 00 - No error

NOTRDY - 05 - Not Ready

OUTPAP - 08 - Out of Paper

STRTNF - 13 - Start Not Found

ENDNF  - 14 - End Not Found

I� non-zer� erro� cod� i� returned,th� carr� a�d overflo� bit�  �

will be set and the zero flag will be cleared also.

.pa�

#Functional description of the driver.

GET STATUS REGISTER#

#Input Parameters# :

DL = GIOST

#Output Parameters# :

AX : Status word:

Layout:

#Symbol    Bit       Function#

Reserved  0,1,3,6,7

TXRDY     2         hardware transmitter buffer is ready for data

TXBRDY    4         FIFO ready for data

CTS       5         always set to one

#Description#

Returns status of output channel hardware as well as

output FIFO status.

.pa�       #SET PARAMETER#

#Introduction#

This function provides subfunctions to set different operating modes

or to pass parameters to the driver.

#Input Parameters# :

DL:  Function code

CODE IN DL          OPERATION Performed

WRATTB              writes AH to internal attribute byte.

Input:

AH : Attribute byte

Bit       function

0         Enable bit 7

i� thi� bi� i� reset,a� an� wit� 7FȠ wil� b� �

performe� befor� an� characte� wil� b� pu� i� th� FIFO

1�        Wai� i� no� read� els� immediat� retur� (Characte� mode)

2         Reserved

3         Reserved

4         Reserved

5         Reserved

6         String mode

7         Block/string mode enable

DISOUTP             disables output channel.

Input:

AL : 0 for disable , 1 for enable of transmitter

SETOWT              puts ES:DI into the internal address register

of the output channel WAIT routine

Input:

ES:DI  pointing to user  routine

SETOTSK             puts ES:DI into the internal address register

of the output channel START routine

Input:

ES:DI  pointing to user  routine

SLOWDS              selects duration of printer data strobe

Input:

AL : 0 for 5 us strobe ,1 for 1ms strobe duration

.pa�       #WRITE#

#Input Parameters# :

AH : Options

bit       Function

1�        Wai� i� no� read� els� immediat� retur� (Characte� mode)

2         Reserved

3         Reserved

4         Reserved

5         Enables the internal attribute byte

6         String mode

7         Block/string mode enable

AL : Character if in character mode

CX : Number of characters in buffer for Blockmode

Start,Stop Character for Stringmode

DX : Buffer size for stringmode (maximum string length)

Buffer [ES:DI] : Data to be send if block or string mode.

#Output Parameters# :

#Description#

This is the basic output driver.

WAIT bit:

I� th� wai� bi� i� rese� an� th� outpu� FIFO i� full,th� drive� wil�

exit with an notready error.

If the wait bit is set the driver will call an user programmable routine

which may be an task switching utility.If the driver becomes ready again it

will call an seperate routine which may wake up the task again.

The wait bit is ignored in block and string mode and the driver will always

call an user programmable routine.

The driver provides internaly an dummy routine which will be used until

the user sets the address of an user specific routine.

The driver provides three different operating modes:

Characte� mod� provide� singl� characte� outpu� i� th� string/bloc�

mode bits are reset

If the block mode bit is set the driver will transfer in fixed number

of characters.

If the block and string mode bits are set,the driver will scan the user

buffe� fo� a� characte� containe� i� CH i� CH� � an� wil�transfer �

unti� a� sto� characte� containe� i� CL wil� b� foun�.

Example:

Start character: '!'

Stop character: '#'

User buffer contains: "ABCD!123#EFG"

Data send: "123"

.pa�

#FLUSH OUTPUT BUFFER#

#Description#

This function performs an flush of the output buffer unconditionally.

.pa�ref 1.0 12.12.84

###################################################################