This is preliminary documentation on the serial line protocol used

between the IBM PC's and the PC Gateway. The protocol has two levels:

the low-level protocol (LLP) and the local net protcol. The low level protocol

wraps a packet and delivers it to the PC Gateway. The local net protocol tells

the PC Gateway what kind of packet it is. Currently there are two types of

packets: Internet and Address Request. When the PC Gateway receives an Internet

packet, its action is to forward the internet packet to a process which checks

the packet for validity and then sends it to the net. On receipt of an Address

Request packet, the PC Gateway sends an Address Request packet back to the PC

with four bytes of data (the PC's internet address) in the body of the packet.

Neither the LLP nor the local net protocol provide for prioritized

transmissions, checksums or complex line control. They are merely a simple way

to get packets to and from machines.

The local net protocol consists of a four byte leader. For an internet

packet, this leader is: 2,1,0,0. For an Address Request, the leader is 2,3,0,0.

There is no data in the Address Request packet sent from the PC; it is only

the four byte local net header wrapped in the serial line protocol.

The receipt of any packet with a local net header that does not

identify the packet as either internet or address request is an error and the

receipt of such a packet should be logged and the packet discarded.

LLP consists of four bytes with special meanings when received over the

serial line. These are: ESC, REQ, ACK, and END.

A packet is enclosed in a REQ and an END. When a PC wishes to send a

packet it first sends a REQ to the PC Gateway. It then waits a suitable length

of time to receive an ACK from the PC Gateway. If no ACK byte is received, the

PC Gateway is assumed to be unable to handle the packet right now and a timeout

is said to have occurred. The PC may either retry, wait or return an error.

After the PC receives the ACK signal, it may begin sending the packet.

The first four bytes of the packet should be the local net header and an error

will occur if they are not valid. When the PC has completed sending the packet,

it should send an END byte to the Gateway. The PC gateway will the consider the

packet and act upon it.

The PC Gateway goes through a similar process when it sends a packet to

a PC; only the roles are reversed. It is an error for the PC Gateway to send

a PC its address if the PC has not requested its address from the Gateway.

If a machine should receive a REQ imbedded in a packet, this indicates

that the END signal was dropped somewhere. The receiving machine should drop

the packet it was receiving and begin to receive a new one. ACKs may be mixed

inside packets to allow immediate response to REQs. The receipt of an ACK by

a machine should be duly logged (and perhaps appropriately ignored if the

PC doesn't have an outstanding request).

The final code, ESC, is used to allow the characters whose codes are

used by ESC, REQ, ACK and END to appear in packets. Receiving an ESC indicates

that the next byte should be looked at to produce the correct character. Here

is a table of the codes for the signals and the ESC sequences to produce the

data whose codes they use.

ESC	242	ESC 0

REQ	243	ESC 1

ACK	244	ESC 2

END	245	ESC 3

A simple way to unstuff the bytes is to add the character following an ESC to

the ESC to get the correct code and then put it in the packet as data. It is

an error to have any character > 3 follow an ESC.

If a machine receives any character other than a REQ or an ACK when it

is not in the process of receiving a packet, it should discard that character.