According to the supplied literature of the D.C.Hayes

MicroModem-100, it cannot be operated above 300 baud.

The limiting factor is not the Uart or it clock

frequency but the Microcoupler itself (according to

its designer). Not accepting that their design was

such that its upper limit was no higher than 300

I decided to experiment. If you replace the diodes

that determine the 12-bit divider for the clock

frequencyto a decimal17 - 00010001 - (I used a

16-pin socket and a removeable header), you can

squeeze out 450 Baud. I couldn't get 600 baud, but

the upper limit probably varies from unit to unit.

If you wish you had a PMMI, give this a try. Anything

is better than 300.

Bob Loesch

Bel Air,Md.

(301) 879-7135