The H89MOD74 program is MODEM74 set up to run on the Heath H89

computer. Use the configur program supplied with your operating system

to set the baud rate for the port.

If you want to add an initialization routine to H89MOD74 use

Modem731.set and following the instructions add the following routine.

LXI	H,XXXX   (XXXX=baud rate)- 180H=300 Baud

MVI	A,83H			   100H=450 Baud

OUT	0DBH			   0C0H=600 Baud

MVI	A,03H

OUT	0DCH

MOV	A,L

OUT	0D8H

MOV	A,H

ANI	0FH

OUT	0D9H

MVI	A,03H

OUT	0DBH

XRA	A

OUT	0D9H

DAD	H

DAD	H

DAD	H

DAD	H

DCX	H

MOV	A,L

ORA	H

JNZ	(PUT SAME ADDRESS AS DCX HERE)

RET

DICK HILL

SYSOP VALPARAISO RCPM

################################