OSBORNE.DOC   -   FIRST IMPRESSIONS

OF THE OSBORNE I MICROCOMPUTER

by:   Steven C. Rich

Computer Connection

Farmington Hills, Michigan

(313) 477-4470

--------------------------------------------

The wait is finally over!	We  received our  demonstra-

tion unit of the Osborne I  Microcomputer last Monday,	July

13.  I have just spent the past several days going over this

unit, and thought you might find my first impressions inter-

esting.

First  some technical information at random.  Note that

this  is  an initial production unit.	There  may  be	many

changes before units get in customer hands. Yes, all digital

electronics are on one board!  (and quite a good looking one

at  that!).  Only the boards for the drives and the  monitor

electronics are seperate.    Memory appears to use 4116 type

chips.	The CPU looks like the Japaneese NEC Z-80 Copy!  The

drives	are  10 Soft Sector with Standard capacity of  102K.

This  is supposed to be expandable through a double  density

option.  The  ones in the unit we have are made by  Seimans.

All  we have received in the way of software is a very	bare

version of CP/M ( SYSGEN,  COPY,  PIP,	and FORMAT,  but  no

Assembler or DDT ),  and Microsoft Basic-80.  Both appear to

work  beautifully on the Osborne.   The caliber of the	CP/M

utilities  sent is excellent (the FORMAT program  automatic-

ally checks the media for example!).  One interesting bit of

information  is  that there is apparently a Real Time  clock

function for DATE and TIME which will be set through a SETUP

program.  The Microsoft manual, by the way, is a copy of the

Standard  Basic 80 one,  but is fully typeset and thus	much

more readable.

OK.  Time for first impressions.	Taken in the context

of an economical and complete way to get into computing (and

CP/M  software in particular) it is a great machine.   It is

true  that  you may not be able to expand  the	machine  for

ever,  but how many of us have been able to afford to create

our  "dream  machines".   Ever	try to talk YOUR  wife	into

agreeing to let you spend $6,000 for a hard disk (but I just

HAVE  to have it!).   The 52 character format of the  screen

may might seen hard to get used to,  but it is a window on a

larger	128 X 36 character screen.  By pressing the  Control

key  and  one of the arrow keys,  you INSTANTLY  scroll  the

screen.  I cannot praise the speed of the scroll enough.  It

is  extremely  smooth  and  fast,  and	will  overcome	many

objections  of	the screen format.  And if you	truly  can't

stand  the  small  screen and don't like  the  9"  ($250.00)

monitor,  then just connect a "Professional" terminal to the

RS-232 port,  reassign the CON:  to the port and off you  go

with a full 80 characters!

I	am going to reserve final judgement until I see  the

rest of the software.	But, from what I have seen so far, I

feel that Mr.  Osborne has done just what he started out  to

do  - Place a full function CP/M business system within  the

reach of anyone and made it portable to boot!

More information will follow shortly.   Please call  me

if you want more info.

STEVE RICH

#############################################################