KPROMDM7.DOC  NON-LINEAR KAYCOMP KAYPRO II DOC FOR MODEM7.

AS OF 08/21/82.      Tom McCormick.  Houston, Tx.

This portable CP/M micro comes with an RS232c serial port,

a BAUD.COM utility to set the baud rates on that port, and

the SELECT editor with which to make the following changes

to MBOOT.ASM.

MBOOT.ASM is a minimal subset of MODEM7: all it will do is

operate in terminal mode, and then receive one file at a time

using the Christensen protocol of MODEM7.  It will not send,

auto dial, send/receive without handshaking, transfer multiple

files from a single command (wildcards), etc. like the full

MODEM7 program will do.   BUT...it is much much shorter to

key in the first time if you do not know anyone with MODEM7

on a KAYPRO II compatable 5" diskette.

Obtain a printed copy of MBOOT.ASM (free, public-domain), and

make the following changes.  You can then use MBOOT to transfer

the full MODEM7 program to yourself, and make the same changes

to it.

----------------------------------------------

Here are the MODEM7 values to patch for KCOMP.

04H   MODEM DATA PORT

06H   MODEM STATUS PORT

04H   BIT MASK: READY TO SEND

01H   BIT MASK: READY TO RECEIVE

----------------------------------------------

We connected the KAYPRO serial port directly to

an H-89 serial port as follows:

KAYPRO	  H-89

2 to 3

3 to 2

5 to 4

20-6-8

(JUMP ALL 3) to 20

...and transferred several files at 9600 baud.

We did not try 19200, might work OK.

----------------------------------------------

#red several files at 9600 baud.

We did not