PROTOCOL NOTES: CIS A PROTOCOL

by Bob Richardson, Carl Raff, and Scott Lowe

Copyright (C) 1982

Performance Business Machines

A Division of MicroPro International Corporation

101 Lucas Valley Road

San Rafael, California 94903

note: this material was not prepared for or by CIS and

they have no responsibility for the accuracy or completeness

of the contents.  We gratefully acknowlege their cooperation

and the super job they are doing, and simply wish to help

document the file transfer protocol.

INTRODUCTION

The  information  presented here is intended to  provide  an

outline of the written specifications for the CIS A Protocol.  It

is  designed  to be used by programmers who may need to debug  or

write  programs to interface with  Compuserve  Information

Services (CIS) terminal programs.

The PBM 1000 supports a communication subsystem based on the

CIS protocol.    One  reason is that the CIS protocol is a step  in

the   direction   of   software   standardization   and    higher

compatibility among different computer systems.  The CIS protocol

supports flexible file transfer and is practical for in-house use

on  a direct-connect medium speed network.   PBM tests show  that

the protocol runs effectively at 19.2 KB and can transfer data at

a rate of up to 50 KB.

1

In transferring files from one system to another, one CP/M-

based  system (with a copy of any of the CIS executives that sup-

ports  the  A protocol) acts as a  terminal.  The  other  system,

referred to as the "host," treats the "terminal" system both as a

console  and as the source or target of the file transfer  proto-

col.

The  following is a brief description of the CIS A  Protocol

used by CIS.    Also included is a list of Executives  compatible

with it,  and two new files which allow anyone with a CP/M remote

system to support this protocol between CP/M systems.

Note: CP/M is a registered trademark of Digital Research.

The Executives

1.   MNEXEC  [Old  Executive]

An  older  version of the  Executive,  MNEXEC  supports

ASCII  transfer  only .   This  executive  is  primarily

useful  for those who wish to buy programs from Softex,

as  its ESC I response is the only one that the  Softex

system  currently  recognizes as a file-xfer   executive

(see ESC I response). This program does not really support

the A protocol, but rather a subset.

2

2.   CSEXEC  [The Compuserve Executive]

CSEXEC  was  the first "vanilla"  version  with  binary

support.   It  is  a good executive to start  with  for

those  wishing to heavily modify the source because  it

suffers  less  from "rampant equate-itis" than  BUFEXEC

(i.e,  it accomplishes tasks in a more general  fashion

without presenting too many options to the user).  The

current version seems to be modified for the Apple computer.

3.   BUFEXEC [Buffered CSEXEC]

BUFEXEC  adds memory buffered hard copy to  CSEXEC,  as

well  as  many  customization  equates.    The  Osborne

version  is  called "BUFEXO.ASM," and is  identical  to

BUFEXEC except for modifications for the Osborne I.

4.   IOBEXEC [IO/Byte Executive]

This executive is identical to BUFEXEC,  except that it

uses  IOBYTE  dispatch to access the serial  port.   If

your system is interrupt driven,  you must use  IOBEXEC

or  modify one of the other executives.   In  addition,

IOBEXEC allows use of cursor positioning.

3

PROTOCOLS

Characteristics of the physical protocol: asynchronous; 8-bit;

8 bits to the byte; 1 stop bit; no parity.

The protocol mode is turned "on" by an SI instruction (shift

in)  from host and terminal  functions in protocol mode until  an

SO (shift out) instruction is received.

The  ESC "I" Response:  Each terminal responds to an ESC "I"

with an ID string (see diagrams below).

The ESC "A" Sequence:  This initiates a file transfer.

The ESC "L" Sequence:  This starts a load sequence.

Notes:

The  "." (period) is used to ACK a message (acknowledge  and

accept).

The  "/" (slash) is used to NAK a message (negative  acknowledge;

request retransmission).

CTRL U is used to abort transmission.

Note: a list of fields enclosed in [ ] marks indicate that 1

of the list will be transmitted, depending on the conditions

at the time.

4

ESC "I" Sequence

Host                               Terminal

"SI" (Protocol on)

------------------------------------------->

ESC I (Who are you?)

------------------------------------------->

ID String (see below)

<-------------------------------------------

"SO"  (Protocol  off)

[------------------------------------------->]

error; feature lock; end of function

Continue in Protocol

[------------------------------------------->]

Action:  The terminal sends its ID string to the host.   The

host checks the features list, etc.

ID  String:  A  full sample of the ESC I  response  for  the

IOBEXEC protocol (assigned by CIS) is as follows:

<#> <CPM> <MYSYSTEM>, CC, HC, PA, PL, <CR>

5

The  first character is the protocol ID,  followed by the OS

name and the computer name (optional).   The string of four pairs

of  characters  are a series of features to be  assigned  if  the

system supports them:

o    CC, HC, PA, and PL  <CR>

CC = full cursor control

HC = hard copy

PA = "A" protocol

PL = "L" protocol

See the comments in any version of the executive for a

more detailed discussion of these fields

MNEXEC is an older version of the executive,  and so

the  ID  string is slightly different in format than the one  for

the CIS Executive.  A sample response follows:

<01>,<BF>,<SYSTEM2>,<&CPM>,<D> <CR>

The  first character is the page address of the lowest

available page of memory in the terminal ,  followed  by

the  page  address of the highest available page in  memory,  the

computer ID, OS name, and "D" for disk drives.

6

ESC "L" Sequence

Host                               Terminal

SI

------------------------------------------->

ESC L

------------------------------------------->

<count-1 byte> <low addr> <hi addr> <data> <CHKSUM>

--------------------------------------------------->

"." = ACK (Received; OK)

[ <------------------------------------------- ]

"/" = NAK (Error; request retransmission)

[ <-------------------------------------------- ]

ESC L

------------------------------------------->

Record                    }more to load

------------------------------------------->

"." = ACK (Received; OK)

[ <------------------------------------------- ]

"/" = NAK (Error; request retransmission)

[ <-------------------------------------------- ]

SO (Done)

[  ---------------------------------------------> ]

Action:   The terminal loads <data> into memory starting  at

address <low> <hi> for <count> bytes.

7

ESC "A" Sequence: File Transfer

Host                               Terminal

SI (Protocol on)

------------------------------------------->

ESC A (Start file transfer)

------------------------------------------->

<1-byte SOH> <1-byte rec #> <U/D> <A/B> <file spec><CR> <ETX> <CHKSUM>

------------------------------------------------------------------>

Note: U/D=direction; A/B=ASCII or binary; file spec=standard CP/M

"." ACK (Received; OK)

[   <------------------------------------------- ]

"/" NAK (Retransmit)

[  <-------------------------------------------- ]

Action:   The  terminal sets up to read or  write  the  file

given  in <file spec>.   Transfer will occur either until end-of-

file <B> or,  if <A> (ASCII) is being used, until the first CTRL-

Z.   The  file  will  be  downloaded   (transferred  from  host  to

terminal)   or   uploaded   (transferred  from  terminal  to  host)

depending  on the contents of the direction byte (U or  D).   The

protocol proceeds as follows:

8

ESC "A" Sequence: File Transfer

Download Subsequence

Host                               Terminal

<SOH> <R/N>  <Text>    <ETX> <CHKSUM>

------------------------------------------->

Note: R/N = Sequential record number

"."  ACK (Accepted record)

[ <------------------------------------------- ]

"/"  NAK (Rejected; retransmit)

[ <------------------------------------------- ]

<SOH> <R/N>    <Text>     <ETX> <CHKSUM>

------------------------------------------->

"."  ACK (Accepted record)

[ <------------------------------------------- ]

"/"  NAK (Rejected; retransmit)

[ <------------------------------------------- ]

<SOH> <R/N> <EOT> <ETX> <CHKSUM>

------------------------------------------->

"." or "/"

<-------------------------------------------

SO  (Shift out; protocol off)

[ ------------------------------------------->

ESC A  (Next file)

[ ------------------------------------------->

^

|

[ Note: This is currently unused by CIS.]

9

ESC  "A" Sequence: File Transfer

Upload Subsequence

Host                               Terminal

"."  ACK

------------------------------------------->

<SOH> <R/N> <   Text   > <ETX> <CHKSUM>

<-------------------------------------------

"."  ACK  (Record accepted)

[ -------------------------------------------> ]

"/"  NAK  (Record rejected; retransmit)

[ -------------------------------------------> ]

<SOH> <R/N> <   Text   > <ETX> <CHKSUM>

<-------------------------------------------

"."  ACK  (Record accepted)

[ -------------------------------------------> ]

"/"  NAK  (Record rejected; retransmit)

[ -------------------------------------------> ]

<SOH> <R/N> <   EOT   > <ETX> <CHKSUM>

<-------------------------------------------

"."  ACK  (Record accepted)

[ -------------------------------------------> ]

"/"  NAK  (Record rejected; retransmit)

[ -------------------------------------------> ]

SO  (Shift out; exit protocol)

[ ------------------------------------------->

ESC A  (Next file)

[ ------------------------------------------->

10

Field Definitions

A/B        Indicates file type.  An "ASCII" file ends in CTRL Z; a

"Binary"    file    ends    with    EOT    (CTRL    D).

CHKSUM      Check  sum on record calculated on all characters  sent

after  (and  not including) start of header  (SOH)  and

until (and not including) ETX.  CHKSUM is calculated as

follows:

1. CHKSUM <--- (CHKSUM * 2) mod 256

2. CHKSUM <--- CHKSUM + character

3. CHKSUM <--- CHKSUM + carry

If the check sum is less than hex '20', it is increased

by  hex  '40'  and  sent  masked  (preceded  by  a  DLE

character).

EOT        End of transmission (CTRL D).   When EOT is encountered

in text field of a record, it ends the current file.

ETX        End of text (CTRL C).  ETX ends each packet (a group or

string  of  related characters),  and  is  followed  by

CHKSUM.

R/N        Indicates  the  record  number in  ASCII  characters  0

through 9.  It is incremented by one ASCII number after

each previous record is acknowledged (ACK).  Length:  1

byte.

11

SI    Shift in (CTRL O).  SI starts the protocol mode.

SO    Shift out (CTRL N).  SO ends the protocol mode.

SOH   Start of  header  (CTRL A).  Each record  begins  with  SOH.

Length: 1 byte.

Text  This indicates  a  variable  length  string,  which  is  the

information content of the packet.

U/D   In an ESC  A  header,  the  U/D instruction  determines  the

direction  of  transfer.   U = transfer to  host;  D  =

transfer from host.

Character Masking

Certain control characters are increased by hex '40' and are

preceded by a DLE character when encountered in the "Text"  field

of a message.  These characters at this time are:

Name      Value

NULL      X'00'

SOH       X'01'

STX       X'02'

ETX       X'03'

EOT       X'04'

DLE       X'10'

NAK       X'15'

Note: These are the characters masked by the executive.  CIS

is currently masking almost all control characters.

An exception is the EOT character in the end of file record,

which is sent unmasked.   In general,  masking is used to prevent

confusion  between  a  character  used as  data  (masked)  and  a

protocol character (unmasked).

12

# and  a

protocol character (unmasked).