PORTABLE FRONT PANEL

HANK SZYSZKA

BEARSVILLE, NY

679-6559 (MODEM)

WIREING LAYOUT FOR MITS FRONT PANEL

UNIT #1   7402    QUAD-NAND

UNIT #2   7404    HEX INVERTER

UNIT #3   7410    DUAL-3 INPUT AND

UNIT #4   7474    DUAL JK FLIP FLOP

UNIT #5   74123   DUAL D FLIP FLOP

TR-1 TRANSISTOR   2N5551

UNIT#-PIN#   TO     UNIT#-PIN#            DISCRIPTION

#2-1................S100-24               WIRE TO BUSS (CLOCK 02)

#2-1................#5-3                  WIRE

#2-2................#4-3                  WIRE

#1-2................#SWITCH/STOP          WIRE

#1-1................#3-1                  WIRE

#1-4................RES 1K .25 (A SIDE)   RESISTOR-3

TR-1B...............RES 1K .25 (B SIDE)   RESISTOR-3

TR-1E...............0.0 VOLTS             WIRE

TR-1C...............S100-3                WIRE (XRDY) BUSS

#1-5................#4-6                  WIRE

#1-6................#4-9                  WIRE

#3-2................S100-39               WIRE (DO5) BUSS

#3-12...............#4-4                  WIRE (2)

#3-12...............SWITCH/INST/STEP      WIRE

#3-13...............S100-76               WIRE (PSYNC) BUSS

#3-13...............#2-3                  WIRE

#2-4................SWITCH/SING/STEP      WIRE

#4-2................SWITCH/RUN            WIRE

#4-1................+5.0 VOLTS            WIRE

#4-10...............+5.0 VOLTS            WIRE

#4-12...............+5.0 VOLTS            WIRE

#4-13...............SWITCH/COMMON/STEP    WIRE

#4-11...............#5-13                 WIRE

#5-1................RES 1K (A SIDE)       RESISTOR-1

+5.0 VOLTS..........RES 1K (B SIDE)       RESISTOR-1

#5-1................SWITCH/STEP/IMPULSE   COMPONENT/PUSH BUTTON SW.

#5-2................+5.0 VOLTS            WIRE

#5-3................+5.0 VOLTS            WIRE

#5-14...............CAP .1 MFD (A SIDE)   CAPACITOR-1

#5-15...............CAP .1 MFD (B SIDE)   CAPACITOR-1

#5-15...............RES 47K (A SIDE)      RESISTOR-2

+5.0 VOLTS..........RES 47K (B SIDE)      RESISTOR-2

00.0 VOLTS..........SWITCH/RUN/STOP/COMMON

RESISTOR-4A.........SWITCH/STEP/IMPULSE   COMMON SIDE

RESISTOR-4A.........CAPACITOR-2 (SIDE-A)  WIRE

RESISTOR-4B.........#4-6                  WIRE

RESISTOR-4B.........CAPACITOR-2 (SIDE-B)  WIRE

INDICATOR WIREING (7340)

ADDRESS BUSS DISPLAY                DATA BUSS DISPLAY

+-------+                           +-------+

32-A15--------3-|       |-7-+5.0    43-DI7--------3-|       |-7-+5.0

86-A14--------2-| MOD-1 |-5- 0.0    93-DI6--------2-| MOD-5 |-5- 0.0

85-A13--------1-|       |-4- 0.0    92-DI5--------1-|       |-4- 0.0

33-A12--------8-|       |-6- 0.0    91-DI4--------8-|       |-6- 0.0

|-------|                           |-------|

87-A11--------3-|       |-7-+5.0    42-DI3--------3-|       |-7-+5.0

37-A10--------2-| MOD-2 |-5- 0.0    41-DI2--------2-| MOD-6 |-5- 0.0

34-A09--------1-|       |-4- 0.0    94-DI1--------1-|       |-4- 0.0

84-A08--------8-|       |-6- 0.0    95-DI0--------8-|       |-6- 0.0

|-------|                           |-------|

83-A07--------3-|       |-7-+5.0    90-DO7--------3-|       |-7-+5.0

82-A06--------2-| MOD-3 |-5- 0.0    40-DO6--------2-| MOD-7 |-5-------*

29-A05--------1-|       |-4- 0.0    39-DO5--------1-|       |-4- 0.0  |

30-A04--------8-|       |-6- 0.0    38-DO4--------8-|       |-6- 0.0  |

|-------|                           |-------|         |

31-A03--------3-|       |-7-+5.0    89-DO3--------3-|       |-7-+5.0  |

81-A02--------2-| MOD-4 |-5- 0.0    88-DO2--------2-| MOD-8 |-5-------*

80-A01--------1-|       |-4- 0.0    35-DO1--------1-|       |-4- 0.0  |

79-A00--------8-|       |-6- 0.0    36-DO0--------8-|       |-6- 0.0  |

+-------+                           +-------+         |

77-NOT-PWR------------------------*

PINOUT FOR 7340 DISPLAY

*----*--*  *-------------> +5.0

|  +-|--|--|--|--+

|  | 5  6  7  8  |

|  |             |

|  |             |

|  |             |

|  |   (7340)    |

|  |             |

|  | WIRING SIDE |

|  |             |

|  | 4  3  2  1  |

|  +-|--|--|--|--+

*----*

|

*------------------> 0.0

PIN LOGIC:

1 INPUT BIT-2           2 INPUT BIT-4

3 INPUT BIT-8           8 INPUT BIT-1

7 PLUS 5 VOLTS          6 GROUND 0.0

5 LATCH ENABLE          4 BLANKING CONTROL

LED INDICATORS

47-MEMR----(LED)----RP-01-220-RESISTOR------------*

46-INP-----(LED)----RP-02-220-RESISTOR------------*

44-M1------(LED)----RP-03 220-RESISTOR------------*

45-OUT-----(LED)----RP-04-220-RESISTOR------------*

48-HLTA----(LED)----RP-05-220-RESISTOR------------*

98-STACK---(LED)----RP-06-220-RESISTOR------------*

97-W0------(LED)----RP-07-220-RESISTOR------------*

96-INTA----(LED)----RP-08-220-RESISTOR------------*

|

|

28-INTE----(LED)----RP-09-220-RESISTOR------------*

27-WOUT----(LED)----RP-10-220-RESISTOR------------*

26-HOLD----(LED)----RP-11-220-RESISTOR------------*

*-RP-14-> 0.0 (GROUND)

NOTES:

0.0 = GROUND (ZERO VOLTS)

+5.0 = PLUS 5.0 VOLT REGULATED POWER.

RP-XX-220-RESISTOR = RESISTOR PACK/COMMON PIN-14

(LED)        = LED INDICATOR

NUMBER INPUTS ARE FROM S100 BUSS.

WIREING OF TTL CHIPS

*--------*-R1-*-------------*---------------*-----------------*--------->+5.0

| *-------------------------*      *--------|-----------------|-----*

| |   +--------+            |      |        |    +--------+   |     |

*-|-01| 74123  |16----------*      |        *--01| 74LS74 |14-*     |

| *-02|        |15-----*-R2-*      |  *--------02|        |13-|--*  |

| *-03| MODUAL |14--C1-*           |  |   *----03| MODUAL |12-*  |  |

|   04|   5    |13-----------------*  | *-|----04|   4    |11-|--|--*

|   05|        |12                    | | |    05|        |10-*  |

|   06|        |11   *----------------|-|-|----06|        |09----|-*

|   07|        |10   |                | | |    07|        |08    | |

|   08|        |09   |                | | |      +--------+      | |

|     +--------+     |                | | |                      | |

|   *----------------*       *--------* | |                      | |

*---|------------------------|--*       | |                      | |

|                 *------|--|-------* |                      | |

|   +--------+    |      |  |         |      +--------+      | |

*|-01| 74LS10 |14  |      |  |         | *--01| 74LS04 |14    | |

*-||-02|        |13--|------|--|--*      *-|--02|        |13    | |

| || 03| MODUAL |12--*      |  |  *--------|--03| MODUAL |12    | |

| || 04|   3    |11  |      |  |  |  *-----|--04|   2    |11    | |

| || 05|        |10  |      |  |  |  |     |  05|        |10    | |

| || 06|        |09  |      |  |  |  |     |  06|        |09    | |

| || 07|        |08  |      |  |  |  |     |  07|        |08    | |

| ||   +--------+    |      |  |  |  |     |    +--------+      | |

| ||                 |      |  |  *--|-----|------PSYNC-BUSS----|-|---->76

*--|-||-----------------|------|--|-----|-----|--------------------|-*

|  *-||-----------------|------|--|-----|-----|---------+DO5 BUSS--|------>39

|    ||                 |      |  |     |     *---------CLOCK-P2---|------>24

|    ||   +--------+    |      |  |     |     |                    |

|    *|-01| 74LS02 |14  |      |  |     *-----|--C-STEP->          |

|  *--|-02|        |13  |      |  |           |           <--SW-2--*

|  | *|-03| MODUAL |12  *------|--|-----------|--I-STEP->

|*-|-||-04|   1    |11         |  |           |

|| | |*-05|        |10         |  |           |

*|-|-||-06|        |09         |  |           |

| | || 07|        |08         |  |           |

| | ||   +--------+           *--|-----------|--RUN---->

| | *|---------------------------|-----------*           <--SW-1--------> 0.0

| *--|---------------------------|--------------STOP--->

|    *------------------------*  |

|          |C-----------------|--|-------------------------BUSS XRDY---->03

*----R3---B|                  |  |

|E----> 0.0        |  *--------------STEP---> |

*-----*-R4-*------STEP---> |

TR1                       *-C2-*

LEGEND:

R1 = 47.K .25 WATT               SWITCH-1  STOP/RUN SWITCH SPDT

R2 = 1.0K .25 WATT               SWITCH-2  INST./CYCLE STEP SWITCH SPDT

R3 = 1.0K .25 WATT               SWITCH-3  STEP SWITCH MOMENTARY SPST

R4 = 220K .25 WATT               TR-1      TRANSISTOR 2N5551

C1 = .1 MFD 10V CAPACITOR        C2  =     .1 MFD 10V CAPACITOR

BOARD LAYOUT

+-----------------------------------------------------------------------------+

|                                                                             |

| +------+                                                                    |

| |      | ===============================================================    |

| | REG  |    +---+---+---+---+   +---+---+   +---+---+  RUN  INST STEP       |

| |      |    | 1 | 2 | 3 | 4 |   | 5 | 6 |   | 7 | 8 |                       |

| | +5.0 |    | 7340 DISPLAY  |   | 7340  |   | 7340  |   SW   SW   SW        |

| |      |    | ADDRESS BUSS  |   | DATA  |   | DATA  |    1    2    3        |

| +------+    |   |   |   |   |   |  IN   |   |  OUT  |                       |

|             +---+---+---+---+   +---+---+   +---+---+  STOP CYCLE PB        |

|          ===============================================================    |

|     (LEDS)  O  O  O  O  O  O  O  O            O  O  O                       |

|                                                                             |

|     +--+    M  I  M  O  H  S  W  I            I  W  H                       |

|     |RP|    E  N  1  U  L  T  0  N            N  R  O                       |

|     |01|    M  P     T  T  A     T            T  I  L                       |

|     |  |                A  C     A            E  T  D                       |

|     +--+                   K                                                |

|                                                                             |

|                                                                             |

|                                                                             |

|                                               +--+ +--+ +--+ +--+ +--+      |

|                                               |  | |  | |  | |  | |  |      |

|                                               |M1| |M2| |M3| |M4| |M5|      |

|            (+) 2N5551                         |  | |  | |  | |  | |  |      |

|                                               +--+ +--+ +--+ +--+ +--+      |

+----------+                                                   +--------------+

|                                                   |

|                                                   |

+---------------------------------------------------+

PIN 1                THRU                             50

51                THRU (BOTTOM)                   100

M1= 74LS02      DUAL D FLIP FLOP

M2= 74LS04      DUAL JK FLIP FLOP

M3= 74LS10      DUAL 3 INPUT AND GATE

M4= 74LS74      QUAD 2 INPUT NAND

M5= 74LS123     HEX INVERTER

2N5551          NPN TRANSISTOR

7340            HEX DISPLAY WITH LOGIC

#############################################################