MODEMS - AN OVERVIEW FOR BULLETIN	BOARD IMPLEMENTORS

-----

by David Boruff

Maryville, Tennessee

8 September 1981

For  the  past  several  months,  I've been researching the

market in an attempt to	find the all-time  hot	set-up	modem to

use with my own	CBBS-RCP/M system.  I've finally made my choice,

and  perhaps some of my	thoughts and observations will help  you

make yours.

To	 get  a	bulletin board or remote file transfer system on

the air, you need  more	than just a simple acoustic coupler such

as the widely-used Novation CAT.  As a minimum,	you need a modem

which can automatically	 answer	 the  telephone,  run  in either

answer	 or   originate	  mode,	 and  offer  a	high  degree  of

sensitivity, selectivity, and  immunity	 to  both electrical and

acoustic  noise.    Although  auto-originate is	nice,  it  isn't

necessary for a	bulletin board system since its	primary	function

is to answer calls, not	originate  them.   But since most people

who implement a	bulletin board or RCP/M	system	also  use  other

systems,  the  added  flexibility  offered  by an auto-originate

modem is worth paying a	little more for.

The requirement that the modem should be capable of running

in either answer or originate mode may not be obvious, but it is

important.    Normally,	 a caller dials	up a bulletin  board  in

originate mode,	and the	bulletin  board	 answers in answer mode.

For the	unitiated, the originate/answer	terms refer primarily to

the various tones that are used	to transmit and	receive	data.  A

binary	1  (or "mark") is transmitted at one  frequency,  and  a

binary 0 (or  "space")	is  transmitted	 at  another  frequency,

(usually lower in pitch	then the mark tone).  The difference  in

frequency  between  the	mark tone and the space	tone is	referred

to  as	the  frequency	shift,	or  just  "shift".    Ham  radio

enthusiasts will recognize  the	 mark, space, and shift	terms as

being hang-overs from teletype terminology.

In	 addition to having a difference  in  frequency	 between

mark and space,	 there is also a difference in frequency between

answer and originate.	 Typically,  an	 answer	modem's	mark and

space  tones  are  about  1  KHz  higher  in frequency	than  an

originate  modem's  tones, but both still use  a  common  shift.

With Bell 103 type modems, the answer modem marks at 2225 Hz and

spaces at 2025 Hz (a  shift  of	 200  Hz).   The originate modem

marks at 1270 Hz and spaces at 1070 Hz (again,	a  shift  of 200

Hz).  In practice, the answer modem transmits at 2225/2025,  and

listens	 at  1270/1070 Hz, with	the originate modem transmitting

at 1270/1070,  and  listening  at 2225/2025 Hz.	 In this manner,

both modems can	talk at	the same  time	since  each  ignores the

tones  its  transmits,	and  only  recognizes  the  tones  being

transmitted  by	 the  other  end.    This  is  referred	 to as a

full-duplex  system  because  communication  is	possible in both

directions simultaneously.  There is nothing particularly  magic

about  the  Bell  103 tones, although they were	chosen with some

thought	as to the characteristics of the telephone network.  But

other frequencies and  shifts  can and are used	by other modems,

Other frequency	and shift combinations are possible, and in fact

are used on non-Bell 103 type modems, particularly those capable

of running at 1200 baud.  But remember	that  a	 modem using the

Bell  103  type	 tones	can only communicate with another  modem

using  the  same tones.	 A  Bell  103  modem  can't  talk  to  a

Racal-Vadic 1200 baud job, at any baud rate.

Okay, enough  terminology.	   What	 difference does it make

whether	my modem transmits in originate	or  answer  mode as long

as  the	 guy  on the other end is using	the complementary  mode?

Assuming that you  have	 a  good  connection,  it really doesn't

matter.	   But	on  a  noisy telephone line, the frequencies  do

matter,	since certain types of spikes and garbage may affect one

tone pair more than the	other.	There are those	who contend that

the high tones are more	 immune	to noise than the low tones, and

there are those	who contend  the  opposite.    I contend that it

depends	on the type of noise being encountered,	but it is a fact

that  you  can	sometimes  salvage  a bad connection  by  simply

swapping tones.	 That is, let the answer  modem	go to originate,

and let	the originate modem go to answer.   If the data	is being

transferred primarily in one direction only, such as when XMODEM

is sending a file, and infrequently listening  for  a  handshake

character, "flipping" tones may	make all the difference	 in  the

world.	However, if data is being transferred in both directions

equally	 often,	 flipping tones	probably won't help since you're

just transferring the hits from	one end	to the other.

So	we see that  the ability to change tones in midstream is

important.  It should  be  obvious  as	well that if you have an

answer-only modem, you can't use it to dial-up	another	bulletin

board or time-sharing system, since the	other end  will	be using

the answer tones when it picks up the phone.

What other	desirables are there to	consider before	buying a

modem?	 Well selectivity and sensitivity are very important  in

that they  allow  you're  modem	 to  communicate  on  a	marginal

telephone  line.    In	this respect, all modems are not created

equal.	A modem	featuring high selectivity usually has some sort

of crystal-controlled filtering	system to allow	it to zero in on

the precise mark and space  tones  its	listening  for,	 and  to

ignore	other  garbage	on  the	 line.	High selectivity is very

important in an	acoustically-coupled modem because the modem may

pick up	ambient	room noise along with the tones.  With a popular

low-priced modem I've played with,  I  can  raise  all	kinds of

havoc  by having my stereo going at the	same time I'm trying  to

transfer  data.	   The	reason	is  that if the	music contains a

frequency close	to the frequency of the	mark or	space tones, the

modem will get confused	unless it really  notches out everything

except the tones its looking for.  With	a direct-connect  modem,

you  avoid  the	 problem  of room noise, but noise, ringing, and

echoes on the line still make high-selectivity very desirable in

either type of modem.

Sensitivity is also a big deal, since it determines how low

a signal level you're modem  can work.	A highly-sensitive modem

can pull a signal out of the mud  where	 a  less-sensitive modem

would  either fail to hear anything at all, or	at  least  would

plague you  with  drop-outs  everytime a little	pop ambled by on

the line.  As with selectivity,	 sensitivity  is  more important

with  an  acoustically-coupled	modem than with	a direct-connect

modem  simply because the direct-connect  modem	 won't	have  to

listen for the tones through any room noise it might pick up.

Both sensitivity and selectivity have to do with how well a

modem will  receive  data.    But  when	 transmitting  data, the

frequency  stability  of the tones being generated is important.

You  don't want	a tone oscillator  that	 drifts	 off  frequency,

especially  if	the  other  guy	is using a modem with very sharp

filtering  ("high selectivity").    The	 best  modems  currently

available  at  hobbyists  prices  all  use  crystal-  controlled

oscillators  to	 generate  the	tones,	and  since  this type of

circuitry  is  cheap to	design and manufacture,	stability  isn't

much of	a problem with all but the cheapest of modems.	However,

an acoustically-  coupled  modem can get fouled	by the telephone

microphone element.  The tones	could  be generated by the modem

right on frequency, but	if the carbon microphone element used in

most  handsets has packed down,	the tone  could	 be  transmitted

down  the   telephone	line  either  distorted	 or  shifted  in

frequency.  Replacement	microphone  elements are available, such

as the Novation	Super Mike, which are  superior	 to the	elements

used  by  Ma Bell.  I can testify from personal	experience  that

the Novation  element  does work and has frequently made all the

difference in the world	 on some connections.  If you go with an

acoustic modem,	no matter  how	good  it  is, I	would definitely

recommend that you install a Super Mike	as standard equipment on

your modem telephone.  Its a ten buck miracle.

From the previous discussion, you may  get	 the  impression

that  a	 direct-  connect  modem  is  definitely superior to  an

acoustically-coupled job.  Theoretically, this line of reasoning

is correct.  In	actual practice	however,  change "definitely" to

"maybe".  My arsenal of	computer equipment  presently includes a

Datec  32  originate/answer acoustic modem, and	 I  can't  speak

highly enough  of  it.	 Its ugly as sin and bulky as bulldozer.

When the handset is inserted  in the coupler, you have to strain

your neck to see the receive-data and carrier-detect lights, and

it takes too long to acquire carrier-detect.  But when the going

gets tough and a lot of	other highly-touted  modems are	ready to

throw in the towel, Datec just hums right along.    With a Super

Mike-equipped  handset,	 the  Datec  is	 a  damn  fine piece  of

equipment.   Hobbyists seldom use them,	but professionals  swear

by  the	 entire	  line	 of   Datecs.	 If  you  want	a  basic

originate/answer modem with performance	 second-to-none, and you

don't need all the fancy auto this and auto  that,  the	Datec 32

and a Novation Super Mike is what you want.

Why this testimonial about	a bare-bones acoustic modem when

we're  talking	about  super-modems?	Simply	this:  the Datec

acoustic  can outperform some of the best direct-connect  modems

on the market  today  including	 the  CATs, U.S. Robotics, D. C.

Hayes  Micromodem  100,	 Microconnection,  and	several	 of  the

high-priced  Racal-Vadics (all	Bell  103-compatible).	  In  my

opinion, you're	 going to have to spend	a big (as in BIG) wad of

money	to  get	 a  Penril   or	  some	 other	 top-of-the-line

"professional-quality"	modem  that  will work any better at the

basic level than my Datec.  So don't let anybody tell you that a

direct-connect	  modem	   will	   "always"    outperform     an

acoustically-coupled  modem.	If  all	 other factors and basic

specifications are the same, a direct- connect	modem  WILL do a

better	job.	But  a	well-designed  acoustic	coupler	will run

circles	 around	 a  poorly-designed direct-connect  modem  every

time.

A	companion  article  may	 be  available	on  this  system

reviewing a newcomer  to  the direct-connect super modem market,

the D. C. Hayes	Smartmodem.    Anyone thinking of using	this new

product	on a CBBS or RCP/M type	 system	 should	read this review

before making your purchase.

#######################################################