THE D. C. HAYES SMARTMODEM

Convenience at What Price?

-----

Reviewed by	David Boruff

Maryville, Tennessee

8 September 1981

-----

The folks at D. C.	Hayes have been	splashing a lot	of color

in  the	computer hobbyist magazines lately about their new super

modem,	the  Smartmodem.   Designed as part of their new line of

external computer components  including	 an  outboard  real-time

clock,	 the   Smartmodem  is  a  compact,  attractively  styled

direct-connect modem  that interfaces to your computer through a

standard serial	port.  For this	reason,	the Smartmodem is highly

portable -- you	can easily hook	it up to ANY computer that has a

serial port in no more	time  than  it	takes to plug in a DB-25

connector,  plug  in  the  power and flip the  switch.	  Easily

programmed for auto-answer,  auto-originate, and a host	of other

features, the Smartmodem touts brains and convenience over basic

performance,  an  item	I shall	dwell  on  extensively	in  this

review.

What sets the Smartmodem apart from all other modems on the

market today is	that it	has  a	built-in  microprocessor and the

ability	  to   accept	ASCII	strings	  as  commands.	     The

microprocessor,	 a Zilog Z-8, takes care of  all  the  intricate

programming  that users	 of  other  auto-answer,  auto-originate

modems might be	used to.  With Smartmodem, you just tell it what

you want to do,	 and sit back while it does it.	 No more worries

with timing loops,  delay  constants,  baud  rates, pulse rates,

counting  rings,  etc.;	 the  Smartmodem's  Z-8	and  onboard  2K

control	 program  in ROM does it all for you.	Programming  the

Smartmodem is so simple	 that all you need is a	bare-bones modem

program	that can transmit and receive data to it.  Then	you just

send it	commands to execute and	wait for it to tell you	that its

done.	 No  special software  is  needed  to  run  it,	 but  to

bastardize Shaw, what price convenience?

The Smartmodem  is	 a  Bell  103-compatible  direct connect

modem that is capable of running 0-300 baud in either  originate

or  answer  mode.  It has auto-originate/dialing and auto-answer

capabilities.	  It  can  dial	 using	either	Touch-Tones,  or

conventional pulse-dialing, or any combination of the two, along

with such niceities  as	 pausing for a second dial-tone	(such as

with a PBX system), dialing  in	either answer or originate mode,

and a built-in loudspeaker to allow you	to monitor the telephone

line  aurally.	 The modem can be  programmed  to  automatically

answer the phone  on  any number of rings from 1 to 255, and you

can also program how long  the	modem  will  wait  for a carrier

before	hanging	 up.  Seven LEDs on the	front panel display  the

modem's	 status	 including  auto-answer	 mode,	carrier	 detect,

off-hook, receive  data, transmit data,	terminal/computer ready,

and modem ready.  When	powered-up and on-line,	the Smartmodem's

low-profile  silhouette	 and   blinking	 lights	 make  a  flashy

appearance sitting on your terminal.

Smartmodem	 is  tall on features,	some  of  which	 can  be

programmed in as power-up  default  values  by	the  use  of dip

switches  located  behind  its front panel.  Almost all	defaults

can be overridden by explicit  commands	 when on-line.	Briefly,

lets take a look at some of Smartmodem's smarts.

Each command is sent to Smartmodem's  data	 port just as if

you  were transmitting data to the remote modem.  If  Smartmodem

is in  "command"  mode,	then all data sent to its data port will

be intercepted and  interpreted	 as commands to	the modem.  When

Smartmodem is in the "on-line" mode,  then all data will be sent

on down	the line as with a conventional	 modem.	   So  in actual

practice,  you really have only	to worry with one port for  both

data communication  and	 modem control.	 Your own modem	program,

primitive or intelligent only  has  to	watch the modem's serial

status port to determine when data is available	 to be input, or

when the modem can accept more data.  In this respect,	you talk

to  Smartmodem just as you would any other dumb	modem, with  the

difference  being  that	 the  modem  can  either  be  on-line or

off-line.  In practice,	I found	this to	be convenient but at the

same  time annoying since I was	often sending commands when  the

modem was  on-line  (and  so the remote	computer got a string of

garbage), and at other times  I	 thought I was transmitting data

when all I was doing was filling Smartmodem's  off-line	 command

buffer	with  meaningless instructions.	 Hayes stresses	in their

manual that any	 terminal  software  must keep up with what mode

the modem is in, since there are no status flags to interrogate.

When in the off-line or command mode,  commands are sent to

Smartmodem as a	string of upper-case ASCII characters terminated

with a carriage	return.	 A minor annoyance is  that Smartmodem's

control	program	doesn't	convert	lower-case to upper-case  before

interpreting  the  command  line.    So	 if  you're  programming

Smartmodem  from  the keyboard,	you have to make sure everything

is in upper-case.   Smartmodem's command buffer	is 40 characters

long, and if you overflow  its	buffer	with  commands or a long

telephone number to dial, it forgets everything	 and  makes  you

start  over.	When  Smartmodem has executed a	command, it will

return a result	code to	 let  you  know	what it	did.  The result

code can either	be a string of ASCII  characters  (preceded with

and followed by	a carriage return-line feed pair),  or	a single

ASCII  digit followed with a carriage return.  In addition,  you

can also  instruct  Smartmodem	to go into a quiet mode	where it

will  not return result	codes at all.	 All  result  codes  are

returned  to  the  host	 computer or terminal via the data port,

just  as  if  it  were	data from  a  distant  terminal.    Very

convenient.  The result	codes that Smartmodem can return are:

VERBOSE    NON-VERBOSE    MEANING

-------    --- -------    -------

OK		   0	     Command line executed

CONNECT	   1	     Carrier has been detected

RING		   2	     Phone is ringing

NO CARRIER	   3	     Carrier lost or never detected

ERROR	   4	     Command line error

Remember that each	of  the	verbose	strings	are preceded and

tagged with a CR/LF pair, and  that  the  non-verbose  codes are

returned  as  ASCII (i.e., VERBOSE would return	0DH 0AH	4FH  4BH

0DH 0AH	and NON-VERBOSE	would return 30H 0DH).	Hayes recommends

that  verbose  mode  be	 programmed  when  the	modem  is  being

controlled from	 the keyboard, and that	non-verbose be specified

when the modem is  being controlled by a terminal program.  Even

so, I didn't like having  to  read two characters in non-verbose

mode.	 If  you  choose  to just grab	the  first  ASCII  digit

Smartmodem returns and don't  input  the  carriage  return, your

UART will flag an overrun error	the next time Smartmodem sends a

result	code or	some serial data to your computer.  Again,  what

price convenience.

Assuming  that  Smartmodem	is in the off-line/command mode,

here's a thumbnail sketch  of  just  what you can tell it to do.

Each  command  line must be preceded with  the	characters  "AT"

which instructs	 Smartmodem  to	pay attention -	commands follow.

Then  each  command  consists of  strings  of  upper-case  ASCII

characters which can tell  Smartmodem  to  (optional  parameters

omitted	for brevity):

A  -	Answer telephone and wait for carrier detect

A/ -	Execute	last command buffer again

C  -	Turn transmitter carrier on or off

D  -	Dial telephone in originate mode and go	on-line

E  -	Toggle command line character echo

F  -	Set full or half-duplex

H  -	Go off-hook or on-hook

M  -	Turn built-in monitor speaker on or off

O  -	Return to on-line state

P  -	Use pulse dialing

Q  -	Enable/disable result codes ("quiet" mode)

R  -	Use answer mode	to dial	a telephone number

S  -	Read or	set any	of the 17 parameter registers

T  -	Use Touch-Tone dialing

V  -	Set verbose/non-verbose	mode for result	codes

Z  -	Reset Smartmodem to power-up default state

;  -	Terminate dialing and return to	command	mode

,  -	Command	line pause character

_  -	Morse code dah symbol

.  -	Morse code dit symbol

/  -	Morse code character pause symbol

The "S" command extends preceding instruction set	allowing

you  to	interrogate or set the contents	of 17 registers	used  by

Smartmodem  to	control	what its doing.	 These 17 registers are:

S0  - Set ring # to auto-answer on

S1  - Number	of rings which have occurred

S2  - Character used	to escape back to command mode

S3  - Character used	as carriage return in command mode

S4  - Character used	as line	feed in	command	mode

S5  - Character used	as backspace in	command	mode

S6  - # of seconds to wait for dial tone before dialing

S7  - # of seconds to wait for carrier detect

S8  - # of seconds of to pause for command line comma

S9  - # of 10ths/sec	carrier	must be	present	to be valid

S10 - # of 10ths/sec	between	loss of	carrier	and hangup

S11 - Touch-Tone dialing speed

S12 - Escape	code guard time	in 50ths of a second

S13 - Bit map of Smartmodem's UART status register

S14 - Bit map of Smartmodem's option	register

S15 - Bit map of Smartmodem's flag register

S16 - Enable	or disable loop	back for modem testing

The 3 bit-mapped  registers  allow	 you  to interrogate the

following:

S13 - Parity	enabled	or disabled, parity odd	or even, word

length	(7 or 8	data bits), command buffer status

(full or overflow), state of data bit 8).

S14 - Auto-answer default mode on power-up, quiet mode

enabled or disabled, verbose or non-verbose mode,

baud rate has or has not been acquired, monitor

speaker mode ("off", "on" until carrier detect,

or "on" at all	times).

S15 - Originate/answer mode,	full or	half-duplex mode,

Touch-Tone or pulse dialing mode.

In	 addition to the 17 registers, recall that  I  mentioned

the dip	switches that can be set to control the	modes Smartmodem

will default  to on power-up.  To gain access to these switches,

you have to pry	 off  Smartmodem's front panel,	and the	switches

control	the following conditions and modes:

Switch 1 - Enable computer control of data terminal ready

(DTR) line or force Smartmodem to	always hold

DTR high.

Switch 2 - Verbose/non-verbose mode

Switch 3 - Quiet/non-quiet mode

Switch 4 - Command line echo	on or off

Switch 5 - Auto-answer on 1st ring on or off

Switch 6 - Enable/disable computer detection	of remote

carrier detect

Switch 7 - Single/Multi-line	phone installation (controls

phone lights)

All of the	dip switch  defaults  except  2, 6, and	7 can be

overridden  after  power-up  by	 simply	 issuing  countermanding

instructions  to  Smartmodem.	 Switches  2,  6  and  7 control

hardware-dependant  features  and  thus	cannot be overridden  in

command	mode.

Although most of the commands,  and  mode toggles described

above  are  self-  explanatory,	 some  of them aren't,	so  I'll

elaborate  on some of them now.	 First off,  how  do  you  issue

commands to  Smartmodem	 once  we're  in  on-line  mode?   Quite

simply,	 you  can't!	What you have to do is get back	into the

off-line/command mode and then start issuing commands.	While in

the on-line mode, Smartmodem  checks  all  data	 transmitted  or

received  for an "escape code" character.  If Smartmodem detects

three consecutive  escape codes	preceded and followed by a delay

time during which no  data is being transferred, then Smartmodem

will abort back	to command  mode.   You	will note that registers

S2 and S12 allow you to	set the	escape	code  (default is a plus

sign "+"), and the delay or "guard" time (default  is 1	second).

In  other words, assuming that S2 and S12 have not been	 changed

by the	user  and  Smartmodem  is  on-line,  the modem will STAY

on-line	 until	it  detects  at	least 1	second of  no  activity,

followed by 3 consecutive plus	signs,	followed  by  at least 1

more  second of	no activity.  Theoretically, the only time  this

condition  would  be met would be if the user specifically typed

these three characters,	 but  it  is  not inconceivable	that the

remote	computer  could	 coincidentally	send  this  sequence  of

characters, with the appropriate  guard	 times,	and thereby take

your  Smartmodem  off-line!    (Also,  the  remote   user  could

deliberately  take  your  modem	 off-line  by sending the escape

sequence.  Smartmodem doesn't care whether the	escape	sequence

is entered locally or from the remote computer -- it will  abort

to  command  mode  regardless).	  In this instance, the	only way

that  your computer would know that  Smartmodem	 was  no  longer

on-line	 would	be  for	it monitor the remote carrier detect (an

option	controlled  by	dip  switch  6).    Although  Smartmodem

continually monitors the remote	carrier	regardless of whether it

is on or off-line, and	displays  remote carrier status	with the

front-panel LED, the carrier detect  line  it makes available to

your  computer is only held high IF remote carrier  is	detected

AND Smartmodem	is on-line.  However, when using Smartmodem on a

remote	bulletin  board	 system,  the  escape  sequence	 can  be

disabled entirely.   If	you have done this, the	only way to take

Smartmodem off-line (short  of	pulling	 its  plug) is to either

lose  remote carrier detect, or	for your computer  to  pull  the

data  terminal	ready  line  low.    Either  one  of  these  two

conditions  will  then force Smartmodem	to disconnect and revert

to command mode.

A drawback	to  Smartmodem's escape	routine	is that	you will

miss any incoming data	while the modem	is in command mode.  For

example, if want to change  from half-duplex to	full-duplex, you

first have to get the remote  computer	to  wait  by  using XOFF

(assuming  it  can  or	will  respond  to  XOFF),  then	get into

Smartmodem's command mode, issue the duplex mode change	command,

then get back into on-line mode	and tell the remote  computer to

carry  on.   If	you can't get the remote computer to halt  while

you do your thing,  then  you  only  have  two	options: stay in

half-duplex,  or  change  to full-duplex and lose whatever  data

comes in while you're changing	modes.	 Here again, convenience

at what	cost?  Personally, I would prefer  being  able	to issue

commands  to  the  modem  independently	 of  data  transfer  and

sacrifice   convenience	  for	flexibility.	Naturally,  such

flexibility  would  eliminate  the  possibility	 of  controlling

Smartmodem   from  a  dumb  terminal  without	an   interfacing

communications program.	 You can't have	both.

You may have  noted  the  Morse code dit and dah symbols in

the command table.  Yes	friends,  Smartmodem can send Morse code

(up to the limit of a single command buffer) using either answer

or originate tones at a	fixed speed  of	 20  words  per	 minute.

Couple	this  capability  with	Smartmodem's  ability  to turn a

transmitter on or off (or lots of other	things as well),  and we

amateur	 radio	operators have a neat little package for sending

ASCII over our radios at up to 300 baud.  The Morse code feature

allows the computer  to	 identify  each	station	at the 10 minute

intervals demanded by the FCC without  the  operators  having to

intervene.    However, those of	you with other super modems  can

have the same capability  if  you're willing to	do a little more

work.  Smartmodem just makes it	more convenient.

Those other special command  mode characters also deserve a

little	more  explanation.  Normally,  when  you  issue	 a  dial

command, Smartmodem goes off-hook (picks up the	phone),	waits at

least  2  seconds   for	  the  dial  tone  (the	 delay	time  is

programmable of	course), and  then  proceeds to	dial.  If you're

dialing	from a PBX system where	you have  to  dial one number to

get  an	 outside  line,	 Smartmodem  can handle	that  by  giving

Smartmodem the outside line access number  (say	 9),a comma, and

then the number	to dial.  Smartmodem will then dial  9,	wait for

the  number  of	seconds	programmed for the comma, and then  dial

the rest of the	 number.    As	mentioned  earlier, you	can also

switch between pulse dialing and Touch-Tone  dialing in	the same

dialing	operation.  For	example, if your PBX system doesn't have

Touch-Tone capability but your outside lines do,  you  can  have

Smartmodem  dial  9  with pulses, wait 2 seconds for the outside

line dial tone,	then dial  the outside number using Touch-Tones.

The advantage of this setup  is	 that  Smartmodem can Touch-Tone

dial a number incredibly fast (I mean, REAL fast), and if you're

in  a hurry, dialing an	11 digit number	with  Touch-Tones  is  a

hell  of  a  lot-faster	 than  using  pulses, even at the 20 pps

accepted by some exchanges.  Smartmodem's  Touch-Tone capability

also  allows  you  to  use  your  computer with	the  new-fangled

computerized banking systems where you call your bank's	computer

and   then  use	 your  telephone's  Touch-Tone	 pad   to   give

instructions to	the bank computer.  Smartmodem can automate this

process	 by  using  the	 semi-colon  at	 the  end  of  the  bank

computer's  phone number.  Normally, Smartmodem	goes on-line and

waits for a carrier  after dialing a number.  But the semi-colon

forces it to revert to	command	 mode  after  dialing so you can

give it	further	commands.  Switch the speaker on so you	can hear

the bank computer talk to you, and have	Smartmodem  talk  to the

computer with Touch-Tones!  Incredible!

The astute	reader has probably concluded that I really like

this little gadget, and	I do.  But I don't want	one to have  and

to  hold  for  my  very	own forever and	ever, because Smartmodem

does have some limitations  that  I  don't  particularly want to

live with.  Before I go	into the negatives, please remember that

I  am speaking from my own experience, and with	 an  eye  toward

what  I	 personally  want  to  do  with	a super	modem.	For you,

Smartmodem might be just the ticket  --	 but  not  for	me,  and

here's why.

Let  me  say right	off that for folks who don't plan to use

Smartmodem for any kind	of bulletin board or RCP/M applications,

either as a user or as a SYSOP,	Smartmodem is a	good buy for the

money.	It has	all  kinds  of	bells  and  whistles  that don't

require	 an intimate knowledge of assembly language and	 digital

electronics  to	implement.  In a word, Smartmodem is CONVENIENT,

and even at the	 retail	 price of $279.95, its a steal.	 But for

people like myself who aren't  afraid  of  getting  their  hands

dirty  at the machine language level, who don't	enjoy paying  Ma

Bell more  than	 she deserves, and who want maximum flexibility,

Smartmodem's convenience becomes an insufferable burden	to bear.

First and foremost,Smartmodem can only go up to	300 baud.  While

the vast majority  of  bulletin	 boards	 and  timeshare	services

can't  go  any	faster either (not with	Bell 103-compatibility),

the majority of	the  RCP/M  systems that I regularly do	business

with can and do	run at 450 baud	 and  higher.	 When you figure

that  a	 file can be transferred in half the time at  600  baud,

you're talking about a considerable savings on phone bills.  For

heavy dialers, that savings could well pay for the extra cost of

a high-speed  modem  and  then	some.  From the	SYSOP's	point of

view, even with	alternative long-distance  services available, a

lot of RCP/M users are going to	higher baud  rates  and	an RCP/M

system	worthy	of  the	name should be able to accomodate  those

people.

Secondly, even though  Smartmodem	can  run from 0-300 bps,

you can't change the baud rate on the fly.  Smartmodem gets your

baud rate, word	length,	and parity mode	the first time you issue

it a command, and that's where she stays.  If  you  power-up  at

300  baud,  and	 a user	calls in at 110	baud, tough nookies.  So

the 0-300 bps capability is there, but with a big "gotcha".

Thirdly,  Smartmodem   is	 timing-oriented   rather   than

logic-oriented.	   For	example, after dialing another computer,

Smartmodem can't tell when  the	 other	phone is ringing or when

its busy.  It simply waits a certain amount  of	time (default of

30 seconds) for	a carrier.  If it doesn't get one in that length

of time, it disconnects.  This limitation means	 that Smartmodem

can't  be  used	to dial	a ring-back system without the	operator

sitting	there  riding  shotgun and manually issuing a command to

disconnect on the first	 ring,	and  then  dial	again.	In other

words,	you  can  do it	under operator control,	 but  not  under

software control.   However,  giving credit where credit is due,

Smartmodem can handle ring-back	 ANSWERING,  as	a bulletin board

modem driver can simply	monitor	the rings register  just as with

any other auto-answer modem -- only with Smartmodem its	 quite a

bit more awkward.

From  the	programmer's  standpoint, Smartmodem is	slow  to

program	and slow to respond to	programming.   It takes	at least

1/4 of a second	for the	modem program to accept	 a  command  and

issue  a response code.	 Plus, the speed with which commands and

result codes  can  be  transferred  is limited by the baud rate.

Aside  from being slow,	Smartmodem is  awkward.	   ASCII  string

programming  is	 convenient  for  BASIC,  but  at  the	assembly

language  level,  its  very  inefficient  to have to run through

lookup	tables and look	for 2 result codes instead  of	1.    It

seems the  folks at Hayes were going primarily after the turnkey

user in	designing  Smartmodem, and the end product is emminently

suited for them.  To  an  end  user conversant with a high-level

language such as BASIC,	with only  limited knowledge of	assembly

language (and no desire	to learn more),	 Smartmodem  is	a little

jewel.	But to someone such as myself who prefers to program  in

assembly  language, Smartmodem is like a black hassle box that I

can't get into	deep enough to have it singing and dancing to MY

tune, instead of  to  the  tune	 D.  C.	Hayes wrote for	it.  For

example,  I  don't  like the way you have  to  escape  from  the

on-line	mode.  If you  send  Smartmodem	the escape sequence, the

other end gets it too.	If you disable the escape sequence, then

the only way you can get back  to  command  mode  is by	fiddling

with the DTR line -- and that will disconnect you from the phone

line.  My basic	gripe then is that I want more control	over the

modem than what	the law	allows.

Although	Hayes	doesn't	 publish  much	in  the	 way  of

meaningful specifications  for Smartmodem, my experience with it

on-line	leads me to believe it leaves a	bit to be desired as far

as sensitivity and selectivity	are concerned.	Connections that

never  gave my acoustically- coupled  Datec  any  problems  have

kicked Smartmodem  off the track.  In all honesty however, those

same marginal lines  also  gave	 a lot of other	modems I've used

trouble	 as  well.  I would say	that  Smartmodem's  performance,

although less than ideal, is par for the course.  And finally, I

don't like having  yet	another	 serial	 port  tied  down to yet

another	modem.	I would	much prefer to have a modem board that I

can  plug  into	 my S-100 buss.	 Motherboard slots are free  and

available - serial  ports cost money (and what with the	price of

gold these days, DB-25s	ain't cheap either).

So	there you have it, the D. C. Hayes Smartmodem.	Now that

I have completed my evaluation	of  the	 little	 booger,  I have

reluctantly  decided not to give him a home in my computer room.

To the question	 I posed quite a few words back	about what price

convenience, my	answer	is  flexibility.    Smartmodem	has more

bells  and  whistles  than a merry-go-round, but I frankly don't

need them.  If I could summarize Smartmodem in a phrase, I would

have to	say that its TOO  SMART!   For me, and for most	CBBS and

RCP/M operators, the Potomac Micro-Magic  MM-103  is  still  the

best  show  on	the  road  .. but Smartmodem sure puts on a good

warmup act.

############################################################################################################################