# Migration from BIOSuite C54x to C5000 Code Composer Studio

This application note explains how to convert TMS320C54x applications developed with BIOSuite v1.1.2 to Code Composer Studio projects. You will need to transfer objects from the old configuration file to a new configuration file. In addition, you will need to make some small changes to the application source code.

## Configuration File Modifications

- Use Code Composer Studio to open the configuration file (*.cdb) developed with BIOSuite. Select FileOpen. In the Open dialog box, specify the name of your configuration file and click Open. Your configuration file is opened in an edit window within Code Composer Studio.

- Create a new configuration file with Code Composer Studio. Select FileNewDSP/BIOS Configuration. In the New dialog box, select the appropriate configuration template for your DSP chip and board and click OK. You should have two configuration files open within Code Composer Studio at this point.

- Copy all objects from the HST, IDL, LOG, PIP, PRD, CLK, and STS module managers in the old configuration to the new configuration file using copy and paste.

a.	In the old configuration file, expand the object lists for the following module managers:
HST, IDL, LOG, PIP, PRD, CLK, and STS.

b.	Select an object listed below any of the module managers.

c.	Press Ctrl+C to copy the selected object.

d.	Move to the new configuration file and select the matching module manager.

e.	Press Ctrl+V to paste the object into the new configuration file.

Repeat steps b through e for all HST, IDL, LOG, PIP, PRD, CLK, and STS objects.

- The "Signal Manager" has been changed to the "Software Interrupt Manager".  Manually recreate any SIG objects as SWI objects in the new configuration file. Set the properties to match the properties of the corresponding objects in the old configuration file.

- Manually compare the property settings for module managers in the old configuration file to the settings in the new configuration file. Change the properties in the new configuration file as necessary.

- For the Global Settings, verify that the DSP MIPS (CLKOUT) value is correct.

- For the Global Settings, verify that the new PMST value matches the value in the old configuration file.

- For the Clock manager, verify that the new Microseconds/Int value matches the value in the old configuration file.  This value defines the system tick for your application.

- Transfer all HWI (Hardware Interrupt Service Routines) from the old configuration file to the new configuration file.

- For the Memory Section Manager, verify that all the memory sections in the new configuration file correspond to the ones in the old configuration file (e.g., VECT, IPROG, IDATA, etc).

See the table below for notes about properties that have changed names or meanings. There are also some new properties that did not exist in the old configuration files.

| Module Name | Old Property Name | New Property Name |
| --- | --- | --- |
| Global Settings | Target Board Driver | No longer Used |
| Memory Section Manager | Application Stack Size | Stack Size (MAUs) |
| Memory Section Manager | Application Stack Memory | Stack Section (.stack) |
| Memory Section Manager | Application Constants Memory | Constant Section (.const) |
| Memory Section Manager | (User defined linker command file) | Text Section (.text) |
| Memory Section Manager | BIOS Program Memory | BIOS Code Section (.bios) |
| Memory Section Manager | BIOS Data Memory | Data Sections (.data, .cio, .sysmem) |
| Memory Section Manager | BIOS Heap Memory | N/A |
| Memory Section Manager | BIOS Startup Code Memory | Startup Code Section (.sysinit) |
| Memory Section Manager | (User defined linker command file) | C Initialization Section (.cinit) |
| Memory Section Manager | (User defined linker command file) | Uninitialized Sections (.bss, .far) |

## Changes to Application Source Code

Make the following changes in your application source code:

- Change all instances of SIG_ to SWI_ in the code.

- Replace all sig.h includes with swi.h includes.

- Add an additional parameter of 0xffff to all HWI_enter and HWI_exit calls to get the non-nested interrupt behavior that was used in DSP/BIOS v1.1.2. For example, change this call:

HWI_enter	C54_CNOTPRESERVED

to the following:

HWI_enter 	C54_CNOTPRESERVED, 0ffffh