- STRIPHEX -

USAGE:

STRIPHEX <input PROM file name> <output file name>

The purpose of 'STRIPHEX' is to strip the PROM formatting information

from a PROM file.  'STRIPHEX' can be used in conjunction with the

'HEX2BITS' program to produce an ascii representation of a LCA configuration

bit file.

PROM formatting information is originally added to a data file so that

a PROM Programmer can correctly address and load a PROM. This information

consists of addresses, checksums, character counts, and other features

specific to the type of PROM format used.

The MAKEPROM program in XACT takes the 'raw' bitstream information of

a design, and makes a PROM formatted file of the bitstream data.  This PROM

formatted file can then be loaded into a PROM via a PROM Programmer, to

eventually be read by a Xilinx LCA during it's configuration process.

During the process of shifting the data from the programmed PROM to

the LCA, each byte of data is 'bit-swapped' (ie. bit 7 of the byte becomes

bit 0, bit 6 becomes bit 1, ...).  In order to nullify this effect,

MAKEPROM also 'bit-swaps' each byte of bitstream data before placing it in

the PROM formatted file.

Because the data in a PROM file has been 'bit-swapped', a 'stripped'

PROM file will not directly correspond to the original bitstream data.

However, the 'stripped' file can be submitted to the 'HEX2BITS' program

which will 'bit-swap' each byte of data again as it produces an

ascii '1's and '0's version of the hexadecimal file.  The final file

produced by 'HEX2BITS', will be a correctly ordered ascii '1's and '0's

representation of the original bitstream data.

'STRIPHEX' will strip the formatting information from a PROM file

that is in either the Intel MCS-Hexadecimal Object, Motorola Exormax, or

Tektronix Hexadecimal format.

SAMPLE

------

This Intel MCS-86 Hexadecimal PROM-formatted file....

---------------------------------------------------------

|    :020000020000FC					|

|    :10000000FF04400FFBEEFFFFFFFFFFFFFFFFB7FD09	|

|    :10001000FDFBFFFBF7F7FFBFFFF7EDFFEFDFFFFF94	|

|    :10002000FFFDFFFDFFFFFFFFFFFFEFFFFDFBFBFB02	|

|    .........						|

|    :1005D000F9F3FBF3E7E6F6DFFFFDFFFFFF7F777739	|

|    :0205E000FFFF1B					|

|    :00000001FF					|

---------------------------------------------------------

submitted to 'STRIPHEX', produces the following file...

-----------------------------------------

|    FF04400FFBEEFFFFFFFFFFFFFFFFB7FD   |

|    FDFBFFFBF7F7FFBFFFF7EDFFEFDFFFFF   |

|    FFFDFFFDFFFFFFFFFFFFEFFFFDFBFBFB   |

|    .....				|

|    F9F3FBF3E7E6F6DFFFFDFFFFFF7F7777   |

|    FFFF				|

-----------------------------------------

When the stripped file above is submitted to 'HEX2BITS',

the following file is produced....

-------------------------------------------------------------------------------

| Produced by Xilinx HEX2BITS version 0.02				      |

| File dice.bits							      |

| Wed Aug 17 14:41:13 1988						      |

| 1111111100100000000000101111000011011111				      |

| 011101111111111111111111111111111111111111111111111111111111111111111111111 |

| 011011011111110111111110111111111111111011111111011111110111111111111111111 |

| 011111111111101111101101111111111111110111111110111111111111111111111111111 |

| ...									      |

| 001111110111111001111111001111110111111100111111100111011001110110111111111 |

| 011111111111011111111111111111111111111111111111110111011101110111011111111 |

| 11111111								      |

-------------------------------------------------------------------------------

DISCLAIMER

==========

XILINX IS PROVIDING THIS SOFTWARE TO USERS OF XILINX PRODUCTS SOLELY FOR

USE IN SUPPORT OF THOSE PRODUCTS. XILINX DOES NOT ASSUME ANY LIABILITY

ARISING OUT OF THE APPLICATION OR USE OF THIS SOFTWARE OR FOR ANY ERRORS

OR DEFECTS THAT MAY EXIST IN THIS SOFTWARE. XILINX EXPRESSLY DISCLAIMS ANY

WARRANTY, EXPRESS OR IMPLIED, INCLUDING ANY WARRANTY OF MERCHANTABILITY OR

FITNESS FOR A PARTICULAR PURPOSE.