# Design Units

Overview

Each VHDL entity consists of an entity declaration and an architecture body. In addition, packages may be defined to store subprograms, types, constants, and component declarations that may be used by any behavioral description.

A VHDL source file consists of either:

- Entity declarations, with their corresponding architecture bodies

- A package declaration, with its corresponding package body

- A package declaration and body followed by entity declarations and architecture bodies

Context clauses may appear immediately before each entity declaration and package declaration, to import subprograms, types, constants, and components.

This chapter describes entity declarations, architecture bodies, packages, subprograms, and context clauses.

Entity Declaration

What Is an Entity Declaration?

The entity declaration defines an entity’s name and its interface. It presents the external view of a component.

### Entity Declaration Format

entity entity_name is
  [generic_declarations]
  [port_declarations]
end [entity_name];

Ports

The entity declaration declares the component’s “ports,” or input and output signals. Each port corresponds to a pin or bus_pin on the component’s ViewDraw symbol.

Ports must be signals of type STD_LOGIC, BIT, STD_LOGIC_VECTOR, BIT_VECTOR, or INTEGER; they may not be variables or constants, and may not be of any other predefined or user–defined type. All ports must be signals.

Each port has a “mode,” which defines its direction. Modes may be in, out, inout or buffer. Within a design entity:

- in ports may be read, but not written

- out ports may be driven, and may not be read

- inout ports may be driven, as well as read

- buffer ports may be both driven and read. However, unlike inout ports, no outside entity may drive such ports.

Ports may be given default values, which are assigned before simulation begins, but such default values are ignored by synthesis. in ports that are connected (i.e., those which are driven by external drivers) are not assigned internal default values from their declarations. If no default value is specified, the default value of each element is X.

Ports are signals that are externally visible. They may be used within the behavioral model as signals, and referenced and assigned using the same naming constructs (such as indices and slices) and statements as signals declared in signal declarations.

### Port Declaration Format

port
  ([signal] port_name {, port_name }: mode 
    type [(index_range [, index_range])] [:= default_value] 
  {; [signal] port_name {, port_name}: mode
    type [(index_range [, index_range])] [:=default_value]});

Generics

The entity declaration may also contain declarations of parameters that customize the behavior of the entity for a particular instance. These parameters may be given values when an entity is instantiated in ViewDraw, by assigning values to the corresponding component’s attributes. The parameters are called “generics.” Each generic corresponds to the ViewDraw component attribute of the same name.

Generics may also be given default values, which are used whenever an attribute of an instantiated component is not assigned. If no default value is specified, the default value of a generic is the “left-most” value of the object’s type. (These values correspond to the IEEE 1076-1993 type attribute T’left.)

Generics may be used within the behavioral model of an entity as constants; for instance, delay values could be specified by this method. The differences between constants and generics are that constant values may not be assigned through ViewDraw attributes, and generics may not be referenced in object declarations.

In synthesis, generics are treated as constants. For example, an n-bit adder could be composed in VHDL where “n” is an Integer generic with an initial value.

### Generic Declaration Format

generic
  ([constant] generic_name {, generic_name}: [in] 
            type [:= default_value] 
  {; [constant] generic_name {, generic_name}: [in]
            type [:=default_value]});

Examples

entity ALU is   –– entity name is ”ALU”
	generic(COUT_DELAY:  TIME := 12 ns;   –– two generic delays
	       REG_DELAY:   TIME :=  4 ns);  –– with default values
	port(DOUT:   out STD_LOGIC_VECTOR(0 to 15);
			                            –– 16–bit output signal
	    COUT:   out STD_LOGIC;         –– 1–bit output signal
	    OP_CODE: in  STD_LOGIC_VECTOR(0 to 1);        
											–– 2–bit input signal
	    A, B:   in  STD_LOGIC_VECTOR(0 to 15);
											–– two 16–bit inputs
	    CIN:    in STD_LOGIC;    -- 1–bit input signal
	    CLK:    in STD_LOGIC);   -- 1–bit input signal
end ALU;  –– end entity declaration

The VHDL below describes an n-bit increment logic using the generate statement with a generic parameter.

entity INCR is
  generic (N: INTEGER := 4);
  port    (XIN    : in BIT_VECTOR(0 to N – 1);
          XOUT   : out BIT_VECTOR(0 to N – 1));
end INCR;

architecture STRUCT of INCR is
  signal TMP: BIT_VECTOR(0 to N–1);
begin
  XOUT(N–1) <= not XIN(N–1);
  TMP(N-1) <= XIN(N – 1);
  LABEL1: for I in N – 2 downto 0 generate
    XOUT(I) <= XIN(I) xor TMP(I + 1);
    TMP(I)  <= XIN(I) and TMP(I + 1);
  end generate LABEL1;
end STRUCT;

Architecture Body

What Is an Architecture Body?

An entity’s architecture body defines its behavior. Each architecture body is associated, by name, with one entity declaration. Each entity declaration has exactly one associated architecture body.

Parts of the Architecture Body

An architecture body contains two parts:

- A declarative part

- A set of statements

The declarative part of an architecture body consists of constant, component, signal, and type declarations. It declares items that are visible only within the architecture body.

The set of statements within an architecture body includes block statements, process statements, and concurrent procedure call, assertion, and signal assignment statements. Except for the executable block statements, which are blocks of other concurrent statements, all statements in this set are executed concurrently.

### Architecture Body Format

architecture architecture_name of entity_name is
	[constant_declarations] 
	[signal_declarations] 
	[type_declarations] 
	[component_declarations] 
	[attribute_declarations] 
	[attribute_specifications] 
begin
	[concurrent_statements] 
end [architecture_name];

### Architecture Body Example

architecture BEHAVIOR of ALU is
	signal ALUOUT: STD_LOGIC_VECTOR(0 to 16); 
	signal RESREG: STD_LOGIC_VECTOR(0 to 15); 
                                  –– internal signal arrays
	type OP_TYPE is (OPADD, OPOR, OPAND, OPXOR); –– op–code type
begin
.
.
.
end BEHAVIOR;   –– end of architecture body

Block Statement

Each block statement defines one hierarchically bounded set of concurrent statements. Items defined within each block statement are visible only within their enclosing block. block statements only define symbol scopes; they imply structure, but do not define behavior.

The scope of constants, signals, and types declared within the block begins when they are declared and ends with the end of the block.

### Block Statement Format

block_label: 
block
	[constant_declarations]
	[signal_declarations ] 
	[type_declarations]
	[component_declarations ] 
	[attribute_declarations] 
	[attribute_specifications ]
begin
	[concurrent_statements] 
end block [block_label];

### Block Statement Example

LATCH: block
  signal LOCAL_CLK: STD_LOGIC := ’0’; –– ”local” clock signal
begin 
–– the following two concurrent statements execute concurrently
	CLOCK_GEN: 			–– concurrent signal assignment
            			–– generate clock signal
		LOCAL_CLK <= not LOCAL_CLK after 1 ns;


	LATCH_INSTANCE:	–– concurrent procedure call 
							–– ”instantiate” latch
		LATCH_PROCEDURE (LOCAL_CLK, DATA, LATCHED_DATA);
end block LATCH;

Component Instantiation Statement

A component instantiation defines the actual “wiring up” of a component within the architecture. Local signals are connected to ports of the component. Local values can be associated with the component generics to modify the behavior of the model.

The VHDL analyzer insures that the component instantiation matches its component declaration and that the component declaration matches its entity declaration.

The component name must reference a previously declared component which is visible at the point of instantiation. The component may be visible directly (declared in the current architecture) or via the use statement (declared in a package).

There is no limit on the number of component instantiations, or the levels of hierarchy created through component instantiation.

### Component Instantiation Statement Format

label: component_name
[generic map (generic_association_list ) ]
port map (port_association_list ) ;

### Examples

This instantiation of the NOT1 component supplies local delay values (which are ignored by synthesis) via the generic map statement and uses named association to connect the local signals NSTATE and STATE to the ports QN and A, respectively.

U1: NOT1
  generic map (2 ns, 3 ns)
  port map (QN <= NSTATE, A <= STATE) ;

The following instantiation of the NOT1 component uses the default delay values by omitting the generic map statement and uses positional association to connect the local signals NSTATE and STATE to the first and second ports, respectively.

U2: NOT1
  port map (NSTATE, STATE);

Generic Map Association

Association of formal generics and actual values in a generic map aspect is made through positional association. The reserved word open may be supplied in place of a actual value. Using open specifies that the default value is used. open may only be used if the component declaration or the entity has a default value defined. If both the component declaration and the entity have a default defined, the component declaration’s default will be used.

The value which represents the actual generic value must be locally static (not an expression). Actual generic map values may be of the following types and classes:

- Scalar literals (integers and enumerations)

- Character string literals

- Constants (declared in the current architecture or an included package)

- Generics from the entity associated with the enclosing architecture

- Time literals, such as 5 ns.

The above constant and generic values may be of any pre-defined or user-defined type. They may not be sliced or indexed, however, and the entire name must be passed as the actual.

IMPORTANT:

Generic maps are for simulation only. They are ignored in synthesis.

Port Map Association

### Positional vs. Named Association

Association of actual signals to formal ports may be made using positional association. In this case, all ports declared in the component must have an actual signal (or the reserved word open) connected in the instantiation.

Association of actual signals to formal ports may also be made using named association. This form is more verbose but has the following advantages.

- This form is required if sub-elements of a composite formal need to be bound separately.

- When making connections, it may be easier to remember the port name than remember the port position.

- If a large number of ports will be left unconnected, it may be easier to just list those that will be connected.

If as described above, a sub-element of a composite formal is associated separately, then all the sub-elements of the formal must be associated and such associations must be contiguous. Also, the sub-element of the formal must be statically determinable at analysis time.

A formal port may also be left unassociated by applying the reserved word open as the actual designator (in named or positional association), or by omitting the association element altogether (in named association). Use of the reserved word open is shown in the following example.

### Example using OPEN

COMPONENT EX
	port (FA: in  STD_LOGIC := ’1’;
		  FB: in  STD_LOGIC;
		  FC: out STD_LOGIC );
end EX;
.



.
.
C1: EX port map (FA => open; -- Using OPEN 
				   FB => B;
				   FC => 0);
.
.

### Illegal Conditions

An error is issued for any ports of mode in which are open (left unconnected) and do not have a default value specified.

Ports of a component instance may be associated with local signals (defined in the architecture body or within a block) or with port signals of the associated entity. In the latter case, it is illegal to associate a port of mode in with a port signal of mode out (only in or inout), and a port of mode out with a port signal of mode in (only out or inout). A port of mode inout can only be associated with a port signal of mode inout.

Process Statement

The fundamental building blocks of architecture bodies are process statements. They define the values of their output signals as a function of their input signals, over time.

A process statement is a concurrent statement containing sequential statements.

Processes may contain all of the sequential statements defined in Chapter 6, except return statements. These statements are executed sequentially, in their order of appearance.

In synthesis, both clocked and combinational processes are supported

### Clocked Processes

Clocked processes specify clocked or registered logic. Clocked processes can take two styles:

1. Using wait statement

2. Using if statement

A wait statement, if used, must be the first statement in the process. The wait statement waits until the clock’s edge is ’1’ or ’0’, or the set/reset conditions are TRUE before continuing the process.

When there are no asynchronous set or resets in the circuit (or set and reset are FALSE) the process only executes once per clock edge, synchronized by the wait statement.

The following example illustrates a typical clocked process:

process
wait until (CLK’EVENT and CLK = ’1’) or (RESET = ’1’));
	if (RESET = ’1’) then
		BCD <= ”0000”;
	else
		BCD <= MUXZ;
	end if;
end process;

The same behavior can be written using if statements and a sensitivity list.  This approach is illustrated below:

process (RESET, CLK) begin
	if (RESET =’1’) then
		BCD <= ”0000”;
	elsif (CLK’EVENT and CLK = ’1’) then
		BCD <= MUXZ;
	end if;
end process;

A clocked process implies edge-triggered flip-flops for all signals. All assignments to signals inside a clocked process imply a flip-flop for that signal. Because signals do not take on a new value assigned to them until after the current simulation cycle, references to a signal in a clocked process always refers to the output of the flip-flop for the signal.

Set and reset are asynchronous signals (independent of the clock). When set or reset are made TRUE, the process is set or reset accordingly.

### Combinational Processes

A combinational process is used to specify combinational, not clocked, logic. A combinational process must conform to the following rules:

- It must have a sensitivity list that includes all of the signals used in the process.

A sensitivity list prevents situations where a value is not evaluated when one of its inputs changes because that input is not on the sensitivity list. Such a circuit could not be implemented. ViewSynthesis for Windows issues an error message when incomplete sensitivity lists are found.

- Variables in the process may not be used before being set.

Procedure calls are fully supported. The procedures called must conform to the rules described in the Subprogram section of this chapter.

### Process Statement Declarations

Process declarations may be one of the following:

[constant_declarations]
	[variable_declarations]
	[type_declarations]
	[attribute_declarations] 
	[attribute_specifications]

### Process Statement Format (Two Examples)

The sensitivity_signal_list shown in the following two format examples represents any signal from a list of signals that invokes the process when its value changes.

[process_label:] 
process (sensitivity_signal_list)   
				[process_declarations;]
begin
				[sequential_statements;] 
end process [process_label] ;

[process_label:] 
process 
				[process_declarations;]
begin
				[wait_statement;]
				[sequential_statements;] 
end process [process_label] ;

### Usage Example

REGISTER1: process
		variable VALUE: STD_LOGIC_VECTOR(1 to 16);          
											-- register VALUE variable
	begin
	-- the following sequential statements execute sequentially:
		wait until CLK = ’1’;		-- suspend until CLK rises
		VALUE := ALUOUT(1 to 16);	-- load input signal
		if ENABLED then
			RESREG <= VALUE;     	-- update output signal
		end if; 
	end process;

Concurrent Procedure Call

A concurrent procedure call represents a process statement that calls a procedure.

Each concurrent procedure call executes at the beginning of simulation. Its action consists of an invocation of the called procedure with the indicated parameters. After the procedure returns, the concurrent procedure call waits for an event to occur on any in or inout signal that appears in the parameter list, and then invokes the procedure again. It loops perpetually in this manner, invoking the procedure whenever an event occurs on an in or inout signal in the parameter list.

A concurrent procedure call whose parameter list includes no in or inout signals suspends indefinitely after the first invocation.

### Concurrent Procedure Call Format

[label:] procedure_call_statement

Example

-- this latch procedure loops indefinitely,
-- updating ”latched data”
	LATCH_INSTANCE:
		LATCH_PROCEDURE (LOCAL_CLK, DATA, LATCHED_DATA);

Concurrent Assertion Statement

Concurrent assertion statements are ignored in synthesis, but may be included in the VHDL description for later use during simulation.

### Concurrent Assertion Statement Format

[label:] assertion_statement

Example

assert (not BITUNKNOWN(SEL)) report ”bad select line”;

Concurrent Signal Assignment

### Conditional vs. Selected Signal Assignments

Each concurrent signal assignment is either a conditional signal assignment or a selected signal assignment. A conditional signal assignment assigns one of several expressions to a target signal. The expression to be assigned is chosen by evaluating the condition associated with each expression value. A selected signal assignment performs a similar function; it assigns one of several expressions to a target signal, choosing which expression to assign by comparing the value of a given condition to the selection value associated with each expression.

### Conditional Signal Assignment Format

[label:]
	signal_name <= 
		{expression when condition else } 
		expression;

The rules governing condition evaluation in conditional signal assignments are identical to those governing sequential if statements.

Any number of expressions and corresponding when conditions may be included. Conditions are evaluated in order of appearance until a TRUE condition is found or the final else is reached. The corresponding expression is assigned to the target signal whose name precedes the assignment operator (<=).

### Conditional Signal Assignment Example

-- choose a value for ALU_OUT by checking a series of conditions,
-- whenever ”A”, ”B”, ”CIN”, or ”OP_CODE” changes:
ALU_OUT <=	ADDEMUP (A, B, CIN)	when OP_CODE = OPADD else
          	’0’ & (A or B) 		when OP_CODE = OPOR  else
         	’0’ & (A and B)  		when OP_CODE = OPAND else
         	’0’ & (A xor B)  		when OP_CODE = OPXOR else
         	’X’ & X”XXXX”;

### Selected Signal Assignment Format

with selection_expression select
	signal_name <=
		waveform when choices
		{, waveform when choices};

Any number of waveforms and corresponding when choices may be included. Choices take the form:

choice { | choice}

The vertical bar (|) indicates that any of the given choices qualifies as a match with selection_expression. Every choice is either an expression, or a range (“expression to expression” or “expression downto expression”), or the keyword others.

The selection expression is evaluated and compared to the given choices until a match is found or the others clause is reached. The choice others matches any value not previously matched. The corresponding expression is assigned to the target signal whose name precedes the assignment operator (<=).

The rules governing choice evaluation in selected signal assignments are identical to those governing sequential case statements.

### Selected Signal Assignment Example

-- select a value for ALU_OUT by checking the value of 
-- ”OP_CODE”, whenever ”A”, ”B”, ”CIN”, or ”OP_CODE” changes:
with OP_CODE select
  ALU_OUT <= ADDEMUP (A, B, CIN) when OPADD,
           ’0’ & (A or B)      when OPOR,
           ’0’ & (A and B)     when OPAND,
           ’0’ & (A xor B)     when OPXOR,
           ’X’ & X”XXXX”       when others;

Generate Statement

A generate statement allows iterative or conditional generation of a portion of a description. The generate statement is used with for or if generation schemes as shown in the following generate statement format and example:

### Generate Statement Format

generate_statement ::=
	generate_label
		for <variable> in constant_range generate
			{concurrent_statement}
		end generate [generate_label];


generation_scheme ::=
	generate_label
		if condition generate
			(concurrent_statement)
		end generate [generate_label];

label ::= identifier

If a label appears at the end of a generate statement, it must repeat the generate label.

The constant_range used in the for generation scheme may take one of the following forms:

1.	m to n

2.	n downto m

3.	a’range

4.	a’reverse_range

Where, m < n are integer constants, and “a” is a vector object.

### Generate Statement Example

entity GEN1 is
 	port(A, B: in  STD_LOGIC_VECTOR(3 downto 0);
		CIN:   in  STD_LOGIC;
 		COUT:  out STD_LOGIC;
 		S:     out STD_LOGIC_VECTOR(3 downto 0) );
end GEN1;
--
architecture SYN of GEN1 is
 	component FADD1
 	port(CI, A, B: in  STD_LOGIC;
		 S, CO:    out STD_LOGIC );
	 end component;
 	signal C: STD_LOGIC_VECTOR(0 to 4);
begin
 	C(0) <= CIN;
	 COUT <= C(4);
	 l2: for I in 0 to 3 generate
		 l1: FADD1 port map( C(I), A(I), B(I), S(I), C(I + 1) );
	 end generate;
end SYN;

Packages

What Is a Package?

A package is a collection of subprograms, constants, types, and components that can be imported by behavioral descriptions and other packages.

Packages must be analyzed before being referenced by a use clause. There are two ways to insure that the package is analyzed:

- include a package in front of the entity/architecture pair in the VHDL description.

- within ViewSynthesis for Windows, run the VHDL command on the package before running VHDL on the design.

Each package declaration declares the subprograms, constants, types, and components that it exports. These may be imported for use in other contexts via use clauses.

Parts of a Package

Each package consists of a package declaration and a package body.

- The package declaration contains declarations of sub- programs, components, constants, and types that are exported.

- The package body contains the bodies of exported subprograms. The package body may also define internal subprograms, constants, and types which are not exported.

Any package containing a declaration imported by a use clause must have the same name as the base name of the file that contains it.

### Package Declaration Format

package package_name is 
	[exported_subprogram_declarations] 
	[exported_constant_declarations]
	[exported_type_declarations]
	[component_declarations ]
	[attribute_declarations ] 
	[attribute_specifications ]
end [package_name];

### Package Declaration Example

package ARITHMET is
	-- declarations of exported ”max” and ”min” functions:
	function MINIMUM (constant A, B: in INTEGER) 
											return INTEGER;
	function MAXIMUM (constant A, B: in INTEGER)
											return INTEGER;
	-- declarations of exported constant and type:
	constant MAXINT: INTEGER := 16#FFFF#;
	type ARITHMETIC_MODE_TYPE is (SIGNED, UNSIGNED);
end;

Package Body

Each package declaration is followed by its package body, if any. The package body contains the bodies of subprograms declared in the package declaration, and the declarations and/or bodies of any internal subprograms, constants, and types used by the package.

Subprograms, constants, and types declared within package bodies are visible only within their immediate scope.

### Package Body Format

package body package_name is
	[exported_subprogram_bodies]
	[internal_subprogram_declarations]
	[internal_subprogram_bodies]
	[internal_constant_declarations]
	[internal_type_declarations ]
end [package_name];

### Package Body Example

package body ARITHMET is
	-- body of exported ”min” function:
	function MINIMUM (constant A, B: in INTEGER) 
									return INTEGER is
	begin
		if A < B then
			return A;
		else
			return B;
		end if;
	end MINIMUM;
	-- body of exported ”max” function:
	function MAXIMUM (constant A, B: in INTEGER)
									return INTEGER is
	begin
		if A > B then
			return A;
		else 
			return B;
		end if;
	end MAXIMUM;
end ARITHMET;

Subprograms

Overview

Subprogram declarations are required in package declarations to declare externally visible subprograms. A subprogram’s body must appear before it is referenced.

Subprogram Declaration

A subprogram is either a procedure or a function. A procedure may modify its arguments, but returns no value. A function may not modify its arguments; it returns one value. A subprogram’s specification declares its name, indicates whether it is a procedure or a function, and declares the parameters it accepts.

### Procedure Declarations

Procedures accept parameters of class constant, variable, and signal, and modes in, out, inout. If no mode is specified, in is assumed. If the mode is out or inout, the class must be variable or signal. If an object class is not specified, constant or variable is assumed, according to the mode.

If a parameter type is an unconstrained array type, an index range may be provided to define each dimension of the array, or the size of the parameter may be defined by the value or object passed to the procedure. If the parameter type is not an unconstrained array type, no index range may appear.

### Procedure Declaration Format

procedure procedure_name 
	[([object_class] arg_name {, arg_name}: 
		[mode] type [(index_range[, index_range])] ;
		{[object_class] arg_name {, arg_name}:
		[mode] type [(index_range [,index_range])]})];

### Procedure Declaration Example

procedure LATCH (signal CLK, DATA:   in  STD_LOGIC;
				   signal LATCHED_DATA: out STD_LOGIC);

### Function Declarations

Functions accept only parameters of class constant or signal and mode in; they may not modify their parameters. If a parameter’s class or mode is unspecified, constant or in is assumed.

If a parameter type is an unconstrained array type, an index range may be provided to define each dimension of the array, or the size of the parameter may be defined by the value or object passed to the procedure. If the parameter type is not an unconstrained array type, no index range may appear.

### Function Declaration Format

function function_name
	[([object_class] arg_name {,arg_name}: 
		[ in ] type [(index_range [, index_range])]
		{; [object_class] arg_name {, arg_name}:
			[in] type [(index_range [, index_range])]})] 
return type;

### Function Declaration Example

function MINIMUM (constant A, B: in INTEGER) return INTEGER;

Subprogram Body

A subprogram body specifies the algorithm that the subprogram implements.

A subprogram body consists of a subprogram specification, which may repeat a previous declaration of the subprogram, a declarative part, and a sequence of statements. The declarative part declares constants, variables, and types used within the subprogram. The subprogram’s statement sequence includes sequential statements.

Subprograms may contain the sequential statements described in Chapter 6, except that functions (and procedures called directly or indirectly by functions) may not contain wait statements or signal assignment statements. These statements are executed sequentially, in their order of appearance.

Functions must terminate with a return statement that returns a value of the type of the function.

Objects declared within subprograms are invisible outside their subprograms. Variables declared within subprograms do not retain their values between invocations.

### Procedure Body Format

procedure procedure_name
	[([object_class] arg_name {, arg_name}:
		[mode] type [(index_range[, index_range])]
		{; [object_class] arg_name {, arg_name}:
			 [mode] type [(index_range [, index_range])]})] 
is
	[constant_declarations] 
	[variable_declarations] 
	[type_declarations]
begin
	sequence_of_statements
end [procedure_name];

### Procedure Body Example

procedure MY_DFF (signal CLK, DATA: in  STD_LOGIC;
               signal Q:         out STD_LOGIC) is
begin
	loop
		-- update ”LATCHED_DATA” whenever ”CLK” rises:
		wait until (CLK’EVENT and CLK = ’1’);
		Q <= DATA;
	end loop;
end MY_DFF;

### Function Body Format

function function_name
	[([object_class] arg_name {,arg_name}: 
		[in] type [(index_range [, index_range])] 
		{; [object_class] rg_name {, arg_name}:
			 [in] type [(index_range [, index_range])]})]
return type is
	[constant_declarations] 
	[variable_declarations ] 
	[type_declarations ]
begin
	sequence_of_statements 
	return (expression);
end [function_name];

### Function Body Example

function MINIMUM (constant A, B: in INTEGER) 
		return INTEGER is
	variable RESULT ; INTEGER;
	begin
		-- return lesser of ”A”, ”B”
		if A < B then
			RESULT := A;
		else
			RESULT := B;
		end if;
		return (RESULT);
	end MINIMUM;

Context Clause

Purpose

Context clauses consist of one or more of the following items:

- library clauses, which are used to specify libraries that are referenced in the entity

- use clauses, which are used to import subprograms, types, and constants that are referenced in the entity

Context clauses may appear before each package and entity declaration.

Library Clause

Every package and entity that references a library other than the working library must be preceded by a library clause that imports the referenced library.

### Library Clause Format

library library_name {, library_name } ;

### Library Clause Example

library PARTS, UTILS;

Use Clause

Use clauses specify subprograms, types, constants, and components to be imported from packages.

Each use clause specifies one or more imported items. All use clauses appear together immediately before the package or entity to which they apply. Any declaration made visible by a use clause is directly visible throughout the following package or entity, except where it is hidden by locally declared items of the same name.

All subprograms must be visible before they may be referenced. Subprograms are automatically visible only within the package body where they are defined. This means that within a package body, any reference to a subprogram declared in another package must be preceded by an appropriate use clause, and within an architecture body, any reference to any subprogram must be preceded by an appropriate use clause.

The use clause specifies the library, package, and name of each imported item. Any package that contains a declaration that is imported by a use clause must have the same name as the base name of the file that contains it.

The suffix ALL can be used to import every item declared within a package in an external library.

After the appearance of a use clause, imported items may be referenced by their simple names. It is not necessary to precede each reference with the library and package name.

### Use Clause Format

use external_name {, external_name } ;

Each external name takes the form:

library_name.package_name.declared_name

### Use Clause Example

library PARTS, UTILS;
use PARTS.LATCHES.LATCH1B, UTILS.ARITHMET.all;