# Objects

About Objects

Legal Object Names

Object names may include letters, underscores, and digits.

Names must begin with a letter and may not end with an underscore, and two underscores may not be adjacent in a name.

VHDL is not case-sensitive.

Where to Declare Objects

Objects may be declared in various places (i.e., architecture, ports, process, subprogram) within VHDL constructs. loop statement parameters are implicitly declared as part of a statement; all other objects must be declared explicitly before they are used.

Object Type and Class

Objects are the containers of values within a model. Each object has a type and a class.

Types are the VHDL pre-defined data types and user-defined types. An object’s type indicates what kind of data it may hold. An object’s class indicates what can be done with its data; classes are:

- constant

- variable

- signal

Indexing Objects

Object indices and index ranges must be integer values as shown in the following examples, where “obj” represents a constant, a variable, or a signal.

obj (integer value)
obj (integer value to integer value)
obj (integer value downto integer value)

The use of variables or signals to index an object is not supported.

Constants

Constants have a single or composite value, which is defined when the constant is declared, and can never change. Constants may be of any supported type.

Variables

Variables have a single or composite value, which changes whenever an assignment is made to the variable. Variables may be of any supported type. Variables assigned and used in the same state of a clocked process will be just a wire. Variables assigned in one state and used in another will be registers. Variables in combinational processes must be initialized before use and will represent a wire.

Variables are defined in both process and subprogram statements. A variable may represent combinational logic or storage elements in the case when a variable is read in a different state from that in which it was set.

- Combinational circuit output is defined by the function of its input.

- Latch output remains at its last value when the enable line is off or FALSE.

- Register output is controlled by a clock signal.

Signals

Signals represent wires whose values are defined by the expression in the signal assignment. Signals can be of any supported type as listed in Chapter 3.

Signals and variables can represent the I/O of combinational logic, latches and registers.

Signals represent combinational logic or latches when assigned in concurrent and not-clocked process statements, and flip-flops when assigned within clocked process statements.

See the “Process Statement” section in Chapter 2 for information on the storage of signals and variables.

Signal Reassignment

Signals must have at most one active driver at a time.

Reassignment of a signal in a following sequential signal assignment is allowed. This signal will take the value of the last signal assignment statement.

Constant Declaration

About Constants

One or more constants may be declared by a constant declaration. The value expression defines the value of the constant(s). This expression’s value must be known at analysis, and its type must be the same as the declared constant type.

If a constant type name denotes an unconstrained array type, an index range must appear to define the size of each dimension; otherwise, no index range may appear. Index ranges are:

integer_value to integer_value

or

integer_value downto integer_value

### Constant Declaration Format

constant name {, name}:
  type [(index_range [, index_range ])]
  := initial_value_expression;

### Constant Declaration Examples

-- (1) INTEGER constant, value 0 --
constant ZERO:    INTEGER := 0;

-- (2) STD_LOGIC constant, value ”0000000” --
constant ZERO_7:  STD_LOGIC_VECTOR (6 downto 0) := ”0000000” ;

-- (3) PLA_TABLE constant declaration --
constant TBL  :  STD_LOGIC_2D (0 to 1, 10 downto 0) := (

--		BCD(3:0)_SEG(6:0)
--		-------------------
		”0000_0111111”,
		”0001_0011000” );
begin
	PLA_TABLE ( BCD, SEG, TBL );

### Referencing Constants

Constants may be referenced by their simple names. In addition, constant arrays may be referenced with the construct:

name(index_element)

where each index_element indexes one dimension of the array, or

name(range)

where the range specifies a slice of an array. The following example specifies a 4-bit slice of the array declared in Example (2) above:

ZERO_7 (3 downto 0);

Variable Declaration

About Variables

Variables assigned and used in the same state of a clocked process will be just a wire. Variables assigned in one state and used in another will be registers. Variables in combinational processes must be initialized before use and will represent a wire.

One or more variables may be declared by a variable declaration.

The initial value of a variable declared in a process is ignored in synthesis. Variable initialization is supported in functions and procedures (subprograms).

Non-registered variables in a combinational process must be explicitly initialized before use by assignment.

The value “10” in the example below is ignored and subsequently results in an error as shown.

### Example: Unsupported Variable Initialization

process (S1,S2)
	variable V1 : STD_LOGIC_VECTOR(0 to 3) :=”0010”; -- value ignored
	begin
		S1 <= V1; -- results in an error
		.
		.
end process;

### Example: Supported Variable Initialization

process (S1,S2)
	variable V1 : STD_LOGIC_VECTOR (0 to 3);
	begin
		V1 :=”0010”;
		S1 <= V1; 
		.
		.
end process;

### Example: Variable Initialization in a Subprogram

function F1 (A, B : STD_LOGIC) return STD_LOGIC is
	variable RESULT : STD_LOGIC := ’0’;
	begin
		if (A and B) then
		  RESULT := ’1’;
		end if;
		return (RESULT);
end F1;

### Initial Values in Subprograms

If no initial value is specified in subprograms, the initial values are set to:

- FALSE for BOOLEAN elements

- ‘0’ for INTEGER elements

- ‘0’ for STD_LOGIC elements

- The default initial value for elements of user-defined enumeration types is the least (left-most) enumeration value of the type. (These values correspond to the IEEE 1076-1993 type attribute T’LEFT.)

### Variable Declaration Format

variable name{, name}: type [(index_range)] [:= initial_value_expression];

### Referencing Variables

Variables may be referenced or assigned by their simple names. In addition, variable arrays may be referenced or assigned with the construct:

name(index_element)

where index_element indexes one dimension of the array, or

name(range)

where the range specifies a slice of the array.

If a variable type name denotes an unconstrained array type, an index range must appear to define the size of each dimension; otherwise, no index range may appear.

Index ranges are:

integer_value to integer_value

or

integer_value downto integer_value

### Variable Declaration Example

variable ADDRESS:   INTEGER;  -- default initial value
variable DATA: STD_LOGIC_VECTOR (0 to 16#FFFF#)
				-- 1-dimensional STD_LOGIC array.
	ADDRESS := 0;   	-- initialize address
	for I in 0 to 16#FFFF# loop
		DATA(I) := ’0’;
	end loop;

Signal Declaration

About Signals

Signals in combinational processes can be wires or latches. Latches represent incompletely specified signals. Completely specified signals are implemented as wires. Signals in clocked processes are always latches.

One or more signals may be declared by a signal declaration.

Signals can be of any supported type, as listed in Chapter 3.

Signal initialization within the declaration is ignored by synthesis.

### Signal Declaration Format

signal name{, name}: type [(index_range)]

### Referencing Signals

Signals may be referenced or assigned by their simple names. In addition, signal arrays may be referenced or assigned with the construct:

name(index_element)

where the index_element indexes one dimension of the array, or

name(range)

where the range specifies a slice of the array.

If a signal type name denotes an unconstrained array type, an index range must appear to define the size of each dimension; otherwise, no index range may appear.

Signals declared with index ranges are arrays of the given dimensions.

Index ranges are:

integer_value to integer_value

or

integer_value downto integer_value

### Signal Declaration Example

signal CLOCK, RESET: STD_LOGIC; -- single bit signals
signal Q1, COUNT:    STD_LOGIC_VECTOR (15 downto 0); 
		-- 1-dimensional STD_LOGIC array; 16 bits

process
	begin

wait until CLOCK = ’1’;
	if RESET = ’1’ then
		Q1 <= ”0000000000000000”;
	 	else
		Q1 <= COUNT;
	end if;

end process;

Component Declaration

About Component Declarations

A component declaration defines the interface to a VHDL model (entity and architecture) possibly defined in another file. This is the definition that is visible at analysis time.

In general, a component declaration and its corresponding entity declaration will be identical. They may only differ in the default values assigned to the generic and signal values. The default values are used whenever a component instantiation leaves a port unconnected via the OPEN keyword.

Component declarations provide a local definition for VHDL models. These models may be behavioral and/or structural. Component declarations may reference schematic symbols.

Component declaration statements are made in architectures and in package headers. The entities referenced by component declarations do not need to exist at analysis time. Also, only those components which are actually instantiated at some point in the design need to exist at elaboration time. This allows a designer to create declarations for a library of parts, for example, and then define the behavior of parts as they are needed.

### Component Declaration Format

component name
	[ port ( port_list ) ; ]
end component ;

### Component Declaration Example

component DETECTOR
	port ( RST, CK, CTL, DIN: in BIT;
			MATCH: out BIT);
end component ;

Attributes

Synthesis and Attributes

Attributes (properties) describe characteristics about associated entities. There are two kinds of attributes--pre-defined and user-defined.

User-defined attributes are not supported. Pre-defined attributes are supported within the scope of an architecture. This section provides a list of the pre-defined attributes supported by ViewSynthesis for Windows.

Assuming ‘A’ is an array type object, the following tables describe the meaning of the supported attributes.

Supported Predefined Attributes

| A’LEFT |  |
| --- | --- |
| Kind: | Function |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | Type of the left bound of the index range of A. |
| Result: | Left bound of the index range of A. |

| A’RIGHT |  |
| --- | --- |
| Kind: | Function |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | Type of the right bound of the index range of A. |
| Result: | Right bound of the index range of A. |

| A’HIGH |  |
| --- | --- |
| Kind: | Function |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | Type of the upper bound of the index range of A. |
| Result: | Upper bound of the index range of A |

| A’LOW |  |
| --- | --- |
| Kind: | Function |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | Type of the lower bound of the index range of A. |
| Result: | Lower bound of the index range of A. |

| A’RANGE |  |
| --- | --- |
| Kind: | Range |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | Type of the index range of A. |
| Result: | The range A’LEFT to A’RIGHT if index range of ‘A is ascending, or range A’LEFT downto A’RIGHT if index range of A is descending. |

| A’REVERSE_RANGE |  |
| --- | --- |
| Kind: | Range |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | Type of the index range of A. |
| Result: | The range A’RIGHT downto A’LEFT if the index range of ‘A is ascending, or range A’RIGHT to A’LEFT if index range of A is descending. |

| A’LENGTH |  |
| --- | --- |
| Kind: | Value |
| Prefix: | Any prefix A that is appropriate for an array object, or that denotes a constrained array type. |
| Result Type: | universal_integer |
| Result: | Number of values in the index range; i.e., A’HIGH_A’LOW+1 |