# Controlling Timing

Timing for Synthesis

Introduction

ViewSynthesis for Windows produces an optimized design by combining the circuit behavior declared in the VHDL source files with user-imposed design constraints. User-imposed design constraints and requirements include a broad range of design characteristics. This chapter describes how and where you specify timing constraints and requirements.

What You Can Control

You can impose timing constraints and requirements on certain signals you have declared in your VHDL source file. Using various ViewSynthesis for Windows command-line switches or interactive commands on the timing parameters you can control

- Global timing

- Port timing

The balance of this chapter describes how to declare certain timing conditions for your synthesized design.

FPGAs vs. ASICs

The discussion in this chapter is mostly applicable exclusively to Gate Array and Standard Cell technologies. For FPGAs, it is best to use the Graphic User Interface to choose optimization for area or speed and not use the material in this chapter at all.

ViewSynthesis for Windows and Path Analysis

Introduction

This section describes how ViewSynthesis for Windows uses path analysis to optimize your design.

Path Analysis

As part of the synthesis process, ViewSynthesis for Windows performs a path analysis on the design. During path analysis, ViewSynthesis for Windows combines all user imposed timing constraints with each cell’s delay/timing information. Using this information, ViewSynthesis for Windows calculates the total time necessary for a signal to propagate through each path within a design.

What Is a Path

ViewSynthesis for Windows recognizes a path using the following criteria:

1. A path begins at a primary input, or register output.

2. The path passes through various elements which implement the circuit’s combinational logic. Timing within these elements is determined by the performance characteristics of the cells used in the design.

3. The path terminates at a register input, or primary output.

Four Types of Paths

ViewSynthesis for Windows recognizes four types of paths:

- Primary Input to Primary Output--if no register is encountered in the path

- Primary Input to Register Input

- Register Output to Primary Output

- Register Output to Register Input

Path From Primary Input to Primary Output

The following figure illustrates a path from a primary input to a primary output. There can be no registers between the input and output ports.

Path From Primary Input to Register Input

The following figure illustrates a path from a primary input to a register input.

Path From Register Output to Primary Output

The following figure illustrates a path from a register output to a primary output.

Path From Register Output to Register Input

The following figure illustrates a path from a register output to a register input.

The Results of Path Analysis

ViewSynthesis for Windows uses the results of path analysis to determine where the area/delay trade-offs should be made by identifying which combination of cells produce the highest degree of optimization for the design.

Controlling Port Timing

Introduction

This section describes how you specify port timing requirements using ViewSynthesis for Windows. You can specify port timing for data signals attached to primary input and primary output ports.

The Port Timing Parameters

You use ViewSynthesis for Windows port timing parameters to impose timing requirements on data signals. There are separate port timing parameters for input ports and output ports. The following table lists and describes the six ViewSynthesis for Windows port timing parameters.

ARRIVAL_TIME	You use the ARRIVAL_TIME parameter to specify timing at primary input ports. The arrival time is the latest time a signal will arrive at a primary input port.

INPUT_DRIVE	Is the additional delay per unit load for an input port. Each technology has a default drive defined for inputs, usually the drive of a single inverter gate.

MAX_LOAD	Is the maximum allowable load for a specific input port. You specify a value and a port. You specify the value in terms of unit load.

OUTPUT_LOAD	Is the load on an output port. It is specified in terms of unit loads.

REQUIRED_TIME	Is the latest time a signal can become stable at an output port. You assign the <R>REQUIRED_TIME parameter to signals attached to primary output ports or register inputs.

Example

The following example shows how you would declare the arrival time for five input ports in a timing file.

ARRIVAL_TIME 5  DATA0[3:0]

ARRIVAL_TIME 5  DATA3[3:0]

ARRIVAL_TIME 12 SEL[0:1]

In this example the busses DATA0 and DATA3 must arrive at their primary input ports no later than 5 nanoseconds after the beginning of the clock cycle. The signal SEL must arrive at its input port no later than 12 nanoseconds after the beginning of the clock cycle.

The Clock Cycle

Using the port timing parameters shown above, the following figure illustrates when the signals must arrive at their respective input ports relative to the clock cycle.

Timing External to the Synthesized Design

Timing in the synthesized portions of your design are dependent upon the non-synthesized logic and the timing of your overall design. Signals which arrive at a primary input port normally were generated by non-synthesized logic. Also, signals which propagate through the synthesized design to a primary output port must exit the synthesized design at a time that is consistent with the timing of the overall design. Having consistent timing in the synthesized and non-synthesized portion of your design is critical to the timing of the overall design.

Calculating Paths with Registers

Introduction

This section describes how ViewSynthesis for Windows calculates the maximum path length of a circuit that contains one or more registers.

Influencing Register Timing

You cannot directly specify required and arrival time at register inputs and outputs as you would at primary inputs and outputs. The required and arrival time for register inputs and outputs are deduced from the:

- Clock waveform specified by the clock timing parameters

- Register setup and hold constraints and propagation delays contained in the .lib file

- Required and arrival times at the primary input and output ports

Timing for Flip-flops and Latches

ViewSynthesis for Windows deduces the required time of a register’s input by including the register’s setup time with any combinational path delay. The various technology vendors determine the setup time for each register in their libraries.

Timing for Flip-Flops

ViewSynthesis for Windows deduces the timing for flip-flops relative to the active edge (typically the leading edge) of the clock signal. The leading edge of the clock occurs when new data is latched into the flip-flop. For data to be properly latched, the data must be stable at the input pin one setup time before the leading edge.

The following figure illustrates how ViewSynthesis for Windows deduces the timing for flip-flops relative to the clock cycle.

Timing for Latches

ViewSynthesis for Windows deduces the timing for latches relative to the latching edge of the clock signal (typically the falling edge). The active edge of a latch occurs when the latch goes from being enabled (data passes through) to being disabled. When latches become disabled, changes on D do not affect Q. For latches, data must be stable one setup time prior to the trailing edge of the clock.

The following figure illustrates how ViewSynthesis for Windows deduces timing for latches relative to the beginning of the clock cycle.