# Run-Time Library

Overview

Using This Appendix

This appendix is an alphabetical usage guide to functions and procedures applicable to ViewSynthesis for Windows synthesis. Table B-1 lists these functions and procedures by type/usage.

| Table B-1. Functions and Procedures |  |  |  |  |
| --- | --- | --- | --- | --- |
| Conversion | Arithmetic | Overload | Edge Detection | Procedure: Memory |
| boo2std | addum | “+” | pfalling | dff |
| std2boo | add2c | “-” | prising | dffc |
| int2vld | bitunknown | -------------- | -------------- | dffp |
| vld2int | comp2c | -------------- | -------------- | dffpc |
| -------------- | extendum | -------------- | -------------- | dff_v |
| -------------- | extend2c | -------------- | -------------- | dffc_v |
| -------------- | shiftlum | -------------- | -------------- | dffp_v |
| -------------- | shiftl2c | -------------- | -------------- | dffpc_v |
| -------------- | shiftrum | -------------- | -------------- | dlatch |
| -------------- | shiftr2c | -------------- | -------------- | dlatchc |
| -------------- | subum | -------------- | -------------- | dlatchp |
| -------------- | sub2c | -------------- | -------------- | dlatchpc |
| -------------- | -------------- | -------------- | -------------- | dlatch_v |
| -------------- | -------------- | -------------- | -------------- | dlatchc_v |
| -------------- | -------------- | -------------- | -------------- | dlatchcpc_v |
| -------------- | -------------- | -------------- | -------------- | srlatch |
| -------------- | -------------- | -------------- | -------------- | jkff |
| -------------- | -------------- | -------------- | -------------- | fkffpc |
| -------------- | -------------- | -------------- | -------------- | pla_table |

addum Function

Format

addum (V1, V2: STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

addum (V1, V2: STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR that is the sum of the two given vectors, interpreted as unsigned magnitude integers; the shorter input is extended to the other’s length and the result is one bit longer than the longer length. The left-most bit of the result contains the carry, if any.

Limitations

None

Note:

This function is provided primarily for compatibility with earlier Viewlogic synthesis products.

+ Function

Format

”+” (V1, V2: STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

”+” (V1, V2: STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR that is the sum of the two given vectors, interpreted as unsigned magnitude integers; the shorter input is extended to the other’s length and the result is one bit longer than the longer length. The left-most bit of the result contains the carry, if any.

entity ADDVEC is
	port(A,	B:	in  STD_LOGIC_VECTOR(2 downto 0);
			C:  	out STD_LOGIC_VECTOR(2 downto 0);
end ADDVEC;

architecture DATAFLOW of ADDVEC is
	begin
		C <= A + B;
end DATAFLOW;

entity ADD_UNSIGNED is
	port(A,	B:	in  STD_LOGIC_VECTOR(2 downto 0);
			C:  	out STD_LOGIC_VECTOR(3 downto 0);
end ADD_UNSIGNED;

architecture DATAFLOW of ADD_UNSIGNED is
	begin
		C <= (”0” & A) + (”0” & B);
end DATAFLOW;

add2c Function

Format

add2c (V1, V2: STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

add2c (V1, V2: STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the sum of the two given vectors, interpreted as two-complement integers; the shorter input is sign extended to the other’s length and the result is one bit longer than the longer length input. The left-most bit of the result contains the carry, if any.

Limitations

None

NOTE:

This function is provided primarily for compatibility with earlier Viewlogic synthesis products.

bitunknown Function

Format

bitunknown (V: STD_ULOGIC) return BOOLEAN;

Returns

BOOLEAN: TRUE if the given STD_ULOGIC value is X or Z; FALSE otherwise.

Limitations

None

comp2c Function

Format

comp2c (V:STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

comp2c (V:STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the arithmetical complement of the given vector, interpreted as a two-complement integer; the result is the same length as the input.

Limitations

None

extendum Function

Format

extendum (V: STD_ULOGIC_VECTOR; I: INTEGER) return STD_ULOGIC_VECTOR;

extendum (V: STD_LOGIC_VECTOR; I: INTEGER)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the input vector, extended to the given length. Additional bits of the result are zero-filled.

Limitations

None

extend2c Function

Format

extend2c (V:STD_ULOGIC_VECTOR; I: INTEGER) return STD_ULOGIC_VECTOR;

extend2c (V:STD_LOGIC_VECTOR; I: INTEGER)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the input vector, extended to the given length. The sign bit is left-shifted (with sign fill) to fill additional bits.

Limitations

None

int2v1d Function

Format

int2V1d (I: INTEGER) return STD_ULOGIC_VECTOR(0 to 31);

int2V1d (I: INTEGER) return STD_LOGIC_VECTOR(0 to 31);

Returns

A 32-bit STD_LOGIC_VECTOR equivalent of the given value.

Limitations

None

pfalling Function

Format

pfalling (signal S: STD_ULOGIC) return BOOLEAN;

Returns

A BOOLEAN result indicating whether the given one-bit signal changed to 0 during the last cycle.

Limitations

None

NOTE:

This function is provided primarily for compatibility with earlier Viewlogic synthesis products. Use instead:

CLOCK’EVENT and CLK = ’0’

pla_table Procedure

Format

pla_table (I				:	in		STD_ULOGIC_VECTOR;
					signal O	:	out	STD_ULOGIC_VECTOR;
					TBL			:	in		STD_ULOGIC_2d);

pla_table (I				:	in		STD_LOGIC_VECTOR;
					signal O	:	out	STD_ULOGIC_VECTOR;
					TBL			:	in		STD_LOGIC_2d);

Returns

- No return.

- The value of the pla table output corresponding to the signal input is assigned to the explicit signal “output.”

Limitations

None

Example

library IEEE;
use IEEE.STD_LOGIC_1164.all;
library SYNTH;
use SYNTH.PACK1164.all;

entity FSM_EPLA is  --  Entity declaration, declare ports
	port(	RESET, CLK, DATA_IN:	in  STD_LOGIC;
			PMATCH:					out STD_LOGIC);
end FSM_EPLA;

architecture FIRST of FSM_EPLA is -- Behavioral description
signal	S, NXT_S:	STD_LOGIC_VECTOR(2 downto 0);
signal	TEMP:			STD_LOGIC_VECTOR(3 downto 0);
constant	TBL:			STD_LOGIC_2D(0 to 11, 6 downto 0) :=(
--S  |D| NXT_S
--210| | 210   (bit numbers)
-----+-+-------
B”000_1_001”,	-- S0 & 1 => S1
B”000_0_000”,	-- S0 & 0 => S0
B”001_1_010”,	-- S1 & 1 => S2
B”001_0_000”,	-- S1 & 0 => S0
B”010_1_011”,	-- S2 & 1 => S3
B”010_0_000”,	-- S2 & 0 => S0
B”011_1_011”,	-- S3 & 1 => S3
B”011_0_1X0”,	-- S3 & 0 => S4
B”1X0_1_001”,	-- S4 & 1 => S1
B”1X0_0_1X1”,	-- S4 & 0 => S5
B”1X1_1_001”,	-- S5 & 1 => S1
B”1X1_0_000”);	-- S5 & 0 => S0

begin
	PMATCH <= S(0) and S(2); -- (S = ”1X1”);
	TMP <= S & DATA_IN;
	pla_tbl (TMP, NXT_S, TBL);

-- Separate clocked process
	CNTL: process
	begin
		wait until (CLOCK’EVENT and CLK = ’1’) or (RESET = ’1’);
		if (RESET = ’1’) then 
			S<= ”000”; -- reset S
		else 
			S <= NXT_S; -- S goes to next state
		end if;
	end process CNTL;
end FIRST;

prising Function

Format

prising (signal s: STD_ULOGIC) return BOOLEAN

Returns

A BOOLEAN result indicating whether the given one-bit signal changed to 1 during the last cycle.

Limitations

None

NOTE:

This function is provided primarily for compatibility with earlier Viewlogic synthesis products. Use instead:

CLOCK’EVENT and CLK = ’1’

shiftlum Function

Format

shiftlum (V: STD_ULOGIC_VECTOR; I: INTEGER) return STD_ULOGIC_VECTOR;

shiftlum (V: STD_LOGIC_VECTOR; I: INTEGER)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the input vector, shifted left by the given number of bits and zero-filled.

Limitations

None

shiftl2c Function

Format

shiftl2c (V: STD_ULOGIC_VECTOR; I: INTEGER) return STD_ULOGIC_VECTOR;

shiftl2c (V: STD_LOGIC_VECTOR; I: INTEGER)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the input vector, shifted left by the given number of bits and zero-filled.

Limitations

None

shiftrum Function

Format

shiftrum (V: STD_ULOGIC_VECTOR; I: INTEGER) return STD_ULOGIC_VECTOR;

shiftrum (V: STD_LOGIC_VECTOR; I: INTEGER)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the input vector, shifted right by the given number of bits and zero-filled.

Limitations

None

shiftr2c Function

Format

shiftr2c (V: STD_ULOGIC_VECTOR; I: INTEGER) return STD_ULOGIC_VECTOR;

shiftr2c (V: STD_LOGIC_VECTOR; I: INTEGER)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the input vector, interpreted as a two-complement integer, shifted right by the given number of bits and sign-filled.

Limitations

None

subum Function

Format

subum (V1, V2: STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

subum (V1, V2: STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the difference of the two given vectors, interpreted as unsigned-magnitude integers; the second argument is subtracted from the first. The shorter input is extended to the other’s length and the result is one bit longer than the longer length. The left-most bit of the result contains the carry, if any. The result is undefined if the first argument is less than the second.

Limitations

None

sub2c Function

Format

sub2c (V1, V2: STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

sub2c (V1, V2: STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the difference of the two given vectors, interpreted as two-complement integers; the second argument is subtracted from the first. The shorter input is sign-extended to the other’s length and the result is one bit longer than the longer length. The left-most bit of the result contains the carry, if any.

Limitations

None

- Function

Format

”-” (V1, V2: STD_ULOGIC_VECTOR) return STD_ULOGIC_VECTOR;

”-” (V1, V2: STD_LOGIC_VECTOR)  return STD_LOGIC_VECTOR;

Returns

A STD_ULOGIC_VECTOR or STD_LOGIC_VECTOR result that is the difference of the two given vectors, interpreted as two-complement integers; the second argument is subtracted from the first. The shorter input is sign-extended to the other’s length and the result is one bit longer than the longer length. The left-most bit of the result contains the carry, if any.

No carry_out bit is returned. The length of the returned vector is the same as the longest operand.

std2boo Function

Format

std2boo (V: STD_ULOGIC)

Returns

boolean

Limitations

None

vecunknown Function

Format

vecunknown (V: STD_ULOGIC_VECTOR) return BOOLEAN;

vecunknown (V: STD_LOGIC_VECTOR)  return BOOLEAN;

Returns

BOOLEAN: TRUE if any element of the given value is X or Z; FALSE otherwise.

Limitations

None

v1d2int Function

Format

V1d2int (V: STD_ULOGIC_VECTOR) return INTEGER;

V1d2int (V: STD_LOGIC_VECTOR)  return INTEGER;

Returns

The INTEGER equivalent of the given value.

Limitations

None

boo2std Function

Format

boo2std (B: BOOLEAN)

Returns

STD_ULOGIC

Limitations

None

dff Procedure

Format

dff (	constant	DATA	:	in			STD_ULOGIC;
			signal		CLK	:	in			STD_ULOGIC;  -- + edge triggered
			signal		Q		:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dffc Procedure

Format

dffc (	constant	DATA		:	in			STD_ULOGIC;
			signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			signal		CLK		:	in			STD_ULOGIC;  -- + edge triggered
			signal		Q			:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

- The signal “CLEAR” is active high, and performs an asynchronous reset of the flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dffp Procedure

Format

dffp (	constant	DATA		:	in			STD_ULOGIC;
			signal		PRESET	:	in			STD_ULOGIC;  -- active high
			signal		CLK		:	in			STD_ULOGIC;  -- + edge triggered
			signal		Q			:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

- The signal “PRESET” is active high, and performs an asynchronous set of the flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dffpc Procedure

Format

dffpc  (constant	DATA		:	in			STD_ULOGIC;
			 signal		PRESET	:	in			STD_ULOGIC;  -- active high
			 signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			 signal		CLK		:	in			STD_ULOGIC;  -- + edge triggered
			 signal		Q			:	inout	STD_ULOGIC);

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

- The signal “PRESET” is active high, and performs an asynchronous set of the flip-flop. The signal “CLEAR” is active high, and performs an asynchronous reset of the flip-flop.

- If both “PRESET” and “CLEAR” are asserted, the output is not defined.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dff_v Procedure

Format

dff_v  (constant	DATA	:	in			STD_ULOGIC_VECTOR;
			 signal		CLK	:	in			STD_ULOGIC;  -- + edge triggered
			 signal		Q		:	inout	STD_ULOGIC_VECTOR);

dff_v  (constant	DATA	:	in			STD_LOGIC_VECTOR;
			 signal		CLK	:	in			STD_ULOGIC;  -- + edge triggered
			 signal		Q		:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dffc_v Procedure

Format

dffc_v (constant	DATA		:	in			STD_ULOGIC_VECTOR;
			  signal	CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
			  signal		Q		:	inout	STD_ULOGIC_VECTOR);

dffc_v (constant	DATA		:	in			STD_LOGIC_VECTOR;
			  signal	CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
			  signal	Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

- The signal “CLEAR” is active high, and performs an asynchronous reset of the flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dffp_v Procedure

Format

dffp_v (constant	DATA		:	in			STD_ULOGIC_VECTOR;
			  signal	PRESET	:	in			STD_ULOGIC;  -- active high
			  signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
			  signal	Q			:	inout	STD_ULOGIC_VECTOR);

dffp_v (constant	DATA		:	in			STD_LOGIC_VECTOR;
			  signal	PRESET	:	in			STD_ULOGIC;  -- active high
			  signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
			  signal	Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

- The signal “PRESET” is active high, and performs an asynchronous set of the flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dffpc_v Procedure

Format

dffpc_v (constant	DATA		:	in			STD_ULOGIC_VECTOR;
			  signal		PRESET	:	in			STD_ULOGIC;  -- active high
			  signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal		CLK		:	in			STD_ULOGIC;  -- + edge triggered
			  signal		Q			:	inout	STD_ULOGIC_VECTOR);

dffpc_v (constant	DATA		:	in			STD_LOGIC_VECTOR;
			  signal		PRESET	:	in			STD_ULOGIC;  -- active high
			  signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal		CLK		:	in			STD_ULOGIC;  -- + edge triggered
			  signal		Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the flip-flop.

- The signal “PRESET” is active high, and performs an asynchronous set of the flip-flop. The signal “CLEAR” is active high, and performs an asynchronous reset of the flip-flop.

- If both “PRESET” and “CLEAR” are asserted, the output is not defined.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” may be any expression, all other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dlatch Procedure

Format

dlatch (constant	DATA	:	in			STD_ULOGIC;
			  signal	GATE	:	in			STD_ULOGIC;  -- active high
			  signal	Q		:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dlatchc Procedure

Format

dlatchc (constant	DATA		:	in			STD_ULOGIC;
			  signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal		GATE		:	in			STD_ULOGIC;  -- active high
			  signal		Q			:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

- The signal “CLEAR” is active high, and performs an asynchronous reset of the latch.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dlatchp Procedure

Format

dlatchp	(constant	DATA		:	in			STD_ULOGIC;
			 signal		PRESET	:	in			STD_ULOGIC;  -- active high
			 signal		GATE		:	in			STD_ULOGIC;  -- active high
			 signal		Q			:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

- The signal “PRESET” is active high, and performs an asynchronous set of the latch.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dlatchpc Procedure

Format

dlatchpc	(constant	DATA		:	in			STD_ULOGIC;
			   signal		PRESET	:	in			STD_ULOGIC;  -- active high
			   signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			   signal		GATE		:	in			STD_ULOGIC;  -- active high
			   signal		Q			:	inout	STD_ULOGIC);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

- The signal “PRESET” is active high, and performs an asynchronous set of the latch. The signal “CLEAR” is active high, and performs and asynchronous reset of the latch.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

dlatch_v Procedure

Format

dlatch_v	(constant	DATA		:	in			STD_ULOGIC_VECTOR;
			   signal		GATE		:	in			STD_ULOGIC;  -- active high
			   signal		Q			:	inout	STD_ULOGIC_VECTOR);

dlatch_v	(constant	DATA		:	in			STD_LOGIC_VECTOR;
			   signal		GATE		:	in			STD_ULOGIC;  -- active high
			   signal		Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dlatchc_v Procedure

Format

dlatch_v	(constant	DATA		:	in			STD_ULOGIC_VECTOR;
			  signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal		GATE		:	in			STD_ULOGIC;  -- active high
			  signal		Q			:	inout	STD_ULOGIC_VECTOR);

dlatch_v	(constant	DATA		:	in			STD_LOGIC_VECTOR;
			  signal		CLEAR	:	in			STD_ULOGIC;  -- active high
			  signal		GATE		:	in			STD_ULOGIC;  -- active high
			  signal		Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

- The signal “CLEAR” is active high, and performs an asynchronous reset of the latch.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dlatchp_v Procedure

Format

dlatchp_v	(constant	DATA		:	in			STD_ULOGIC_VECTOR;
				 signal		PRESET	:	in			STD_ULOGIC;  -- active high
				 signal		GATE		:	in			STD_ULOGIC;  -- active high
				 signal		Q			:	inout	STD_ULOGIC_VECTOR);

dlatchp_v	(constant	DATA		:	in			STD_LOGIC_VECTOR;
				 signal		PRESET	:	in			STD_ULOGIC;  -- active high
				 signal		GATE		:	in			STD_ULOGIC;  -- active high
				 signal		Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

- The signal “PRESET” is active high, and performs an asynchronous set of the latch.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

dlatchpc_v Procedure

Format

dlatchpc_v	(constant	DATA		:	in			STD_ULOGIC_VECTOR;
				   signal		PRESET	:	in			STD_ULOGIC;  -- active high
				   signal		CLEAR	:	in			STD_ULOGIC;  -- active high
				   signal		GATE		:	in			STD_ULOGIC;  -- active high
				   signal		Q			:	inout	STD_ULOGIC_VECTOR);

dlatchpc_v	(constant	DATA		:	in			STD_LOGIC_VECTOR;
				   signal		PRESET	:	in			STD_ULOGIC;  -- active high
				   signal		CLEAR	:	in			STD_ULOGIC;  -- active high
				   signal		GATE		:	in			STD_ULOGIC;  -- active high
				   signal		Q			:	inout	STD_LOGIC_VECTOR);

Returns

- No return.

- The value of the d latch is available as the signal “Q”.

- The “GATE” input is enabled (transparent) high.

- The signal “PRESET” is active high, and performs an asynchronous set of the latch. The signal “CLEAR” is active high, and performs an asynchronous reset of the latch.

- If both “PRESET” and “CLEAR” are asserted, the output is not defined.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “DATA” and “GATE” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

- “Q” and “DATA” can be one-dimensional arrays of any length, but they must both be the same length.

jkff Procedure

Format

jkff  (signal	J		:	in			STD_ULOGIC;
		  signal	K		:	in			STD_ULOGIC;
		  signal	CLK	:	in			STD_ULOGIC;  -- + edge triggered
		  signal	Q		:	inout	STD_ULOGIC);

Returns

- No return.

- The value of the j-k flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the j-k flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “J” and “K” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

jkffc Procedure

Format

jkffc  (signal	J			:	in			STD_ULOGIC;
			 signal	K			:	in			STD_ULOGIC;
			 signal	CLEAR	:	in			STD_ULOGIC;  -- active high
			 signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
			 signal	Q			:	inout	STD_ULOGIC);

Returns

- No return.

- The value of the j-k flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the j-k flip-flop.

- The signal “CLEAR” is active high, and performs an asynchronous reset of the j-k flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “J” and “K” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

jkffp Procedure

Format

jkffp (signal	J			:	in			STD_ULOGIC;
		  signal	K			:	in			STD_ULOGIC;
		  signal	PRESET	:	in			STD_ULOGIC;  -- active high
		  signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
		  signal	Q			:	inout	STD_ULOGIC);

Returns

- No return.

- The value of the j-k flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the j-k flip-flop.

- The signal “PRESET” is active high, and performs an asynchronous set of the j-k flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “J” and “K” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.

jkffpc Procedure

Format

jkffp (signal	J			:	in			STD_ULOGIC;
		  signal	K			:	in			STD_ULOGIC;
		  signal	PRESET	:	in			STD_ULOGIC;  -- active high
		  signal	CLEAR	:	in			STD_ULOGIC;  -- active high
		  signal	CLK		:	in			STD_ULOGIC;  -- + edge triggered
		  signal	Q			:	inout	STD_ULOGIC);

Returns

- No return.

- The value of the j-k flip-flop is available as the signal “Q”.

- The rising edge of the “CLK” input triggers the j-k flip-flop.

- The signal “PRESET” is active high, and performs an asynchronous set of the j-k flip-flop. The signal “CLEAR” is active high, and performs an asynchronous reset of the j-k flip-flop.

Limitations

- Must be invoked in a concurrent signal assignment statement.

- “J” and “K” may be any expression. All other parameters must be input or internal signals.

- “Q” must be an inout or internal signal.