# Operator Inferencing and ALUCons 						Model Generators

Overview

ViewSynthesis for Windows includes integration with ALUCons models for automatic generation of optimal Datapath elements. This methodology produces a better results in significantly less time than the traditional method of generating equations.

During the compilation, ALUCons components are generated as black boxes for the following operators:

+, -, >, >=, <, <=, =

or one of the following ViewSynthesis predefined functions:

addum, add2c, subum, sub2c

ALUCons are not generated when one or both of the operants are constant (other than 1) as this can be better handled by equations. An expression (X + 1) or (X - 1) will be treated specially as an incrementer or decrementer respectively.

The following examples will generate ALUCons components (A, B, and Z are all signals):

Z <= A + B;
	Z <= A + B + 1;     -- An adder and an incrementer
	if (A > B) then ...
	if (A = B) then ...
	Z <= A + (”0” & B); -- We allow an explicit one bit zero extend

The following examples will not generate ALUCons components:

Z <= A + ”111”;
	Z <= A (”011” & B);
	if (A = ”111”) ...
	Z <= A + (”00” & B);

In ViewSynthesis for Windows, the carry out is tied to the high bit of the results when the add2c function is used, and the result is exactly one bit larger than the size of the larger operand. Similarly, you can prepend a zero to the operands to specify that you want a carry when using a “+” operator:

For example:

entity ADD2 is
		port(
				A: in  STD_LOGIC_VECTOR(6 downto 0);
				B: in  STD_LOGIC_VECTOR(6 downto 0);
				Z: out STD_LOGIC_VECTOR(7 downto 0));
	end ADD2;

	architecture BEHAVIOR of ADD2 is
	begin

		Z <= (”0” & A) + (”0” & B);

	end BEHAVIOR;

will generate a 7-bit adder with the carry out tied to Z(7). This will only work when A and B are the same size, and Z is exactly one bit larger.

After compilation, the ALUCons component will appear as a black box. The implementation will be defined by ALUCons during synthesis based on the technology and speed parameter provided by the user. The result may be a reference to a vendor provided macro, a structural implementation, or equations.