# Using ViewSynthesis for Windows in a 					DOS Window

Overview

About This Appendix

This appendix shows you how use vsyn commands to invoke ViewSynthesis for Windows in a DOS window.

NOTE:

ViewSynthesis for Windows, the Windows tool for synthesizing Workview Office designs, automatically runs vsyn, as well as the Notepad Editor, the Analyzer, Viewsim, and Viewgen.

Basic Process

Invoking vsyn

Before invoking ViewSynthesis for Windows, you must edit your synth.ini file at least to set your technology. It is a good idea besides to set other compile and synthesize options at the same time.

To run ViewSynthesis for Windows, entering the following command in a DOS window:

vsyn <commands> <options>

To issue a command from DOS:

1.	First enter the vsyn command.

2.	Follow the vsyn command with one or more options as described in this appendix.

3.	Press RETURN.

In this Appendix

The command-line processes are presented in these sections:

- The synth.ini file

- Compiling

- Synthesizing

- Hierarchy Controls

- Special Cases

-	Forcing a Run

-	Xilinx Specific Optimization

The synth.ini File

Setting synth.ini

Before invoking ViewSynthesis for Windows, you must have a synth.ini file with at least the name of the technology you expect tot use. Other options should be set before compiling any VHDL files as well. The allowed synth.ini options are (defaults are capitalized):

technology <tech>
					encoding (1HOT sequential)
					cpu_effort {LOW medium high}
					opt {AREA speed off}
					alu_speed <number>
					flatten {OFF low medium high}
					redundancy_removal {OFF on}
					max_delay <number>    [default is no constraint]

Figure D-1. synth.ini Syntax

Compiling

Overview

When you “compile,” ViewSynthesis for Windows generates netlists from the vhdl files. It also resolves all hierarchical references, and creates the tracking files used by the built-in engine for version control tracking. Use the -vhdl option to compile one or more vhdl files:

vsyn -vhdl <file1> <file2> ...

For example:

vsyn -vhdl test1 test2

In this example, files test1 and test2 are vhdl files. Because ViewSynthesis for Windows assumes this, you have the option of excluding or including the .vhd extension.

Cleaning Up an Old Design

When you start a new design you may need to clean up the old design database. ViewSynthesis for Windows incrementally adds compiled files to the existing design database. Use of the -cleanup option before starting a new design is strongly recommended.

vsyn -cleanup

Synthesizing

Optimizing for Area

To optimize for area, make sure you have set the appropriate entries in the synth.ini file (“opt area”), and you have selected a “cpu_effect”.

Then use the -synth option as shown below:

vsyn -synth <options> <entity>

For some FPGA technologies (such as XC4000), a useful option is:

vsyn -synth -fpga <entity>

which causes VSfW to use the technology specific mapper for XC4000.

Optimizing for Speed

To optimize for speed, make sure you have set the appropriate entries in the synth.ini file (“opt speed”), and you have selected a “cpu_effect”.

Then use the -synth option as shown below:

vsyn -synth <options> <entity>

For some FPGA technologies (such as XC4000), a useful option is:

vsyn -synth -fpga <entity>

which causes ViewSynthesis for Windows to use the technology specific mapper for XC4000.

Optimizing for Speed with Constraints

To optimize with constraints, you can set the “max_delay” in the synth.ini file and/or set port constraints for each entity.

To set individual port constraints, go into each synth\<entity> directory, and copy a file named <entity>.pdf into a file named <entity>.mcm in the same directory. All constraints in this file will be set to 0. You then edit the .mcm file to set constraints.

Controlling Hierarchy

Dissolve or Flatten

You can optionally restructure the design tree’s hierarchy before performing synthesis. Two operations can be performed on an entity in a design tree:

- Dissolve: the boundary of the entity is dissolved into its parent entity, and the parent then contains the entities netlist.

- Flatten: when an entity is flattened, its children are recursively dissolved into it, leaving a flat representation of the original entity.

An entity can be flattened or dissolved at two stages:

- before optimization

- before mapping

By dissolving an entity before optimization (-dis_preopt ), the parent entity is optimized as a bigger netlist. This can take advantage of cross-boundary functionality at some computational cost.

Dissolving after optimization ( -dis_premap ) optimizes each entity, and dissolves boundaries before mapping so that the final netlist does not include dissolved entities.

Hierarchy Control Commands

Use these commands:

vsyn  -dis_preopt <entity>
vsyn -fla_preopt <entity>
vsyn -dis_premap <entity>
vsyn -fla_premap <entity>

Special Cases

Forcing a Run

Normally, ViewSynthesis for Windows only performs optimization and mapping on components that need updating. To defeat this mechanism, and force a synthesis run, add the -f command-line option as shown below:

vsyn -f -synth <entity>

Xilinx Specific Optimization

NOTE:

Much of the Xilinx-specific material presented here is also discussed in Appendix C, “Xilinx-Specific Synthesis”.

To generate netlists that include clock enable circuitry, include a Xilinx technology in the synth.ini file before compiling the vhdl files, then add one or more of the following commands.

- The -xtech command option performs Xilinx CLB packing.

- Add the -fpga command option for advanced packing instead of -xtech.

- Add the -iopad option for iopad insertion.

Syntax:

vsyn -fpga -iopad -synth <entity>

Lock File

Occasionally, if vsyn exits abnormally, you may get a message which says:

There is already a synthesis run in progress in this project directory. Running multiple sessions would compromise the data integrity and render the “make” mechanism ineffective.

If there is no synthesis run in progress, delete the file c:\test\hac.lck and re-run.

Vsyn creates a zero-length file called hac.lck when it starts running to prevent two synthesis runs from happening in the same directory. If you see this message, it is safe to delete the hac.lck files as long as you are sure no synthesis run is occurring in that directory.

Built-in Help

Accessing Built-in Help

Use the -help command to invoke the following vsyn help message. This lists all of the ViewSynthesis for Windows commands you can use from DOS, and what they do. The usage is:

vsyn <options>

options:

-vhdl <list_of_files>		: compile vhdl files

-gnl <list_of_files>		: compile gnl files

-cleanup		: remove all synthesis generated design data

-synth “*”		: run synthesis on top level entity

-synth <list_of_entities>	: run synthesis on entities

-opt <list_of_entities>		: run logic optimization (no mapping) on entity entities

-fla_preopt <list_of_entities>		: flatten entities prior to optimization

-fla_premap <list_of_entities>		: flatten entities prior to mapping

-dis_preopt <list_of_entities>		: dissolve entities prior to optimization

-dis_premap <list_of_entities>	: dissolve entities prior to mapping

-uniq <List_of_entities>	: clone entities by instantiating

-iopad	: insert iopads on the top level entity

-xtec: run Xilinx XC2000 or XC3000 specific optimization

-fpga	: run Xilinx XC4000 or Actel specific optimization

-f		: force synthesis even if the entity has been synthesized already

-n		: update status, but do not run synthesis

-sisscript <script>		: customized optimization

-show	: show the area/slack statistics of the design

-help	: prints this help message

Legal combination of options:

vsyn -cleanup

vsyn -vhdl <list_of_files>

vsyn -gnl <list_of_files>

vsyn -opt <list_of_entities>

vsyn [-synth_options] -synth <list_of_entities>
		synth_options = [sisscript | xtech | fpga | iopad | xcons | show | n | f]

vsyn -fla_preopt <list_of_entities>

vsyn -fla_premap <list_of_entities>

vsyn -dis_preopt <list_of_entities>

vsyn -dis_premap <list_of_entities>

vsyn -help