- PSPLIT -

USAGE:

PSPLIT [-g] [<input PROM file name>] [<PROM size>]

The purpose of 'PSPLIT' is to break a large PROM file into a number of

smaller PROM files.  This enables someone to build a PROM file and not worry

about whether or not it will fit onto a single PROM, because 'PSPLIT' will

correctly break the file into smaller files that will each fit in a PROM of the

desired size.  This function supports the Intel MCS-86 Hexadecimal Object,

Motorola Exormax, and Tektronix Hexadecimal formats and will convert a file in

any one of these formats into shorter files of the same format.

If the file format is Intel MCS-86 Hexadecimal Object, PSPLIT requires

that the first record of the input file be an Extended Base Address record.

'PSPLIT' has two different methods of breaking a file into smaller files.

One of these methods (with the -g option) will preserve all the blank space or

unaddressed locations within the PROM file.  If the file format is Intel MCS-86

Hexadecimal Object, the program will adjust the address and the extended base

address to preserve the value of the original extended base address and all the

unaddressed locations.  The other method (without the -g option) eliminates all

the spaces making a continuous set of bytes in the file.  This later option is

mainly used with the Xilinx XC1736 serial PROM.

The PROM size can be any number of kilobytes between 0 and 4000.  Any size

larger than this maximum will not be accepted.  The default for the PROM size

is XC1736 which is the size for a Xilinx serial PROM.  If someone desires to

use a serial PROM, they must enter XC1736 for the PROM size.

The naming convention for the shorter files is to remove the extension

from the original filename, append a number that corresponds with how many

smaller files have already been created, and finally append a new extension

which would depend upon the format of the file (.exo, .mcs, or .tek).  So

if the original file was named dice.test and was of the Motorola Exormax

format, the first output file would be named dice1.exo.  If the file dice1.exo

already existed, you would be asked if you wanted to overwrite dice1.exo or to

change the output filename.

DISCLAIMER

==========

XILINX IS PROVIDING THIS SOFTWARE TO USERS OF XILINX PRODUCTS SOLELY FOR

USE IN SUPPORT OF THOSE PRODUCTS. XILINX DOES NOT ASSUME ANY LIABILITY

ARISING OUT OF THE APPLICATION OR USE OF THIS SOFTWARE OR FOR ANY ERRORS

OR DEFECTS THAT MAY EXIST IN THIS SOFTWARE. XILINX EXPRESSLY DISCLAIMS ANY

WARRANTY, EXPRESS OR IMPLIED, INCLUDING ANY WARRANTY OF MERCHANTABILITY OR

FITNESS FOR A PARTICULAR PURPOSE.