MAKESRC - Program Specifications

Background

The MAKESRC program is intended for use by customers who want to configure

an LCA in the slave mode as a microprocessor peripheral. The input file is a

PROM data file in the MCS-86 format. The format of the output file can be

specified by the user through a control input file.

Usage

makesrc -i <input file>[.mcs] [-o <output file>[.hex] ] [-c <control file.extn>]

Input

Data file

The primary input is a PROM file in the MCS-86 hex format. The LCA

configuration data is normally produced in the binary bitstream file ( .bit )

by the MAKEBITS program. The PROM file in the MCS format can then be produced

by the MAKEPROM program. The MAKEPROM program will also take care of

daisy-chaining if several LCA devices are to be configured.

Control file

A control file can optionally be used to specify the format of the output file.

The user can edit the fields in the control file. A sample control file is

shown below.

;control file for makesrc program

begindata = "unsigned\nlong\nlca_data[]=\n{"

beginline = ""

endline = ","

endline_last = ""

bytes_per_word = 4

byte_order = 1,0,3,2

items_per_line = 5

item_prefix = "0x"

item_suffix = ""

item_separator = ","

shift_out_first = "lsb"

format = "hex"

enddata = "}"

An explanation for the different fields in the control file follows.

comment 		Any line starting with a ';' is treated as a

comment line

begindata 		Text which appears as a header , e.g. for assembler

files it could be ".segment"  whereas for C files it

may be "unsigned long lca_data[] = {" . Default is "".

beginline         	Text which appears at the beginning of every data line,

e.g. ".DB" or ".DW" . Default is "".

endline			line ending - will appear at the end of every data line,

( except the last data line- if a special last line

ending is specified.) e.g. ";" , "," etc. Default is "".

endline_last		end of last data line. If this is not specified, the

end of last data line will be the same as the end of

every data line.

bytes_per_word 		number of bytes per word - valid values are 1,2 or 4

corresponding to 8,16 and 32 bit words. Default is 1.

byte_order 		byte position for the bytes within a word.

for 8 bit words , byte order must be 0 ( default )

for 16 bit words, byte order can be 0,1(default) or 1,0

for 32 bit words, it should be the required sequence,

e.g. 0,1,2,3 ( default )  - big endian

3,2,1,0            - little endian

1,0,3,2            - any other order

items_per_line 		number of data items per line of output . This number

should be chosen keeping the bytes_per_word in mind to

get reasonable line size. Default is 8.

item_prefix 		prefix for data item - "0x" , "0" etc. This will appear

as a prefix to every byte/word in the output file.

Default is "".

item_suffix 		suffix for data item , e.g."h". This will appear as a

suffix to every byte/word in the output file. Default

is "".

item_separator		separator between two data items on the same line, e.g.

"," . This will appear between two bytes/words in the

output file. Default is "".

shift_out_first 	shift order - whether "lsb" or "msb" is shifted out

first corresponding to right or left shift.

Default is "lsb".

format 			the format specifier for the file, e.g. hex, dec, oct.

Default is hex.

enddata 		end data - stuff at the end of the output file.

Default is "".

Note that all the fields in the control file need not be specified. Only the

ones which are different from the default values are required to be specified.

If the control file is not used at all, ( -c option not used ), it is

equivalent to using the default value for all the fields.

Parameters must be separated from their values by an '=' sign. Non-numeric

parameter values must be specified within double-quotes. The special

characters which may be used as part of a string value are

\t -  tab

\n -  new line

\f -  form-feed

\\ - '\' character

OUTPUT

The output file contains the configuration data in the specified format.

If enough bytes of data are not available in the input file to build the last

word in the specified format,"FF"s would be padded at the end of the file.

The length count is not changed.

Rev 1.04

6/05/91

Kiran Buch