# Abstract

Peripheral Component Interconnect (PCI) is becoming a fundamental building block of todays high performance PC and workstation peripherals. Developing products which meet the stringent PCI Local Bus Specification is the next challenge for system designers.

Xilinx provides a wide range of programmable logic devices which help designers develop compliant high-performance PCI solutions. The XC7300 Erasable Programmable Logic Devices (EPLDs) offer designers a range of programmable logic devices that are fully compliant with the PCI Local Bus specification.

# This application note describes a flexible PCI bridge interface implemented using these XC73OO EPLDs. This applicaion example is meant as a guide for desgining PCI bus interfaces.

# PCI Local Bus Overview

This section describes the PCI interface and reviews the PCI bus operation. The PCI bus is a 32-bit or 64-bit interface with multiplexed  address and data lines. It is intended for use as

Figure 1: Typical System Architecture

an interconnect mechanism between highly integrated peripheral controllers components, peripheral add-in boards, and processor/memory systems. Figure 1 depicts a typical PCI-based system. The basic system architecture consists of a processor and memory subsystem connected by the processor's local bus. The host bridge connects the local bus and the PCI bus and allows them to communicate with each other.

High performance controllers (SCSI, LAN, Graphics, etc.) directly connect to the PCI bus, giving them a high-bandwidth, low latency path to memory. Also shown is a standard bus bridge, that allows low-cost system expansion using a standard bus.

Each agent connected to the PCI bus functions as either PCI targets (slaves) or combined PCI master/targets (initiators/slaves). The master function enables an agent to take control of the bus and drive the address, data, and control signals. Target functions can not initiate bus transfer cycles and rely on masters  to transfer data to itself.

## PCI Interface Signals

The PCI interface requires a minimum of 47 pins for a target-only device and 49 pins for a master to handle data and addressing, interface control, arbitration and system functions.  Figure x shows the pins in a master/slave device, with required pins on the left side and optional pins on the right side.

Figure 2: PCI Pin List

The following is a summary of the PCI signal pins, including the signal mnemonic. electrical type, name and description. A # following the signal name indicates the signal is active-low. In this document, PCI signal names appear as bold.

| Type | Description |
| --- | --- |
| in | Input is a standard input-only signal |
| out | Totem Pole Output is a standard active drive |
| t/s | Tri-state is a bi-directional, tri-state input/output pin. |
| s/t/s | Sustained Tri-State is an active low tri-state driven by only one agent at a time. |
| o/d | Open Drain allows multiple devices to share signals as a wired-OR |

Table 1: PCI Signal Types

| Signal | Type | Description |
| --- | --- | --- |
| AD[31:0] | t/s | Add and Data Bus |
| C/BE#[3:0] | t/s | Bus Command and Byte Enable. |
| CLK | in | PCI System Clock |
| DEVSEL# | s/t/s | Device Select, identifies target has decoded its address. |
| FRAME# | s/t/s | Cycle Frame, identifies start and length of bus transaction. |
| GNT# | t/s | Grant, identifies master has been granted bus access |
| IDSEL | in | Initialization Device Select, identifies device selected for configuration |
| IRDY# | s/t/s | Initiator Ready, identifies when master ready to complete data transfer |
| PAR | t/s | Parity across AD[31:0] and C/BE[3:0] |
| PERR# | s/t/s | Parity Error |
| REQ# | out | Request, identifies master request to use the bus |
| RST# | in | PCI System Reset |
| STOP# | s/t/s | Stop, target request to stop current transaction |
| TRDY# | s/t/s | Target Ready, identifies target ready to complete data transfer. |

Table 2: PCI Signal Summary

## PCI Data Transfer

The basic bus transfer mechanism on PCI is a burst transaction. A burst is composed of an address phase and one or more data phases. PCI supports bursts in both memory and I/O address spaces. The fundamentals of all PCI data transfers are controlled with three signals: FRAME#, IRDY#, and TRDY#. The first clock edge on which FRAME# is asserted is the address phase, and the address and bus command code are transferred on that clock edge. FRAME# will remain asserted to indicate each data phase to follow, and will deassert to indicate the final burst data phase.

Data is transferred between master and target on each clock edge for which both IRDY# (Initiator Ready) and TRDY# (Target Ready) are asserted. Wait cycles may be inserted in a data phase by either the master or the target with IRDY# and TRDY# signals respectively. The source of the data is required to assert its xRDY# signal unconditionally when data is valid. The receiving agent may assert its xRDY# as it chooses.

Once a master has asserted IRDY# it cannot change IRDY# or FRAME# until the current data phase completes regardless of TRDY#. Once a target has asserted TRDY# or STOP# it cannot change DEVSEL#, TRDY#, or STOP# until the current data phase completes.

At such time as the master intends to complete only one or more data transfers, FRAME# is deasserted and IRDY# is asserted indicating the master is ready. After the target indicates the final data transfer (TRDY# asserted), the interface returns to the IDLE state with both FRAME# and IRDY# deasserted.

The maximum transfer rate of the PCI local bus is one 32-bit word every 30 ns, or 132 MB/sec. The following timing diagrams show the relationship of significant signals involved in 32-bit  data transactions.

Figure 3: PCI Read Transaction (PCI LBS p28)

## Figure 4: PCI Write Transaction (PCI LBS p28)

## PCI Bus Commands

All PCI devices are required to respond as a target to configuration read/write commands. PCI targets that contain relocatable functions/registers provide host agents (masters) the option of mapping those functions into memory space,  allowing a device to work where I/O space is not available The PCI bus commands, encoded on CBE[3:0]# lines, are summarized below:

| C/BE[3:0]# | Command |
| --- | --- |
| 0000 | Interrupt Acknowledge |
| 0001 | Special Cycle |
| 0010 | i/O Read |
| 0011 | I/O Write |
| 0100 | Reserved |
| 0101 | Reserved |
| 0110 | Memory Read |
| 0111 | Memory Write |
| 1000 | Reserved |
| 1001 | Reserved |
| 1010 | Configuration Read |
| 1011 | Configuration Write |
| 1100 | Memory Read Multiple |
| 1101 | Dual Address Cycle |
| 1110 | Memory Read Line |
| 1111 | Memory Write and Invalidate |

Table 3: PCI Command Summary (PCI LBS p19)

## PCI Address Space

PCI defines three address spaces: memory, I/O, and configuration. Memory and I/O spaces are the same as those for Intel microprocessors; configuration space is unique to PCI.

PCI configuration space is divided into a predefined header region (64 bytes) and a device dependent region (192) bytes. A PCI-compliant device is not required to implement all (256) registers; however, all unimplemented registers must return a value of zero when read. The format for the predefined header portion is shown in Table 4.

## PCI definition provides for a fully software driven  initialization and configuration via a separate configuration address space. Exactly how the configuration space is implemented is left to the discretion of the designer.

## One method is to simply pass configuration request to dedicated memory in the host interface. The designs in Appendices A and B contain configuration cycle decode logic and control signals to support "pass through" configuration requests to host memory.

| 31 | 16 | 15 | 0 |  |
| --- | --- | --- | --- | --- |
| Device ID |  | Vendor ID |  | 00h |
| Status |  | Command |  | 04h |
| Class Code |  |  | Revision ID | 08h |
| BIST | Header Type | Latency Timer | Cache Line Size | 0Ch |
| Base Address Registers |  |  |  | 10h 14h 18h 20h 24h |
| Reserved |  |  |  | 28h |
| Reserved |  |  |  | 2Ch |
| Expansion ROM Base Address |  |  |  | 30h |
| Reserved |  |  |  | 34h |
| Reserved |  |  |  | 38h |
| Max_Lat | Min_Gnt | Interrupt Pin | Interrupt Line | 3Ch |

## Table 4:  Configuration Space Header

## The Base Address Register is used by the operating system to determine the Memory or I/O space requirements of a device. In this design, the interface assumes a 16 MB address space (24-bits); thus the upper 8 bits of  the Base Address Register (40h) have been implemented with read-write capability and the lower 24 bits will always return a read value of zero. To expand or contract the address space allocated, the user would simply adjust the size of the Base Address Register and its associated address comparator.

## 5V and 3.3V Signaling Environment Support

The output buffers of the XC7300 family of devices are designed to meet the 5V and 3.3V PCI V/I specifications. The I/O structures can operate at either 5V or 3.3V levels. This allows XC7300 devices to be used to interface mixed 5V and 3.3V designs to the PCI bus.

PCI cards and connectors are keyed to manage the 5V to 3.3V transition. In one orientation, the connector is keyed to accept 5V system signaling environment boards; turned 180o the key is located to accept 3.3V system signaling environment boards.

Universal add-in cards, cards built to work in both 5V and 3.3V system signaling environments, have two key slots so that they can plug into either connector.

Figure 6: PCI Board Connectors (PCI LBS p85)

# Flexible Bridge Design

This section discusses various design aspects of the bridge: the interface between the processor/memory subsystem and the PCI bus. Many of these discussions are generic to any bridge in a PCI-system, including address mapping, data buffering, and data path management.

The primary bus unit deals with incoming and outgoing cycles from the processor. During configuration cycles, it writes configuration information for the bridge to a block of registers. During data transactions, the primary and secondary bus units can buffer bursts of data in the bridge to allow concurrent operations on the PCI bus. The secondary bus unit manages the PCI bus interface, performing byte swapping, driving necessary signals, or any other operation required by the bus.

Figure 7: Typical Bridge Architecture

In addition to mapping the address space of one bus into the address space of another, the bridge acts as an elastic data buffer between two synchronous systems for the purpose of buffering data.

## Bridge Interface Operation

As shown in Figure 8, the PCI bridge controller consists of a PCI bus interface module, memory controller, and error handler. The bridge

controller design provides a generic solution to illustrate the connection of an agent to PCI. When the PCI bus interface (XC73108) detects a PCI bus transaction, it passes D[31:0] (address/data) to the memory controller (XC3190). It sets HIT, S[1:0], and MEM_WR appropriately, to inform the host interface of the concurrent bus transaction. Lastly it asserts DEV_ACK#, which tells the host that all of the other signals are being driven with valid data.

When the host device has finished the transaction, it asserts DEV_REQ#. Upon completion of the PCI bus transfer (IRDY# and TRDY# asserted simultaneously), it deasserts DEV_ACK#. This host interface asserts DEV_REQ# until it sees DEV_ACK# deasserted. The host interface device can also signal an error to the PCI bus interface by asserting T_ABORT# or RETRY#. The PCI interface handles these by asserting and deasserting the appropriate PCI bus signals.

The memory controller state machine in the host device is programmed using PCI bus I/O commands. Once the controller is properly setup and started, the decoded activity is started as a memory operation or until a new I/O command is received. The DMA controller handles all FIFO and DMA read & write cycles. Start address and count lengths are in I/O data bits D[23:0]. I/O commands are decoded from data bits D[27:24].

Figure 8: PCI Bridge Controller Design

## Device specific functions such as buffers, burst address generators  and FIFOs vary from one application to the next. For specific information on memory controller designs and FIFO devices, refer to the specific manufacturer data sheets.

## PCI Bus Interface Design

## This section describes the implementation of the PCI Bus Interface design. The state machine, design equations, and configuration registers are discussed.

## Many designs will make use of either a target or master bus interface. The target state machine implemented in this design supports some options (but not all) discussed in the PCI specification.

## The bus interface consists of two parts. The first is a target bus sequence that performs the actual bus operation. The second part is the backend or hardware application. In a target the backend determines when a transaction is terminated. The bus sequence performs the bus operation as requested and guarantees that the PCI protocol is not violated.

The state machine for the target bus sequencer is shown in Figure 9. This figure is similar to the one found in the PCI Local Bus Specification, but has been modified to support the memory interface described above.

## The following paragraphs discuss each state and

## Figure 9: Target Bus Sequencer Machine.

## describe how transitions occur during actual data transfers.The IDLE state is the default state. The bus interface latches signals from the PCI bus to determine the start of a data transfer or configuration access.

## When a bus access begins, the address is compared to the base address. If the address matches (HIT is asserted), the sequencer transitions to the S_DATA state to start a data transfer. Otherwise, it goes to the B_BUSY state.

B_BUSY is a state where the agent waits for the current transaction to complete and the bus to return to idle.

S_DATA is the state where the target transfers data. When the transfer has completed, the sequencer will transition to the TURN_AR state (non-burst transfers) or BACKOFF state (burst transfer attempt).

TURN_AR is the state used to release the PCI bus once the data transfer is complete. It automatically transitions back to the idle state.

## BACKOFF is the state where the sequencer goes after it asserts STOP# and waits for the master to deassert FRAME#. It then transitions back to the TURN_AR state.

## PCI Error Handler Design

PCI error coverage may range from devices having no interest in errors to agents that detect, signal, and recover from errors. To allow this flexibility, the generation of parity is required on all transactions by all agents.

Parity on PCI provides a mechanism to determine transaction by transaction if the master is successful in addressing the desired target and if data being transferred between them occurs correctly. During address and data phases, parity covers AD[31:0] and C/BE[3:0] lines. The number of "1"s on AD[31:0], C/BE[3:0], and PAR equals an even number.

On any given bus phase, PAR is driven by the agent that drives AD[31:0] and lags the corresponding address or data by one clock.

## PCI defines two signals, PERR# and SERR#, for detecting and reporting system errors. PERR# is used exclusively for reporting data parity errors on all transactions except Special Cycle commands. SERR# is used for signaling any other non-parity or system errors.

Xilinx XC7300 family of devices provide an efficient macrocell structure to easily generate parity. Each High-Density Function Block (FB) macrocell is capable of performing a 4-bit parity calculation. Using a pipe-lined design approach with the wide AND logic in the Universal Interconnect Matrix (UIM), the 36-bit parity generator can be accomplished in two clock cycles as specified in the PCI Local Bus Specification. Appendix B contains the ABEL source code (PARITY.ABL) for the Error Handler Design implemented in the XC7354-10 68-pin PLCC package.

# PCI Bus Design Verification

The hardware designs can be completely compliant with the PCI standard using simulation models and the PCI Test Suite from Logic Modeling Group. The 27 test scenarios in the PCI Test Suite have been developed in collaboration with the PCI Special Interest Group to exactly match the SIG's compliance checklist.

The tests provide cycle-by-cycle specifications of what it takes to pass the protocol checklist of the PCI SIG's compliance plan. Combined with Logic Modeling's XC7300 simulation models, the PCI Test Suite can be used to check any PCI design.

The PCI Test Suite is verified and supported on all major VHDL and Verilog Simulators. For detailed product information call Logic Modeling at 800-34-MODEL or e-mail modelinfo@lmc.com

# Summary

This application note discusses the implementation of a PCI bus interface using Xilinx XC73108. Xilinx XC7300 family of programmable EPLDs offers a wide range of devices which can be used to implement various PCI interface solutions.

As a programmable device, the design described in this application note provides the foundation for PCI systems and allows customization for different design variations and applications.

Contact your local Xilinx or distribution sales office for more information or to obtain assistance in evaluating Xilinx FPGAs and EPLDs for your PCI designs.

# Interfacing with Xilinx XC7300 Devices

This section provides all the electrical specifications and operating conditions in both 5V and 3.3V signaling environments for the Xilinx XC7300 devices. This section references the details of the PCI Local Bus Specification Revision 2.0 concerning DC, AC Clock and Timing parameters. It purposefully is divided into section titles that are identical to the PCI specification for easy reference. These specifications show that the Xilinx XC7300 devices in Table 5 comply with all PCI Local Bus specifications.

The PCI specification calls for components to operate within the "commercial" range of environmental parameters. All of the

specifications listed for Xilinx XC7300 devices are for the commercial range of Vcc = 5.0V ± 5% and a ambient temperatures of 0oC to 70oC.

| PCI Compliant XC7300 Devices |  |
| --- | --- |
| XC7318 -5, -7, -10 | PQ44,PC44 |
| XC7336 -5, -7,-10 | PQ44,PC44 |
| XC7354 -7, -10 | PC44,PC68,PC84 |
| XC7372 -7, -10 | PC84,PQ100 |
| XC73108 -7, -10 | PQ100,PQ160,BG225 |
| XC73144 -10 | PQ160.BG225 |

Table 5: Xilinx XC7300 PCI Compliant Devices

DC Specification for 5V Signaling

| Symbol | Parameter | Condition | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |  |
| Vcc | Supply Voltage |  | 4.75 | 5.25 | 4.75 | 5.25 | V | Ö |
| Vih | Input High Voltage |  | 2.0 | Vcc+0.5 | 2.0 | Vcc+0.5 | V | Ö |
| Vil | Input Low Voltage |  | -0.5 | 0.8 | -0.5 | 0.8 | V | Ö |
| Iih | Input High Leakage Current | Vin = 2.7 |  | 70 |  | 10 | mA | Ö |
| Iil | Input Low Leakage Current | Vin = 0.5 |  | -70 |  | -10 | mA | Ö |
| Voh | Output High Voltage | Iout = -2mA | 2.4 |  | 2.4 |  | V | Ö |
| Vol | Output Low Voltage | Iout = 3mA, 6mA |  | 0.55 |  | 0.5 | V | Ö |
| Cin | Input Pin Capacitance |  |  | 10 |  | 5 | pF | Ö |
| Cclk | CLK Pin Capacitance |  | 5 | 12 |  | 7 | pF | Ö |
| CIDSEL | IDSEL Pin Capacitance |  |  | 8 |  | 5 | pF | Ö |
| Lpin | Pin Inductance |  |  | 20 |  | 10 | nH | Ö |

AC Specification for 5V Signaling (Fast Function Block)

| Symbol | Parameter | Condition | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |  |
| Ioh(AC) | Switching Current High | 0<Vout£1.4 | -44 |  |  | -100 | mA | Ö |
|  |  | 1.4<Vout<2.4 |  | EqnA |  | -88 | mA | Ö |
|  | (test point) | Vout = 3.1 |  | -142 |  | -40 | mA | Ö |
| Iol(AC) | Switching Current Low | Vout ³ 2.2 | 95 |  |  | 140 | mA | Ö |
|  |  | 2.2 >Vout >0.55 | Vout /0.023 | EqnB |  | 50 | mA | Ö |
|  | (test point) | Vout = 0.71 |  | 206 |  | 65 | mA | Ö |
| Icl | Low Clamp Current | -5 < Vin£ -1 |  |  |  |  | mA | Ö |
| tr | Unloaded Output Rise Time | 0.4V to 2.4V | 1 | 5 | 1.05 |  | V/ns | Ö |
| tf | Unloaded Output Fall Time | 2.4V to 0.4V | 1 | 5 | 1.85 |  | V/ns | Ö |

AC Specification for 5V Signaling (High Density Function Block)

| Symbol | Parameter | Condition | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |  |
| Ioh(AC) | Switching Current High | 0<Vout£1.4 | -44 |  |  | -60 | mA | Ö |
|  |  | 1.4<Vout<2.4 |  | EqnA |  | -48 | mA | Ö |
|  | (test point) | Vout = 3.1 |  | -142 |  | -30 | mA | Ö |
| Iol(AC) | Switching Current Low | Vout ³ 2.2 | 95 |  |  | 100 | mA | Ö |
|  |  | 2.2 >Vout >0.55 | Vout /0.023 | EqnB |  | 30 | mA | Ö |
|  | (test point) | Vout = 0.71 |  | 206 |  | 35 | mA | Ö |
| Icl | Low Clamp Current | -5 < Vin£ -1 |  |  |  |  | mA | Ö |
| tr | Unloaded Output Rise Time | 0.4V to 2.4V | 1 | 5 | 1.20 |  | V/ns | Ö |
| tf | Unloaded Output Fall Time | 2.4V to 0.4V | 1 | 5 | 1.92 |  | V/ns | Ö |

V/I Curves for 5V Signaling

PCI is based on reflective wave rather than incident wave signaling. This means that the bus drivers are sized to only switch the bus half way to the required high or low voltage. PCI output buffers are specified in terms of their AC switching characteristics, specifically the current-to-voltage relationship (V/I curves) of the driver through the active region.

Xilinx XC7300 outputs are designed to meet the 5V V/I curves as given in PCI Local Bus Specification, Revision 2.0. Output drive characteristics are within the PCI specified range (shaded area) as illustrated in Figure

Equation A:						Equation B:

Ioh 	= 11.9*(Vout -5.25)*(Vout + 2.45)		Iol 	= 78.5*Vout *(4.4-Vout)

for Vcc > Vout > 3.1v					for 0v < Vout < 0.71v

DC Specification for 3.3V Signaling

| Symbol | Parameter | Condition | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |  |
| Vcc | Supply Voltage |  | 3.0 | 3.6 | 3.0 | 6.0 | V | Ö |
| Vih | Input High Voltage |  | 0.475 Vcc | Vcc+0.5 | 2.0 | Vcc+0.5 | V | Ö |
| Vil | Input Low Voltage |  | -0.5 | 0.325Vcc | -0.5 | 0.8 | V | Ö |
| Vipu | Input Pull-up Voltage |  | 0.7 Vcc |  |  |  | V | Ö |
| Iil | Input Low Leakage Current | 0< Vin < Vcc |  | ±10 |  | ±10 | mA | Ö |
| Voh | Output High Voltage | Iout = -500mA | 0.9 Vcc |  | 2.4 |  | V | Ö |
| Vol | Output Low Voltage | Iout = 1500mA |  | 0.1Vcc |  | 0.5 | V | Ö |
| Cin | Input Pin Capacitance |  |  | 10 |  | 5 | pF | Ö |
| Cclk | CLK Pin Capacitance |  | 5 | 12 |  | 7 | pF | Ö |
| CIDSEL | IDSEL Pin Capacitance |  |  | 8 |  | 5 | pF | Ö |
| Lpin | Pin Inductance |  |  | 20 |  | 10 | nH | Ö |

AC Specification for 3.3V Signaling (Fast Function Blocks)

| Symbol | Parameter | Condition | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |  |
| Ioh(AC) | Switching Current High | 0<Vout£0.3Vcc | -12 Vcc |  |  | -150 | mA | Ö |
|  |  | 0.3Vcc<Vout<0.9Vcc |  | EqnC |  | -40 | mA | Ö |
|  | (test point) | Vout = 0.7Vcc |  | -32 Vcc |  | -50 | mA | Ö |
| Iol(AC) | Switching Current Low | Vcc>Vout³0.6Vcc | 16 Vcc |  |  | 100 | mA | Ö |
|  |  | 0.6>Vout>0.1Vcc | 26.7 Vout | EqnD |  | 55 | mA | Ö |
|  | (test point) | Vout = 0.18 Vcc |  | 38 Vcc |  | 44 | mA | Ö |
| Icl | Low Clamp Current | -3<Vin£-1 |  |  |  |  | mA |  |
| Ich | High Clamp Current |  |  |  |  |  | mA |  |
| tr | Unloaded Output Rise Time | 0.2Vcc - 0.6Vcc | 1 | 4 | 1.14 |  | V/ns | Ö |
| tf | Unloaded Output Fall Time | 0.6Vcc - 0.2Vcc | 1 | 4 | 1.5 |  | V/ns | Ö |

AC Specification for 3.3V Signaling (High Density Function Blocks)

| Symbol | Parameter | Condition | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |  |
| Ioh(AC) | Switching Current High | 0<Vout£0.3Vcc | -12 Vcc |  |  | -150 | mA | Ö |
|  |  | 0.3Vcc<Vout<0.9Vcc |  | EqnC |  | -22 | mA | Ö |
|  | (test point) | Vout = 0.7Vcc |  | -32 Vcc |  | -27 | mA | Ö |
| Iol(AC) | Switching Current Low | Vcc>Vout³0.6Vcc | 16 Vcc |  |  | 70 | mA | Ö |
|  |  | 0.6>Vout>0.1Vcc | 26.7 Vout | EqnD |  | 30 | mA | Ö |
|  | (test point) | Vout = 0.18 Vcc |  | 38 Vcc |  | 23 | mA | Ö |
| Icl | Low Clamp Current | -3<Vin£-1 |  |  |  |  | mA |  |
| Ich | High Clamp Current |  |  |  |  |  | mA |  |
| tr | Unloaded Output Rise Time | 0.2Vcc - 0.6Vcc | 1 | 4 | 1.03 |  | V/ns | Ö |
| tf | Unloaded Output Fall Time | 0.6Vcc - 0.2Vcc | 1 | 4 | 1.53 |  | V/ns | Ö |

V/I Curves for 3.3V Signaling

Xilinx XC7300 outputs are designed to meet the 3.3V V/I curves as given in PCI Local Bus Specification, Revision 2.0. Output drive characteristics are within the PCI specified range (shaded area) as illustrated in Figure

Equation C:						Equation D:

Ioh 	= (98.0/Vcc)*(Vout-Vcc)*(Vout + 0.4Vcc)	Iol 	= (256/Vcc)*Vout *(Vcc-Vout)

for Vcc > Vout > 0.7Vcc					for 0v < Vout < 0.18 Vcc

Timing Specification

The PCI specification requires components to support frequencies up to 33 MHz as shown in the following tables:

Clock Specification

| Symbol | Parameter | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Min | Max |  | Ö |
| tcyc | CLK Cycle Time | 30 | ¥ | 16 |  | ns | Ö |
| thigh | CLK High Time | 12 |  | 5.0 |  | ns | Ö |
| tlow | CLK Low Time | 12 |  | 5.0 |  | ns | Ö |
| - | CLK Slew Rate | 1 | 4 | 1 | 4 | V/ns | Ö |

The clock waveform must be delivered to each PCI component in the system. In expansion boards, compliance with the clock specification is measured  at the expansion board component, not the connector slot.

5V and 3.3V Timing Parameters

| Symbol | Parameter | PCI Specification |  | XC73108-10 |  | Units | Compliant |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Min | Max |  |  |
| tval | CLK to Signal Valid Delay- bussed signals | 2 | 11 |  | 10 | ns | Ö |
| tval(ptp) | CLK to Signal Valid Delay- point to point | 2 | 12 |  | 10 | ns | Ö |
| on | Float to Active Delay | 2 |  | 10 |  | ns | Ö |
| toff | Active Float Delay |  | 28 |  | 10 | ns | Ö |
| tsu | Input Set up Time to CLK- bussed signals | 7 |  | 5 |  | ns | Ö |
| tsu(ptp) | Input Set up Time to CLK- point to point | 10,12 |  | 5 |  | ns | Ö |
| th | Input Hold Time from CLK | 0 |  | 0 |  | ns | Ö |
| trst | Reset Active Time After Power Stable | 1 |  |  |  | ms |  |
| trst-clk | Reset Active Time After CLK Stable | 100 |  |  |  | ms |  |
| trst-off | Reset Active to Output Float Delay |  | 40 |  |  | ns |  |
| trrsu | REQ64# to RST# setup time | 10*Tcyc |  |  |  | ns |  |
| trrh | RST# to REQ64# hold time | 0 | 50 |  |  | ns |  |

Appendix A

PCI Bridge Interface VHDL Source Code

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.STD_LOGIC_misc.all;

use IEEE.STD_LOGIC_arith.all;

use IEEE.STD_LOGIC_components.all;

entity pci is

port (AD : inout std_logic_vector (31 downto 0);

MD : in std_logic_vector (31 downto 0);

MA : buffer unsigned (20 downto 2);

CBE : in std_logic_vector (3 downto 0);

CLOCK : in bit;

FRAME, TERM, READY, LOCK, IRDY, IDSEL : in boolean;

L_LOCK, T_ABORT, TAR_DLY, E_PERR, R_PERR : in boolean;

TRDY, STOP, DEVSEL, PERR, PAR : buffer std_logic;

MEM_EN : buffer std_logic);

end pci;

architecture BEHAVIOR of pci is

signal AD_REG : std_logic_vector (31 downto 0);

signal CFG_DATA_HI : std_logic_vector (31 downto 21);

signal CFG_DATA_LO : std_logic_vector (7 downto 0);

signal BA : std_logic_vector (31 downto 21);

signal INTR_VECT : std_logic_vector (7 downto 0);

signal CONFIG : unsigned (7 downto 2);

signal MEM_RD, MEM_WR, CFG_RD, CFG_WR, CMD_RD, CMD_WR : boolean;

signal TARGET_OE, TRDY_VAL, STOP_VAL, BD_EN, BURST, AVALID, HIT, MATCH : boolean;

signal PAR_OE, PERR_OE, PERR_OE1 : std_logic;

type TARGET_TYPE is (IDLE_STATE, B_BUSY_STATE, S_DATA_STATE, TURN_AR_STATE, BACKOFF_STATE);

signal TARGET_SEQ, NEXT_TARGET : TARGET_TYPE;

type LOCKED_TYPE is (FREE_STATE, LOCKED_STATE);

signal LOCK_SEQ : LOCKED_TYPE;

signal IDLE, B_BUSY, S_DATA, TURN_AR, BACKOFF, LOCKED : boolean;

type SELECT_TYPE is (BASE_SEL, INTR_SEL, MEM_SEL, NO_SEL);

signal CFG_SEL : SELECT_TYPE;

constant INTR_ACK       : std_logic_vector := "0000";

constant MEMORY_RD      : std_logic_vector := "0110";

constant MEMORY_WR      : std_logic_vector := "0111";

constant CONFIG_RD      : std_logic_vector := "1010";

constant CONFIG_WR      : std_logic_vector := "1011";

constant MEMORY_RD_MULT : std_logic_vector := "1100";

constant MEMORY_RD_LINE : std_logic_vector := "1110";

constant MEMORY_WR_INVL : std_logic_vector := "1111";

constant DEVICE_VENDOR  : unsigned := "000000";

constant STATUS_CMD     : unsigned := "000001";

constant CLASS_CODE     : unsigned := "000010";

constant BASE_ADD_REG   : unsigned := "000100";

constant INTR_LINE      : unsigned := "001111";

constant STATUS_VALUE   : std_logic_vector := "00000010000";

constant CLASS_VALUE    : std_logic_vector := "00000100000";

constant CLASS_DATA     : std_logic_vector := "00001000";

begin

MEM_RD <= CBE = MEMORY_RD or CBE = MEMORY_RD_MULT or CBE = MEMORY_RD_LINE;

MEM_WR <= CBE = MEMORY_WR or CBE = MEMORY_WR_INVL;

CFG_RD <= CBE = CONFIG_RD and IDSEL;

CFG_WR <= CBE = CONFIG_WR and IDSEL;

AVALID <= MEM_RD and BD_EN;

BURST <= MEM_WR and BD_EN;

CMD_RD <= MEM_RD or CFG_RD;

CMD_WR <= MEM_WR or CFG_WR;

HIT <= AD(31 downto 21) = BA(31 downto 21);

MATCH <= HIT and AVALID;

CONFIG <= MA(7 downto 2);

CFG_SEL <= BASE_SEL when CONFIG = BASE_ADD_REG and CFG_WR else

INTR_SEL when CONFIG = INTR_LINE and CFG_WR else

MEM_SEL when CONFIG = STATUS_CMD and CFG_WR else

NO_SEL;

CFG_DATA_HI <= STATUS_VALUE when CONFIG = STATUS_CMD else

CLASS_VALUE when CONFIG = CLASS_CODE else

BA(31 downto 21) when CONFIG = BASE_ADD_REG else

"00000000000";

CFG_DATA_LO <= INTR_VECT when CONFIG = INTR_LINE else

CLASS_DATA when CONFIG = CLASS_CODE else

"00000000";

AD <= AD_REG when S_DATA and TAR_DLY and CMD_RD else

"ZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZ";

CONFIG_REG: process

begin

wait until CLOCK'event and CLOCK = '1';

if MEM_RD then

AD_REG <= MD;

elsif CFG_RD then

AD_REG(31 downto 21) <= CFG_DATA_HI;

AD_REG(20 downto 8) <= "0000000000000";

AD_REG(7 downto 0) <= CFG_DATA_LO;

else

AD_REG <= "00000000000000000000000000000000";

end if;

case CFG_SEL is

when BASE_SEL =>

BA(31 downto 21) <= AD(31 downto 21);

when INTR_SEL =>

INTR_VECT <= AD(7 downto 0);

when MEM_SEL =>

MEM_EN <= AD(1);

when NO_SEL =>

end case;

end process;

ADDRESS_COUNT: process

function slv2uns (ARG : std_logic_vector)

return unsigned is

variable RESULT : unsigned (ARG'range);

begin

for i in ARG'range loop

if ARG(i) = '0' then

RESULT(i) := '0';

else

RESULT(i) := '1';

end if;

end loop;

return RESULT;

end;

begin

wait until CLOCK'event and CLOCK = '1';

if AVALID then

BD_EN <= HIT;

end if;

if BURST then

if MATCH then

MA(20 downto 2) <= slv2uns (AD(20 downto 2));

else

MA(20 downto 2) <= MA(20 downto 2) + 1;

end if;

end if;

end process;

TARGET: process (TARGET_SEQ, LOCKED, FRAME, HIT, TERM, READY, LOCK, L_LOCK, IRDY,

TRDY_VAL, STOP_VAL)

begin

case TARGET_SEQ is

--  IDLE -- Idle condition.

when IDLE_STATE =>

if FRAME then NEXT_TARGET <= IDLE_STATE;

elsif not HIT then NEXT_TARGET <= B_BUSY_STATE;

elsif (not TERM or READY) and (not LOCKED or LOCK) then NEXT_TARGET <= S_DATA_STATE;

else NEXT_TARGET <= BACKOFF_STATE;

end if;

--  B_BUSY -- Agent not involved in current transaction.

when B_BUSY_STATE =>

if FRAME then NEXT_TARGET <= IDLE_STATE;

elsif not IRDY and not HIT then NEXT_TARGET <= B_BUSY_STATE;

elsif not IRDY and HIT and (not TERM or READY) and (not LOCKED or L_LOCK) then

NEXT_TARGET <= S_DATA_STATE;

else NEXT_TARGET <= BACKOFF_STATE;

end if;

--  S_DATA -- Agent has accepted request and will respond.

when S_DATA_STATE =>

if FRAME and (not TRDY_VAL or not STOP_VAL) then NEXT_TARGET <= TURN_AR_STATE;

elsif not STOP_VAL and (TRDY_VAL or not IRDY) then NEXT_TARGET <= BACKOFF_STATE;

else NEXT_TARGET <= S_DATA_STATE;

end if;

--  TURN_AR -- Completed transaction on bus

when TURN_AR_STATE =>

if FRAME then NEXT_TARGET <= IDLE_STATE;

elsif not HIT then NEXT_TARGET <= B_BUSY_STATE;

elsif (not TERM or READY) and (not LOCKED or LOCK) then NEXT_TARGET <= S_DATA_STATE;

else NEXT_TARGET <= BACKOFF_STATE;

end if;

--  BACKOFF -- Agent busy unable to respond at this time.

when BACKOFF_STATE =>

if FRAME then NEXT_TARGET <= TURN_AR_STATE;

else NEXT_TARGET <= BACKOFF_STATE;

end if;

end case;

end process;

TARGET_REG: process

begin

wait until CLOCK'event and CLOCK = '1';

TARGET_SEQ <= NEXT_TARGET;

if FRAME and LOCK then

LOCK_SEQ <= FREE_STATE;

elsif (not FRAME and LOCK and HIT and (IDLE or TURN_AR)) or (L_LOCK and HIT and B_BUSY) then

LOCK_SEQ <= LOCKED_STATE;

end if;

if S_DATA and not TRDY_VAL and CMD_RD then

PAR_OE <= '1';

else

PAR_OE <= '0';

end if;

if not IRDY and CMD_WR then

PERR_OE1 <= '1';

else

PERR_OE1 <= '0';

end if;

PERR_OE <= PERR_OE1;

end process;

IDLE <= TARGET_SEQ = IDLE_STATE;

B_BUSY <= TARGET_SEQ = B_BUSY_STATE;

S_DATA <= TARGET_SEQ = S_DATA_STATE;

TURN_AR <= TARGET_SEQ = TURN_AR_STATE;

BACKOFF <= TARGET_SEQ = BACKOFF_STATE;

LOCKED <= LOCK_SEQ = LOCKED_STATE;

TARGET_OE <= BACKOFF or S_DATA or TURN_AR;

TRDY_VAL <= not (READY and not T_ABORT and S_DATA and (CMD_WR or (CMD_RD and TAR_DLY)));

STOP_VAL <= not (BACKOFF or (S_DATA and (T_ABORT or TERM) and (CMD_WR or (CMD_RD and TAR_DLY))));

TRDY <= 'Z' when not TARGET_OE else

'1' when TRDY_VAL else

'0';

STOP <= 'Z' when not TARGET_OE else

'1' when STOP_VAL else

'0';

DEVSEL <= 'Z' when not TARGET_OE else

'0' when (BACKOFF or S_DATA) and not TAR_DLY else

'1';

PAR <= 'Z' when PAR_OE = '0' else

'1' when E_PERR else

'0';

PERR <= 'Z' when PERR_OE = '0' else

'1' when R_PERR else

'0';

end;

Appendix B

PCI Bridge Interface ABEL Source Code

module TARGET

title 'PCI Target Interface State Machine'

target device ;

"Inputs

CLK,AD1,AD0,HIT,TERM,READY,IRDY         pin;

T_ABORT,TAR_DLY                         pin;

FRAME,CBE3..CBE0                        pin;

IDSEL,BRD_EN                            pin;

"Outputs

TRDY,STOP,DEVSEL                        pin istype 'com';

AVALID                                 node istype 'com';

TS2,TS1,TS0                            node istype 'reg';

TRDY_OE,AD_OE,MEM_OE                   node istype 'reg';

PAR_OE,PERR_OE,RD_WR,S0,S1              pin istype 'reg';

H,L,X   = 1,0,.X.;

"PCI Bus Command Encoding

COMMAND_TYPE    = [CBE3,CBE2,CBE1];

IO_SPACE        = COMMAND_TYPE == [0,0,1];

MEMORY_SPACE    = COMMAND_TYPE == [0,1,1];

CONFIG_SPACE    = COMMAND_TYPE == [1,0,1];

MEMORY_MULT     = COMMAND_TYPE == [1,1,0];

MEMORY_CACHE    = COMMAND_TYPE == [1,1,1];

"TARGET Sequencer State Register Assignment

TARGET_SEQ      = [TS2,TS1,TS0];

IDLE_           = [ 0 , 0 , 0 ];

B_BUSY_         = [ 0 , 0 , 1 ];

S_DATA_         = [ 0 , 1 , 1 ];

TURN_AR_        = [ 0 , 1 , 0 ];

BACKOFF_        = [ 1 , 1 , 0 ];

STATEN1_        = [ 1 , 1 , 1 ];

STATEN2_        = [ 1 , 0 , 1 ];

STATEN3_        = [ 1 , 0 , 0 ];

IDLE            = TARGET_SEQ == [0,0,0];

B_BUSY          = TARGET_SEQ == [0,0,1];

S_DATA          = TARGET_SEQ == [0,1,1];

TURN_AR         = TARGET_SEQ == [0,1,0];

BACKOFF         = TARGET_SEQ == [1,1,0];

@ALTERNATE

@DCSET

Equations

TRDY            = /(READY * /T_ABORT * S_DATA * (/RD_WR + RD_WR * TAR_DLY));

TRDY.OE         = TRDY_OE;

TRDY_OE        := BACKOFF + S_DATA + TURN_AR;

TRDY_OE.CLK     = CLK;

AD_OE          := S_DATA * TAR_DLY * RD_WR;

AD_OE.CLK       = CLK;

PAR_OE          := S_DATA * /TRDY * RD_WR;

PAR_OE.CLK       = CLK;

PERR_OE        := /IRDY * /RD_WR;

PERR_OE.CLK     = CLK;

MEM_OE         := IDLE * FRAME * RD_WR * /AD1 * /AD0

+ HIT * /RD_WR

+ S_DATA * /RD_WR * /T_ABORT;

MEM_OE.CLK      = CLK;

STOP            = /(BACKOFF + S_DATA * (T_ABORT + TERM) * (/RD_WR * RD_WR * TAR_DLY));

STOP.OE         = TRDY_OE;

DEVSEL          = /((BACKOFF + S_DATA) * /TAR_DLY);

DEVSEL.OE       = TRDY_OE;

RD_WR          := IDLE * /CBE0

+ /IDLE * RD_WR;

RD_WR.CLK       = CLK;

S0             := IDLE * FRAME * (MEMORY_SPACE + MEMORY_MULT + MEMORY_CACHE)

+ S_DATA * S0;

S0.CLK          = CLK;

S1             := IDLE * FRAME * (IO_SPACE)

+ S_DATA * S1;

S1.CLK          = CLK;

AVALID          = ((IO_SPACE) + (MEMORY_SPACE) + (CONFIG_SPACE) + (MEMORY_MULT) + (MEMORY_CACHE)) * BRD_EN;

TARGET_SEQ.clk  = CLK;

state_diagram TARGET_SEQ

"IDLE -- Idle condition.

state   IDLE_:  if FRAME then IDLE_

else if /HIT then B_BUSY_

else if HIT * (/TERM + READY)

then S_DATA_

else BACKOFF_;

"B_BUSY -- Agent not involved in current transaction.

state   B_BUSY_: if FRAME then IDLE_

else if /IRDY * /HIT then B_BUSY_

else if /IRDY * HIT * (/TERM + READY)

then S_DATA_

else BACKOFF_;

"S_DATA -- Agent has accepted request and will respond.

state   S_DATA_: if FRAME * (/TRDY + /STOP) then TURN_AR_

else if /STOP * (TRDY + /IRDY) then BACKOFF_

else S_DATA_;

"TURN_AR -- Completed transaction on bus

state   TURN_AR_: if FRAME then IDLE_

else if /HIT then B_BUSY_

else if /TERM + READY then S_DATA_

else BACKOFF_;

"BACKOFF -- Agent busy unable to respond at this time.

state   BACKOFF_: if FRAME then TURN_AR_

else BACKOFF_;

"Unassigned states

state   STATEN1_: goto IDLE_;

state   STATEN2_: goto IDLE_;

state   STATEN3_: goto IDLE_;

end;

module CONFIG

title 'PCI Host Bridge Interface Module'

config device ;

"Inputs

CLK,S0,S1,RD_WR                         pin;

AD31..AD24,CAD7..CAD2                   pin;

"Outputs

BA31..BA24                              node istype 'reg';

BA_WR                                   node istype 'com';

BASE_ADD_REG    = [BA31..BA24];

PCI_ADDRESS     = [AD31..AD24];

CFG_ADDRESS     = [CAD7,CAD6,CAD5,CAD4,CAD3,CAD2];

BASE_ADD_SEL    = [0,0,0,1,0,0];

@ALTERNATE

Equations

BA_WR         = (CFG_ADDRESS == BASE_ADD_SEL) * /S0 * /S1 * /RD_WR;

BASE_ADD_REG := (PCI_ADDRESS) * BA_WR

+ (BASE_ADD_REG) * /BA_WR;

BASE_ADD_REG.CLK = CLK;

end

module ADDRESS

title 'PCI Address Block'

address device ;

"Inputs

CLK,AVALID,AD31..AD24,AD7..AD2          pin;

BA31..BA24                              pin;

"Outputs

CAD7..CAD2                             node istype 'reg';

BRD_EN                                  pin istype 'reg';

HIT,MATCH                              node istype 'com';

BASE_ADDRESS    = [BA31..BA24];

PCI_ADDRESS     = [AD31..AD24];

CFG_ADDRESS     = [CAD7..CAD2];

PCI_DATA        = [AD7..AD2];

@ALTERNATE

Equations

HIT             = (PCI_ADDRESS == BASE_ADDRESS);

MATCH           = HIT * AVALID;

BRD_EN         := HIT * AVALID

+ BRD_EN * /AVALID;

BRD_EN.CLK      = CLK;

CFG_ADDRESS    := (PCI_DATA) * MATCH

+ (CFG_ADDRESS) * /MATCH;

CFG_ADDRESS.CLK = CLK;

end

module DATA_MUX

title  'PCI Host Bridge Interface  Module

This module implements Host Memory/PCI Base Address Multiplexor and

passes Configuration Byte Enable lines CBE[3:0] from the Host

to the PCI Local Bus.'

data_mux device ;

"Inputs

CLK,S0,S1,AD_OE,MEM_OE                          pin;

ADI31..ADI0,HDI31..HDI0,BA31..BA24              pin;

CBEI3..CBEI0,BEI3..BEI0                         pin;

"Outputs

ADO31..ADO0,CBEO3..CBEO0                        pin istype 'reg';

HDO31..HDO0,BEO3..BEO0                          pin istype 'com';

PCI_AD_IN       = [ADI31..ADI0];

PCI_AD_OUT      = [ADO31..ADO0];

PCI_CBE_IN      = [CBEI3..CBEI0];

PCI_CBE_OUT     = [CBEO3..CBEO0];

HOST_AD_IN      = [HDI31..HDI0];

HOST_AD_OUT     = [HDO31..HDO0];

HOST_CBE_IN     = [BEI3..BEI0];

HOST_CBE_OUT    = [BEO3..BEO0];

BA_ADDRESS      = [BA31..BA24,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0];

@ALTERNATE

Equations

PCI_AD_OUT     :=  BA_ADDRESS * /S0 * /S1

+  HOST_AD_IN * (S0 + S1);

PCI_AD_OUT.CLK  = CLK;

PCI_AD_OUT.OE   = AD_OE;

PCI_CBE_OUT    := HOST_CBE_IN;

PCI_CBE_OUT.CLK = CLK;

PCI_CBE_OUT.OE  = AD_OE;

HOST_AD_OUT     = PCI_AD_IN;

HOST_AD_OUT.OE  = MEM_OE;

HOST_CBE_OUT    = PCI_CBE_IN;

HOST_CBE_OUT.OE = MEM_OE;

end

Appendix C

PCI Error Handler Source Code

module parity

title 'PCI Parity Generator Block'

parity device ;

"Inputs

CLK,PAR_OE,CBE0..CBE3,AD0..AD31                pin;

"Outputs

UIM0..UIM7                                    node istype 'com';

PAR0..PAR8,PAR                                 pin istype 'reg,xor';

xepld PROPERTY 'NODE (UIM) UIM0 UIM1 UIM2 UIM3 UIM4 UIM5 UIM6 UIM7';

xepld PROPERTY 'IOPIN PAR0 PAR1 PAR2 PAR3 PAR4 PAR5 PAR6 PAR7';

xepld PROPERTY 'FASTCLOCK CLK';

xepld PROPERTY 'OPTIONS OFF REG_OPT';

PAR_NODE                = [PAR0..PAR8];

@ALTERNATE

XOR_FACTORS     PAR0    =  (AD2 :+: AD3);

XOR_FACTORS     PAR     =  (((UIM4 + UIM5) :+: (UIM6 + UIM7)) :+: PAR8);

Equations

PAR0    := (AD0 :+: AD1) :+: (AD2 :+: AD3);

PAR1    := (AD4 :+: AD5) :+: (AD6 :+: AD7);

PAR2    := (AD8 :+: AD9) :+: (AD10 :+: AD11);

PAR3    := (AD12 :+: AD13) :+: (AD14 :+: AD15);

PAR4    := (AD16 :+: AD17) :+: (AD18 :+: AD19);

PAR5    := (AD20 :+: AD21) :+: (AD22 :+: AD23);

PAR6    := (AD24 :+: AD25) :+: (AD26 :+: AD27);

PAR7    := (AD28 :+: AD29) :+: (AD30 :+: AD31);

PAR8    := (CBE0 :+: CBE1) :+: (CBE2 :+: CBE3);

PAR_NODE.CLK = CLK;

UIM0    = (PAR0 * /PAR1.pin);

UIM1    = (PAR1 * /PAR0.pin);

UIM2    = (PAR2 * /PAR3.pin);

UIM3    = (PAR3 * /PAR2.pin);

UIM4    = (PAR4 * /PAR5.pin);

UIM5    = (PAR5 * /PAR4.pin);

UIM6    = (PAR6 * /PAR7.pin);

UIM7    = (PAR7 * /PAR6.pin);

PAR      := ((UIM0 + UIM1) :+: (UIM2 + UIM3)) :+: (((UIM4 + UIM5) :+: (UIM6 + UIM7)) :+: PAR8 );

PAR.CLK   = CLK;

PAR.OE    = PAR_OE;

end