Figure 8. Configuration program listing.

/******************************************************************************************/

/*   												*/

/*  Rev 1.0 Config.  © 1995 Xilinx Inc.							*/

/*												*/

/*  uP Download program, requires an ASCII bit configuration data file.			*/

/*												*/

/*  Program source and executeables are not supported.					*/

/*  No guarantees are provided or implied by this program or sources			*/

/*  Xilinx customers may use this program and sources for configuring			*/

/*  Xilinx Parts without royalty.								*/

/*												*/

/*  This program must be used with the Application circuit given in the			*/

/*  accompanying note.  									*/

/*												*/

/*  WARNING THIS PROGRAM’S CONFIGURATION CIRCUIT SHOULD ONLY BE USED			*/

/*  WITH THE FOLLOWING PART FAMILIES: XC31xxA, XC4xxx & A, XC52xx				*/

/*												*/

/*												*/

/* Complied with Borland C++ version 3.0, not tested with other compilers			*/

/*												*/

/******************************************************************************************/

#include  <stdio.h>

#include  <io.h>

#include  <conio.h>

#include  <string.h>

#define IOPORTADDR 0x300		/* This must be an unused I/O address for the IBM PC.  	*/

/* Other options are in the range of Hex 300 to Hex 31F	*/

#define MAXSIZEBITS 55000 		/* Number of device configuration data bits.	*/

/* Dependent upon actual device used.		*/

/* See the data book for details.		*/

#define DEVICEFACTOR 900000		/* Waiting factor, this is a device and	*/

/* host machine dependent delay factor.	*/

/* unfortunately we can not use the PC’s	*/

/* timers due to them being I/O mapped.	*/

/* See App Note for further details.		*/

FILE *fp;				/* Pointer to the ASCII config data file.	*/

char dataarray[MAXSIZEBITS];		/* Memory array for the config data.		*/

char filechar;  				/* ASCII bit value from config data file.	*/

unsigned int addr ,addrcount; 	/* Misc counters.  				*/

void delay(long loopval)		/* We need to allow time for the device to 	*/

{					/* finish various operations.  		*/

long j,k;

for (j = 1; j <= loopval; ++j )  {

k = inportb(IOPORTADDR);		/* Users must ensure that their compiler does not  */

/* optimise this loop out as the value of k is     */

/* never used.   				      */

/* This is a loop delay - as we can not use timers */

/* located in I/O space or interrupts, during the  */

/* call.  					      */

/* This has to allow for the FPGA device size and  */

/* CPU, the only way to achieve some CPU timing    */

/* independence for the loop period is to use      */

/* I/O reads.  This gives the loop a fixed timing  */

/* overhead, as the I/O is clocked at 8 MHz.       */

/* Note however that some systems can run I/O at   */

/* 10 or 12 MHz, through user options.             */

}

}

void ReadConfigFileData(char filename[11])

{

addr = 0;

fp = fopen(filename, “r”);

printf(“%s”, filename);

while (!feof(fp)){

fscanf(fp, “%c”, &filechar);

dataarray[addr] = filechar;

++addr;

}

printf(“ Length of File read is %u”, addr-1, “ bytes”);

/* Note that addr is incremented 1 too many times, hence -1 above.  */

}

void InitFPGA(long delayval)

{

outportb(IOPORTADDR, 0x00);	/* Initialise the config memory.  					*/

outportb(IOPORTADDR, 0xff);	/* both output’s required.  						*/

delay(delayval);  			/* Period to allow device to do internal clear.  			*/

/* This has to be a program loop, as interrupts are 			*/

/* now switched off, so we can’t read the timers.			*/

/* Delayval must be calculated for each device.  			*/

/* Alternatively we can assume worst case delay.  			*/

/* ie for largest device, and then add a margin.  			*/

}

void WriteConfigDataToFPGA(long delayvalue)

{

addrcount = 0;

while (addrcount != addr){

if (dataarray[addrcount] ==  ‘1’) outportb(IOPORTADDR,0xff);

else outportb(IOPORTADDR,0xfe); 	/* ‘0’ case selected. 						*/

/* We assume a valid data bit either ASCII ‘1’ or ‘0’.	*/

/* A default case could be provided for error checking.  	*/

addrcount++;

}

delay(delayvalue);   			/* Allow the device time to complete the cofiguration.	*/

}

int main()

{

printf(“\n \n”);

printf(“ Started Configuration Process.  \n”);

printf(“ File data read, now press any key to do download.  \n “);

while (!kbhit());

ReadConfigFileData(“bit_data.stp”); 			/* This file must be a stripped 		*/

/* version of a Makebits 			*/

/* generated rawbits data file.		*/

asm cli;							/* Disable interrupts - any I/O such		*/

/* as disk access 				*/

/* will kill the download.  	  		*/

InitFPGA(DEVICEFACTOR);					/* Strobe the program line, start 		*/

/* device init  				*/

WriteConfigDataToFPGA(DEVICEFACTOR);  			/* Write data to device.  			*/

asm sti;      							/* Re-enable interrupts, then we 		*/

/* can use the PC 				*/

/* again, otherwise it is locked up.		*/

printf(“\n Finished %d \n”);

fclose(fp);

return 0;							/* Main completed OK.  			*/

}