PCI Compliance Checklist XC73108-10

Revision 2.0b

This checklist applies to the following Component/Manufacturer:	__XC73108-10 / Xilinx, Inc.____________

All items were verified over the following range of junction temperatures:	_52C  min  115C max

OR

All items were verified over the following range of CASE  temperatures:	___43C  min  108C  max

5V Signaling

| CE1. | Component supports 5V signaling environment? |  |  | yes __ na___ |  |
| --- | --- | --- | --- | --- | --- |
|  | NOTE: if "na", skip to section "3.3V Signaling" below. |  |  |  |  |
| CE2. | Component operates over voltage range 5V +/- 5% ? Note: "na" allowed for components that support 5V signaling, but draw power from a supply other than Vcc 5V. (see PCI Spec 2.0 page 86; footnote) | na___ yes  no___ |  |  |  |
| CE3. | Voltages between 2.0V and Vcc+0.5V are recognized as logic high? |  |  | yes __ no___ |  |
| CE4. | Voltages between -0.5V and 0.8V are recognized as logic low? |  |  | yes __ no___ |  |
| CE5. | All inputs sink less than 70uA when pulled to 2.7V DC? |  |  | yes __ no___ |  |
| CE6. | All inputs source less than 70uA when pulled to 0.5V DC? |  |  | yes __ no___ |  |
| CE7. | All outputs drive to 2.4V (min) in the high state while sourcing 2mA? |  |  | yes __ no___ |  |
| CE8. | All outputs drive to 0.55V (max) in the low state, sinking 3 or 6*mA? *note: 6 mA required on FRAME#, TRDY#, IRDY#, DEVSEL#, STOP#, SERR#, PERR#, and when used, LOCK#, AD[63::32], C/BE[7::4]#, PAR64, REQ64#, ACK64#. |  |  | yes __ no___ |  |
| CE9. | Outputs source at least 44mA at 1.4V in the high state? proven at: _Vcc= min, __ process=worst/slow, _ junction temp=115C (max) by: ___ SPICE simulation, __ device characterization, ___ other:________________ note: applies to all outputs except REQ#, GNT#, CLK, RST#, and SERR# |  |  | yes __ no___ |  |
| CE10. | Outputs source no more than 142mA at 3.1V in the high state? proven at: _ Vcc=max, _ process=best/fast, __ junction temp=_52C (min) by: ___ SPICE simulation, _ device characterization, ___ other:________________ note: applies to all outputs except REQ#, GNT#, CLK, RST#, and SERR# |  |  | yes __ no___ |  |
| CE11. | Outputs sink at least 95mA at 2.2V in the low state? proven at: _ Vcc=max, _ process=worst/slow, __ junction temp=115C (max) by: ___ SPICE simulation, _ device characterization, ___ other: _______________ note: applies to all outputs except REQ#, GNT#, CLK, and RST# |  |  | yes __ no___ |  |
| CE12. | Outputs sink no more than 206mA at 0.71V in the low state? proven at: _ Vcc=max, _ process=best/fast, __ junction temp=_52C (min) by: ___ SPICE simulation, _ device characterization, ___ other:________________ note: applies to all outputs except REQ#, GNT#, CLK, and RST# |  |  | yes __ no___ |  |
| CE13. | REQ#, GNT# outputs source at least 22mA at 1.4V in the high state? proven at: _ Vcc=min, _ process=worst/slow, _ junction temp=115C (max) by: ___ SPICE simulation, _ device characterization, ___ other:________________ |  |  | yes __ no___ |  |
| CE14. | REQ#, GNT# outputs sink at least 47mA at 2.2V in the low state? proven at: _ Vcc=max, _ process=worst/slow, __ junction temp=115C (max) by: ___ SPICE simulation, _ device characterization, ___ other:________________ |  | na ___ yes _ no___ |  |  |
| CE15. | Clamps on all signals source at least 25mA at -1V, and 91mA at -2V? |  | na ___ yes _ no___ |  |  |
| CE16. | Unloaded rise times are no lower than 1 V/nS between 0.4 and 2.4V? The unloaded maximum rise time is: _5___V/nS (measured at pin) |  |  | yes __ no___ |  |
| CE17. | Unloaded fall times are no lower than 1 V/nS between 2.4 and 0.4V? The unloaded maximum fall time is: _5___V/nS (measured at pin) |  |  | yes __ no___ |  |

3.3V Signaling

| CE18. | Component supports 3.3V signaling environment? NOTE: if "na", skip to section "Loading and Device Protection" below. |  | yes __ na___ |  |
| --- | --- | --- | --- | --- |
| CE19. | Component operates over voltage range 3.3V +/- 0.3V? |  | yes __ no___ |  |
| CE20. | Voltages between 0.475Vcc and Vcc+0.5V are recognized as logic high? |  | yes __ no___ |  |
| CE21. | Voltages between -0.5V and 0.325Vcc are recognized as logic low? |  | yes __ no___ |  |
| CE22. | All inputs sink/source less than 10uA at any voltage from 0V to Vcc? |  | yes __ no___ |  |
| CE23. | All outputs drive to 0.9Vcc (min) in the high state while sourcing 500uA? |  | yes __ no___ |  |
| CE24. | All outputs drive to 0.1Vcc (max) in the low state, sinking 1500uA? |  | yes __ no___ |  |
| CE25. | Outputs source at least 36mA at 0.9V in the high state? proven at: __ Vcc=3.0V, __ process=worst/slow, __ junction temp=__ (max) by: ___ SPICE simulation, __ device characterization, other: ________________ note: applies to all outputs except REQ#, GNT#, CLK, RST#, and SERR# |  | yes __ no___ |  |
| CE26. | Outputs source no more than 115mA at 2.5V in the high state? proven at: _ Vcc=3.6V, _ process=best/fast, __ junction temp=__52C_ (min) by: ___ SPICE simulation, _ device characterization, other: ________________ note: applies to all outputs except REQ#, GNT#, CLK, RST#, and SERR# |  | yes __ no___ |  |
| CE27. | Outputs sink at least 48mA at 1.8V in the low state? proven at: _ Vcc=3.0V, _ process=worst/slow, _junction temp=_115C (max) by: ___ SPICE simulation, _ device characterization, other: ________________ note: applies to all outputs except REQ#, GNT#, CLK, and RST# |  | yes __ no___ |  |
| CE28. | Outputs sink no more than 137mA at 0.65V in the low state? proven at: _ Vcc=3.6V, _ process=best/fast, _junction temp=_52C (min) by: ___ SPICE simulation, _ device characterization, other: ________________ note: applies to all outputs except REQ#, GNT#, CLK, and RST# |  | yes __ no___ |  |
| CE29. | REQ#, GNT# outputs source at least 18mA at 0.9V in the high state? proven at: __ Vcc=3.0V, __ process=worst/slow, __ junction temp=__115C (max) by: ___ SPICE simulation, __ device characterization, other: ________________) |  | yes __ no___ |  |
| CE30. | REQ#, GNT# outputs sink at least 24mA at 1.8V in the low state? proven at: _ Vcc=3.0V, _ process=worst/slow, _ junction temp=_115C (max) by: ___ SPICE simulation, _ device characterization, other: ________________ |  | yes __ no___ |  |
| CE31. | Clamps on all signals source at least 25mA at -1V, and 91mA at -2V? |  | yes __ no___ |  |
| CE32. | Clamps on all signals sink at least 25mA at Vcc+1V, and 91mA at Vcc+2V? | na ___ yes _ no___ |  |  |
| CE33. | Unloaded rise times are no lower than 1 V/nS between 0.2Vcc and 0.6Vcc? The unloaded maximum rise time is: _4__V/nS (measured at pin) | na ___ yes _ no___ |  |  |
| CE34. | Unloaded fall times are no lower than 1 V/nS between 0.6Vcc and 0.2Vcc? The unloaded maximum fall time is: _4__V/nS(measured at pin) |  | yes __ no___ |  |

Loading and Device Protection

| CE35. | Capacitance on all PCI signals (except CLK, IDSEL) is less than or equal to 10 pF? | yes __ no___ |
| --- | --- | --- |
| CE36. | Capacitance on CLK signal is between 5 and 12 pF? | yes __ no___ |
| CE37. | Capacitance on IDSEL signal is less than 8 pF? capacitance guaranteed by: _ device characterization, ____other: _________________ The maximum inductance on any PCI pin is: _20_ nH | yes __ no___ |
| CE38. | Read, understand section "Maximum AC Ratings and Device Protection"? _ believe to be non-issue given technology used proven robustness when exposed to prescribed test condition | yes __ no___ |

Timing Specification

| CE39. | Component is operational at any frequency between DC and 33 MHz? Note: "na" implies component intended for motherboard use only To satisfy this requirement, designs are allowed to require software to place the component in the proper state before stopping the clock and return it to an operational state after restarting the clock. |  | yes __ no___ |
| --- | --- | --- | --- |
| CE40. | Component is operational with a CLK High Time of 12nS? | na ___ yes __ no___ |  |
| CE41. | Component is operational with a CLK Low Time of 12nS? | na ___ yes __ no___ |  |
| CE42. | All bussed signals are driven valid between 2 and 11 nS after CLK? |  | yes __ no___ |
| CE43. | REQ# and GNT# signals are driven valid between 2 and 12 nS after CLK? |  | yes __ no___ |
| CE44. | All Tri-state signals become active no earlier than 2 nS after CLK? |  | yes __ no___ |
| CE45. | All Tri-state signals float no later than 28 nS after CLK? |  | yes __ no___ |
| CE46. | All bussed inputs require no more than 7 nS setup to CLK? |  | yes __ no___ |
| CE47. | REQ# requires no more than 12 nS setup to CLK? | na ___ yes __ no___ |  |
| CE48. | GNT# requires no more than 10 nS setup to CLK? | na ___ yes __ no___ |  |
| CE49. | All inputs require no more than 0 nS of hold time after CLK? |  | yes __ no___ |
| CE50. | All outputs are Tri-stated within 40 nS after RST# goes low? all timings (CE39 through C50) verified by (check all that apply): ____ static timing design tools (MOTIVE, QTV, QuickPath, Veritime ...) ____ dynamic timing design tools (Verilog, Qsim, Quicksim, ViewSim, VHDL, ...) __ silicon AC testing ____ other _______________________________________________ note: maximum timings assume 50pF load, while minimums assume 0pF load. |  | yes __ no___ |

64-bit Components	Component is 32-bit only, this section is        NA__

| CE51. | Component senses, during RST# active, its connection to 64-bit wires? | yes ___ no___ |
| --- | --- | --- |
| CE52. | 64-bit signals will be stable when not connected? | yes ___ no___ |

Explanations:

Note1: XC73108 features 3.3V/5V I/Os which meet PCI Local Bus signaling

environment requirements CE3 or CE23.    Component internal logic  operates

over voltage range 5V +/- 5%._____________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

This section should be used to clarify any answers on checklist items above.  Please key explanation to item number.