Xilinx Adds Another PCI Compliant Family

September 26, 1994

From:  Barry Chaffin (408) 879-4751

The XC3100A-2 is PCI compliant.

PCI specific characterization results for the XC3100A-2 devices show that they comply with all of the 5V requirements of the Component Electrical checklist for Peripheral Component Interconnect bus applications.  A press release to this effect will be issued on October 10 (or whatever the current date).  A preliminary application note with design files for implementing a slave interface in an XC3190A-2 (which part?) will be available by November 1st.  Look for additional announcements in the future regarding the full story on compliance for the XC4000 and HardWire families as well.

Additional points to be aware of:

¥	The XC3100A is compliant in the -2 speed grade and the PQFP package ONLY.

¥	The XC3400 HardWire family meets all of the same requirements as the XC3100A-2, providing a compliant cost reduction path.

¥	XC3100A-2 products are available for immediate delivery or with short lead times.

The XC3100A-2 devices join the XC7300 EPLDs to give designers a choice of PCI compliant products from Xilinx for any system need.  The following table can serve as a guide in determining the appropriate product for specific applications.  Detail explanations for each of these items is on the back of this sheet.

|  | XC7300 -10,7,5 | XC3100A-2 XC3400 | XC4000 XC4300 | XC4400 |
| --- | --- | --- | --- | --- |
| Design Method | PAL/CPLD | FPGA/ASIC |  |  |
| Process Technology | CMOS EPROM | SRAM FPGA Reconfigurable |  |  |
| PCI Compliance | 100% | 100% | Partial | 100% |
| Gate Density | ² 5K | ² 7.5K | ² 25K | ² 25K |
| Configuration Memory | External | Internal/ External | Full Internal | Full Internal |
| Signalling | 5V & 3.3V | 5V Only |  |  |
| Interface | 32 bit | 32 bit | 32/64 | 32/64 |

Design Method - For customers more comfortable with a PAL, CPLD or equation based design entry method, the EPLDs will be the best choice.  For those with an ASIC or schematic based design entry method, the FPGAs will be best.

Process Technology - Those prefering a one-time-programmable (OTP) device will choose the EPLD.  Those in need of reconfigurability will prefer the FPGAs.

Compliance - Both the XC7300 and the XC3100A families offer full PCI compliance.  If partial compliance is OK, the XC4000 family can be used.  For a more detailed discussion of compliance, see Xnote 5A ÒThe Peripheral Component Interconnect BusÓ.

Gate Density - The general PCI bus interface require approximatly 5K gates or less to implement. Complex master/slave designs with additional functionality will require higher gate densities.

Configuration Memory - PCI requires registers for auto-configuration that are most easily implemented using the on-chip RAM of the XC4000.  However, the minimum set of configuration registers can be implemented within the XC3100A and even the EPLDs, depending upon the particular design requriements.

Signaling - PCI provides a technology migration path from 5V to 3.3V technology. The vast majority of designs being implemented today are 5V-only, in which all Xilinx devices will work.  The XC7300 devices are the ONLY programmable devices available in the industry today that are compliant at both 5V and 3.3V.

Interface - PCI specifies 32- and 64-bit interface versions.  The difficulties in implementing a 64-bit interface involve I/O count and potential ground-bounce problems created by SSO conditions.  Most designs today are 32-bit.

Supporting Documentation

Xilinx now offers the ÒPCI Compliance PacketÓ which includes the following items:

¥	Xnote 5A:  The Peripheral Component Interconnect Bus, a market and technical overview of PCI applications and compliance.

¥	Designing Flexible PCI Interfaces with Xilinx EPLDs, an application note for a fully compliant PCI slave interface using EPLDs.

¥	The PCI Component Electrical Checklist for the XC73108-10.

¥	The PCI Component Electrical Checklist for the XC3100A-2 devices will be added following the press announcement.  Use the one attached with customers who request it.

This packet will be continuously updated to include new checklists and application notes as they are completed.  Packets are available now through Xilinx literature by calling Karene at (408) 879-5017.